3GPP TSG-RAN WG2 Meeting #132	R2-2508974

# Dallas, USA, Nov. 17th – 21st, 2025

Source: KDDI Corporation

Title:　 Considerations on Initial and System Access and Others

Agenda Item:	10.3.2.3

Document for:	Discussion

- Introduction

The new SID for “study on 6G Radio” was approved in RAN#108 meeting and was revised in RAN#109 meeting, with the objective to develop 6GR to meet the requirements of the next generation of RAN, including those related to the control plane. Based on that, RAN2#131bis had initial discussion and the following agreements for the future study. In this contribution, we would like to share our consideration regarding system information and system access.

Agreements on System Information

- -	System information design should consider energy efficiency and low complexity for both network and UE.

-	System information design should ensure that the public warning requirements are met.

-	Study RAN2 aspects of system information designs (e.g. on-demand SIB and SSB, periodicity of SSBs)

-	Study how to improve flexibility and extensibility of scheduling SI design

-	Study mechanisms to improve SI update mechanism

-	Study aspects related to SIB1 size.  Understand the issues and desired content/size from RAN2 point of view.

-	Study areas specific SIB design.  Understand 5G pain points and what improvements can be considered.

NOTE: this is a starting list

- Discussion

  - Discussion points for 6GR system information from RAN2 perspective

Based on the agreements reached in previous discussions, it is assumed that system information design for 6GR will involve several key considerations, particularly regarding energy efficiency for both NW and UE. This direction suggests potential enhancements such as on-demand SSB, on-demand SIB1, and SSB/SIB1 adaptation mechanisms. We would like to share our views from deployment scenarios, RRM measurements

System information provides the necessary configuration for a UE to camp on or connect to a cell. In particular, essential system information includes parameters such as cell barring status and initial access configuration. This information must be available to all types of UEs, regardless of whether they are in IDLE or CONNECTED states. This fundamental principle of system information delivery will remain unchanged in 6GR.

Proposal 1: The system information framework in 6GR should continue to ensure that essential configuration, such as cell barred and initial access parameters, is reliably provided to all types of UE, both in IDLE and CONNECTED modes.

From RAN2 perspective, 5G-A introduced several mechanisms aimed at improving energy efficiency on the NW side, including cell-level DTX/DRX, OD-SSB, OD-SIB1, and SSB adaptation. On the UE side, paging and DRX/eDRX-related mechanisms are the primary tools for reducing power consumption. In 6G, to maximize energy savings, it is important to coordinate these mechanisms effectively across both NW and UE-side. For example, a combined approach that leverages cell-level DTX/DRX/OD-SSB/OD-SIB1 and UE DRX/eDRX could support all types of UEs in both IDLE and CONNECTED mode, which would help achieve a balanced and scalable energy-efficient system information framework.

Proposal 2: RAN2 study a coordinated approach to system information design in 6GR that integrates network-side mechanisms (e.g., cell-level DTX/DRX, OD-SSB, OD-SIB1) with UE-side mechanisms (e.g., DRX/eDRX), aiming to support all types of UEs in both IDLE and CONNECTED modes and enabling comprehensive energy efficiency across the system.

However, it seems the intended purpose cannot be effectively achieved in a single carrier scenario. In particular, increasing the transmission periodicity of SSB or SIB1 may lead to longer cell search delays for UEs, which is undesiable in areas with limited coverages. Therefore, a multi-carrier design may need to be considered, similar to the concept of an anchor cell that provides the OD-SIB1 configuration to the UE, and a NES cell that does not transmit SIB1 during certain time periods or within specific spatial areas.

Proposal 3: RAN2 study the system information design that needs coordination between NW and UE side to realize both sides energy efficiency is mainly for a multi-carrier scenario.

Furthermore, we think that the multi-carrier scenario also contributes to the delivery of public warning message. In 5G, ETWS/CMAS indication messages are delivered via short messages transmitted over PDCCH. However, the configuration for PDCCH is provided through SIB1. In 5G-A, UE in IDLE mode stores the OD-SIB1 request related configuration that is obtained from another cell and performs RACH procedure to request SIB1 from a NES cell. If the periodicity of SIB1 is increased, or if SIB1 is transmitted in a on-demand manner, if 6G adopts a similar mechanism to 5G-A, it may delay the UE’s ability to receive the necessary configuration for public warning messages. To address this, a mechanism, such as providing/ piggybacking the SIB1-related configuration or PDCCH monitoring configuration provided/piggybacked via a anchor cell in a muti-carrier scenario or separating and delivering limited control channel-related information from SIB1 to UE, can be studied. However, while the multi-carrier delivery mechanism can be studied within RAN2, the approach of delivering limited control-related information separately from SIB1—which also contributes to reducing the size of SIB1 can be considered in coordination with discussions in RAN1.

Proposal 4: From RAN2 perspective, to ensure the public warning requirements are met, a multi-carrier mechanism that provides the PDCCH related configuration for another cell can be studied first.

In 5G, RRM measurements are primarily based on SSB, and SSB periodicity is typically aligned across cells. Measurement gap is configured to the UE when performing inter-frequency measurement. The maximum number of frequencies that a UE can measure at a time during the measurement gap depends on its capability. CSI-based RRM measurement is specified in Rel-16 but not widely commercialized in 5G due to the late specification and complexity. If in 6G, on-demand SSB or SSB adaptation-like mechanism is adopted from 5G-A for connected UE, it may introduce longer measurement gap and reporting interval, potentially delaying handover decisions. Therefore, in this case, a simplified CSI-based RRM measurement that can provide stable measurement results, or a measurement mechanism without setting measurement gap might be helpful.

Proposal 5: RAN2 take into account the RRM measurement in 6GR system information design, aiming not negatively impact handover performance.

In 5G, the SIB1 structure evolved to include multiple feature specific cellBarred flags, such as,

cellBarred2RxXR

cellBarredATG

cellBarred-eRedCap1Rx

cellBarred-eRedcap2Rx

cellBarredFixedVSAT

cellBarredMobileVSAT

cellBarredNES

cellBarredNTN

cellBarredRedcap1Rx

cellBarredRedcap2Rx

all the above added incrementally as new features emerged. While the approach has served its purpose, it introduces scalability and maintainability as the number of features grows. For 6G, a unified and extensible structure that consider accommodate current and future access types and device categories would be helpful to reduce signaling overhead, simply decoding, and enable much more smooth evolution of the SIB1. Establishing such a design from the first release of 6G will ensure long-term flexibility and robustness of the system information signaling.

Proposal 6: RAN2 aims for a unified and extensible structure for the cellBarred-information within SIB1, if it is needed in 6G.

- 3. Conclusion

Proposal 1: The system information framework in 6GR should continue to ensure that essential configuration, such as cell barred and initial access parameters, is reliably provided to all types of UE, both in IDLE and CONNECTED modes.

Proposal 2: RAN2 study a coordinated approach to system information design in 6GR that integrates network-side mechanisms (e.g., cell-level DTX/DRX, OD-SSB, OD-SIB1) with UE-side mechanisms (e.g., DRX/eDRX), aiming to support all types of UEs in both IDLE and CONNECTED modes and enabling comprehensive energy efficiency across the system.

Proposal 3: RAN2 study the system information design that needs coordination between NW and UE side to realize both sides energy efficiency is mainly for a multi-carrier scenario.

Proposal 4: From RAN2 perspective, to ensure the public warning requirements are met, a multi-carrier mechanism that provides the PDCCH related configuration for another cell can be studied first.

Proposal 5: RAN2 take into account the RRM measurement in 6GR system information design, aiming not negatively impact handover performance.

Proposal 6: RAN2 aims for a unified and extensible structure for the cellBarred-information within SIB1, if it is needed in 6G.