3GPP TSG-RAN WG2 Meeting #132                                                                       R2-2508987

Dallas, USA,  Nov. 17th - 21st , 2025

Agenda item:	10.3.3.3

Source:	China Telecom

Title:	Discussion AS Security for 6G

Document for:	Discussion

- Introduction

A new study item on next generation RAT was approved [1]. The 6GR Study Item aims to develop one non-backward compatible radio access technology to meet a broad range of use cases, and requirements as defined during the RAN requirements study. The detailed objectives of the study on radio interface protocol are as follow:

(3) Radio interface pro tocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and pro tocol design [RAN2] - Control Plane architecture (including RRC states) and pro tocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

In RAN2#131bis, security aspects related to MAC CEs and other Layer 2 control signaling were discussed. This document presents our initial views on further access stratum security issues.

- Discussion

In the 5G Access Stratum(AS) security framework, encryption and integrity protection are implemented at the PDCP layer, ensuring the security of both the control plane and the user plane. The core network provides the required security keys to enable secure communication between the UE and the gNB. Within the protocol stack, the PDCP layer offers integrity and confidentiality protection for RRC signaling exchanged between the UE and the gNB, while the lower layers remain without integrity protection. During PDU session establishment, the core network delivers the user plane security policy to the gNB. Based on this policy, the gNB activates user plane confidentiality and/or integrity protection for each DRB through RRC signaling.

Given the increasing need to enhance security in 6G systems, the 5G security framework can serve as a foundational reference for 6G research. From the RAN2 perspective, it is therefore essential to ensure that the 6G system supports robust security protection for both the control plane and the user plane from Day-1.

Proposal 1: To support security protection for at least both the control plane and the user plane in the 6G system.

In addition, control information such as MAC CE and RLC status reports currently lack security protection. Previous studies have identified this as a potential security vulnerability [2–3]. During the RAN2#13bis meeting, it was agreed to send a LS to SA3 outlining the existing 5G MAC CE information and noting that some of this control information may continue to be used in 6G Layer 2. RAN2 may begin studying the implications for the Access Stratum (AS) once a response is received from SA3. However, introducing additional encryption layers would increase latency, and the resulting overhead in MAC PDU size after applying encryption and integrity protection is also a concern. Therefore, simply replicating PDCP-layer security mechanisms at the MAC layer is not theoretically feasible. Accordingly, it is recommended not to support redundant security protection mechanisms within the AS layer. That said, for certain emerging services, it may be valuable to evaluate whether security should be applied at the PDCP or MAC layer based on network-side configuration. In summary, redundant security mechanisms at the AS layer should be avoided, while allowing to study whether security should be applied at the PDCP or MAC layer based on NW configuration.

Proposal 2: It is recommended to avoid redundant security protection mechanisms within the AS layer, while allowing to study whether security should be applied at the PDCP or MAC layer based on NW configuration.

The emergence of new services in 6G, such as AI/ML-based applications and sensing, covering data collection, transport, processing, storage, access, and exposure (including AI/ML models, compute artifacts, and sensing data), will involve large-scale data collection from UEs. This raises the question of whether additional security protection is required for such data, an aspect that warrants further study within RAN2 in alignment with SA3. In the 6G era, AI is expected to serve not merely as an enhancement but as a fundamental design principle. Unlike traditional networks that employ AI for specific tasks such as traffic prediction or fault detection, a natively AI-driven network will be designed from the outset to operate autonomously, intelligently, and adaptively across all layers.

Proposal 3: To study whether additional security protection is required for data collection from UE for new services such as AI/ML and sensing.

The initial RRC messages exchanged between the UE and the gNB are not security protected until the UE is authenticated and a UE-specific AS security context is established. This absence of protection limits the UE’s ability to safely include sensitive parameters (e.g., S-NSSAI, device type, or location information) in Msg5, which carries the initial NAS Registration Request message. In many scenarios, such parameters are required by network elements, such as the base station, to properly route the initial NAS Request/Response procedure to the appropriate network slice or AMF. Therefore, enabling secure communication between UEs and authorized base stations prior to authentication and AS security establishment is becoming increasingly important, particularly for features such as Non-Terrestrial Networks (NTN) and network slicing. The Small Data Transmission (SDT) mechanism, introduced in Release 18, allows a UE in inactive status to send small data packets and signalling messages to the network based on the configured security context in the source cell. Accordingly, it is necessary to study enhanced security protection for initial RRC messages (e.g., RRC connection setup procedure) in 6G systems.

Proposal 4: To study enhanced security protection for initial RRC messages (e.g., RRC connection setup procedure) in 6G systems.

The current 5G system employs dedicated algorithms for encryption and integrity protection, which are negotiated independently. Recently, 3GPP SA3 completed specifications for 256-bit cryptographic algorithms. In addition, the 3GPP Authenticated Encryption with Associated Data (AEAD) mechanism is being studied for both 5G-A and 6G systems. Unlike traditional approaches that separate encryption and integrity protection, AEAD integrates both functions into a single algorithm using one key. This design simplifies key management, reduces power consumption, and improves processing efficiency through single-pass operation. AEAD adoption has direct implications for AS procedures. For intra-RAT mobility, AEAD simplifies handover, as all security endpoints can support the same mechanism. However, inter-RAT mobility and legacy interworking present challenges, as legacy 4G and 5G systems do not support AEAD. This gap necessitates reconciliation of key derivation methods and security contexts. Another consideration is the security processing order: 5G currently uses MAC-then-Encrypt (MtE) for AS and Encrypt-then-MAC (EtM) for NAS. Since AEAD aligns with the EtM approach, adapting it to AS may require a less efficient two-pass process. Therefore, if AEAD is ultimately adopted in 6G, it will affect the overall security selection and negotiation procedures between the network and the UE. For example, PDCP procedures may need to be updated accordingly to accommodate this change.

Proposal 5: To study the impact of new security mechanism AEAD on RAN side.

- Conclusions

Based on the discussion in the previous sections, we propose the following:

Proposal 1: To support security protection for at least both the control plane and the user plane in the 6G system.

Proposal 2: It is recommended to avoid redundant security protection mechanisms within the AS layer, while allowing to study whether security should be applied at the PDCP or MAC layer based on NW configuration.

Proposal 3: To study whether additional security protection is required for data collection from UE for new services such as AI/ML and sensing.

Proposal 4: To study enhanced security protection for initial RRC messages (e.g., RRC connection setup procedure) in 6G systems.

Proposal 5: To study the impact of new security mechanism AEAD on RAN side.

- References

- RP-251881 New SID: Study on 6G Radio, NTT DOCOMO (Moderator), Prague, Czech Republic

- R2-2506896, Initial Considerations for 6GR UP/CP Common Aspects, Sharp

- R2-2506937, Discussion on Security Requirements in 6GR, CMCC