3GPP TSG-RAN WG2 Meeting #132	R2-2508988

Dallas, USA, Nov. 17th-21st, 2025

# Agenda item:	10.3.3.1

# Source:	ETRI

# Title:	Discussion on data transfer and AI/ML framework

# Document for:	Discussion, Decision

- Introduction

In RAN2#131bis meeting, agrrements on data collection for various types of data and general AI/ML framework have been made as the followings:

Agreements 1. Study the standardized data collection framework with these requirements as a baseline at least for AI/ML - The data collected is secured and data integrity and confidentiality for that data is ensured. - User data privacy, anonymity and user consent is respected. - The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE side data collection, without the need of Service Level Agreement (SLA) for this purpose - This includes initiating, terminating, and fully managing data transfer. - MNO has full visibility for standardized data. - The design is future-proof and extendable. - The UE data collection should minimize impact to the UE battery, UE processing and memory utilization. - UE data collection should minimize impact to user traffic transmission and power saving features These requirements can apply to both UE and NW sided data collection. FFS if some don’t apply to both. 2. Study further requirements of other non-AI/ML use cases/services and understand if we can have more common requirements across different use cases. 3. Study termination points (i.e. understand who are producers, consumers of the data, data collection points. Understand requirements of use cases sensing, aI/ml, son/mdt, QoE etc) and protocol used for data transfer (e.g. IP vs. non-IP). 4. Study model transfer/delivery requirements/functions

Agreements on LCM Study 6G AI/ML LCM framework that supports at least the following functions/procedures: - UE AI/ML Capabilities Exchange; - Applicable Functionality Reporting; - Inference Configuration and Reporting; - Performance Monitoring Configuration and Reporting; - Functionality (De-)Activation and Fallback/Switching to the AI/ML/non-AI/ML Functionality;

This contribution provides leveraging the existing NR AI/ML framework as a basis and extending its capabilities to establish a comprehensive 6G AI/ML framework designed to encompass emerging use scenarios. Furthermore, key considerations are presented regarding the necessity for enhanced data collection mechanisms, catering to both AI/ML training and integrated sensing functionalities. Finally, methodologies for the transfer of this collected data to the designated processing server are discussed.

- Discussion

Data transfer for various type of data

For NR data collection over the air interface, two primary models are currently considered. While the UE-sided model is not yet defined for L3 measurement, the network-sided model extends the existing MDT (Minimization of Drive Tests) framework. In this network-sided approach, logged MDT configuration is managed via the LoggedMeasurementConfiguration message, with reporting carried out using UEInformationRequest/Response messages transmitted. The measurement framework for Immediate MDT configuration is reused. The UE is configured via the RRCReconfiguration message and reports the measured data via MeasResults. For network-side data collection and the CSImeasurement framework with CSI resources, UE is configured via CSI-loggedMeasurementConfig and reported using a UEInformationResponse. For UE-side data collection within the CSI framework, a CSI-ReportConfig configures and the UE reports using the CSI framework.

Additionally, data collection can utilize the Application Layer Measurement Report framework, where configuration is set via RRCReconfiguration (specifically the appLayerMeasConfig), and the measurement results are reported using MeasurementReportAppLayer messages transported. The aforementioned data collection procedures are transmitted as control messages over the control plane connection.

Observation 1: 5G AI/ML data collection is currently supported on the control plane as an extension of the MDT framework, meaning the aforementioned data collection procedures are transmitted as control messages over the control plane connection.

6G data collections considers diverse data types. When AI/ML functionality is applied at the interface, a data collecting transfers collected data to a consumer node. For AI/ML training a training entity, which can be located either outside or within the network, collects data from UE or gNB. The transferred data is typically large in volume and non-real-time. It’s appropriate for data transmission to occur on the data bearer. Meanwhile, for AI/ML inference a collected data from UE transfers to gNB is relatively small and real-time for fast execution. The typical measurement & report framework over the Uu interface is used to transfer collected data from the UE to the gNB. Similarly AI/ML monitoring requires measured data from the UE to be transferred to the gNB, utilizing this same measurement & report framework.

Observation 2: Data transfer for AI/ML training involves a large data volume and is non-real-time, whereas data transfer for AI/ML inference involves a small data volume but has a real-time characteristic

Proposal 1: Study reusing measurement & report framework for AI/ML inference and monitoring.

The 5G NR Integrated Sensing and Communication (ISAC) RAN1 study proposed six types of sensing measurement metrics, ranging from large, raw channel data (Type 1, kB-GB) and spectral components (Type 2, hundreds of kB) to small, processed intermediate results (Types 3 & 4, tens to hundreds of bits) and final object detection results (Type 5, tens of bits), including SINR/RSRP variants (Type 6). Future 6G research (6GR) must study various sensing modes (monostatic, bistatic, DL, UL) involving both TRPs and UEs, and explore cooperative/multi-static sensing mechanisms utilizing multiple network entities.

For 6G data collections, a unified data collection framework is necessary to support a wide array of AI/ML applications. The 6G AI/ML use scenarios are expanding significantly, building upon 5G-A AI/ML functions like CSI prediction, beam management, and AI mobility, and proposing new functionalities such as CSI-RS overhead reduction, AI receiver, AI modulation, and Non-linear Power Amplifier (PA) optimization. As AI/ML will be applied across all communication nodes (including the RAN radio, inter-node RAN interfaces, the core network, and OAM), the radio interface data is relevant to multiple applications. Generating data independently for each AI/ML use case would lead to data collection redundancy. To enhance efficiency, the collection of radio data must be optimized for diverse AI/ML applications. Furthermore, the sensing functionality required to detect the radio environment at communication nodes, which involves data collection and processing, must be integrated; thus, the scope of AI/ML data collection should be extended to encompass the field of radio environment sensing.

For 6G data transfer, to ensure collected data is effectively utilized by diverse AI/ML entities, including communication nodes and 3rd party AI/ML agents, establishing a data delivery framework is studied. For efficient data transport, a centralized architecture is recommended, wherein a data server centrally aggregates data collected from various measurement communication nodes. This server would then efficiently distribute the data to various requesting communication nodes for supporting diverse AI/ML and sensing applications.

In NR, data collection procedures are currently transmitted over the control plane. The communication entities handling the exchange of control messages are configured primarily to process signaling procedures. However, the transmission of collected data often involves either the frequent delivery of short data packets or the transfer of large data volumes. This characteristic makes the current control plane data transfer mechanism inadequate. Therefore, we must enhance the capabilities of the user plane (which is designed for user traffic) to transfer and distribute collected data. Furthermore, we should discuss introducing new connection type to support the functionality for delivering both AI/ML data and Sensing data. While the introduction of entirely new planes or layers might be considered if essential, this contribution specifically advocates for prioritizing the introduction of new connection type to resolve the existing transport limitations.

Observation 3: 6G data collection requires a unified data transmission structure that supports AI/ML data for RAN, sensing data, and application AI/ML data.

Proposal 2: Study a unified data transfer to provide common delivery function for transporting a variety of data types including RAN AI/ML data, sensing data, and application AI/ML data, which is separate from the control plane.

General AI/ML framework considerations

NR AI/ML LCM framework provides a comprehensive approach to manage the lifecycle of AI/ML models, including to data collection, model training, model storage, management, and inference. This framework leverages a functionality-based approach to manage UE-sided models, which is facilitated through UE capability and applicability reporting and network configuration. While the framework was being developed, a complete data collection framework was not yet available for the UE-sided model. Instead, data collection configurations were determined on a use-case basis. For the NW-sided model, a key consideration was to extend the existing MDT (Minimization of Drive Tests) framework to support data collection. When it comes to model delivery and transfer, the process occurs directly between the network and the UE. It is important to note that the network is not directly involved in the transfer of UE-sided models, which provides a degree of transparency in their delivery.

For 6G AI/ML LCM, the expanding adoption of AI/ML applications, coupled with the proliferation of devices and models, poses a challenge to Lifecycle Management. The problem is the signaling overhead, which can burden both the User Equipment (UE) and the network. For instance, a UE may execute multiple AI/ML operations. Managing the models' applicability in this scenario requires the UE to exchange signaling with the network to report applicability and control their activation/deactivation. Indivisual signaling for multiple AI/ML tasks in a UE and multiple AI/ML devices makes a signaling overheads. To tackle these issues, new signaling procedures are needed that prioritize efficiency and scalability.

For 6G AI/ML monitoring, AI/ML models require monitoring of the inference data to assess its similarity to the training environment. Inference data similar to the training environment is crucial for maintaining high performance of the AI/ML model; performance degrades significantly in dissimilar environments. For monitoring these degrading radio environments, the method must minimize the radio signaling used for monitoring and require the minimum necessary operations. Furthermore, it is essential to introduce and enhance common monitoring methods applicable across multiple AI/ML use cases within the radio environment monitoring framework.

For 6G AI/ML model switching, when the monitoring results detect a performance degradation, the system may fall back to a non-AI function, and the monitoring results should be reported to the network. Subsequently, the network initiates the process of switching to a new AI/ML model, typically proceeding through RRC signaling procedures. To address the need for reduced latency and overhead associated with these signaling procedures, which is vital for rapid model switching, we propose the following enhancement: Instead of improving the signaling procedure for each individual AI/ML model switching process, we propose establishing AI/ML model groups. The active model within a group is then configured based on the monitoring results. This approach requires signaling during the initial model group configuration, but the subsequent on-the-fly model changes within the group can be simplified and achieved without a full signaling procedure, thereby minimizing overhead and accelerating the switching time.

Observation 4: 5G NR LCM framework for NR air interface consists of data collection, model training, model storage, management, and inference, and defines interworking between them.

6G air interface data collection is intended to support diverse domains, such as AI/ML model training and inference for communication nodes and their applications.  The AI/ML is anticipated to be extensively deployed across both the radio access network (RAN) and the core network (CN) applied for a wide range of applications. To accommodate this extensive deployment, different LCM procedures have been defined across various Working Groups (WGs): R1/R2 for the radio interface, R3 for intra-RAN communication nodes, and SA for the core network. To enhance efficiency and inter-relationship, the AI/ML LCM should be harmonized and enhanced as a common function by leveraging the interdependencies with other WGs. Specifically, we must focus on a data collection strategy where radio-collected data can be utilized by the gNB/Core Network as well as by 3rd party AI/ML entities.

Proposal 3: Consider the NR LCM framework as baseline and study potential enhancements for 6G LCM

- Conclusions

In this paper, we discuss the framework for the data transfer and AI/ML framework. Followings are observations and proposals made in this paper:

Observation 1: 5G AI/ML data collection is currently supported on the control plane as an extension of the MDT framework, meaning the aforementioned data collection procedures are transmitted as control messages over the control plane connection.

Observation 2: Data transfer for AI/ML training involves a large data volume and is non-real-time, whereas data transfer for AI/ML inference involves a small data volume but has a real-time characteristic

Observation 3: 6G data collection requires a unified data transmission structure that supports AI/ML data for RAN, sensing data, and application AI/ML data.

Observation 4: 5G NR LCM framework for NR air interface consists of data collection, model training, model storage, management, and inference, and defines interworking between them.

Proposal 1: Study reusing measurement & report framework for AI/ML inference and monitoring.

Proposal 2: Study a unified data transfer to provide common delivery function for transporting a variety of data types including RAN AI/ML data, sensing data, and application AI/ML data, which is separate from the control plane.

Proposal 3: Consider the NR LCM framework as baseline and study potential enhancements for 6G LCM

- References

- RP-251395, Revised SID: Study on 6G Scenarios and requirements.

- RP-251881, New SID: Study on 6G Radio.

- TR 38.914, Study on 6G Scenarios and requirements.

- R2-2507564, Discussion on Common User plane and Control plane