3GPP TSG RAN WG2 Meeting #132	 R2-2508998

Dallas, USA, Oct. 17th-21st, 2025

Agenda Item:	10.3.2.1

Source: 	ITL

Title: 	Discussion on RRC modelling and spectrum aggregation for 6GR

Document for:	Discussion and decision

Introduction

In RAN#108, a new Study Item (SI) on 6G Radio (6GR) was approved [1]. This contribution discusses RRC modelling and spectrum aggregation with the intention of facilitating further study in RAN2. In RAN2#131bis, agreements related to RRC modelling and spectrum aggregation are as follows:

Agreements

Continue study

-	Idle mode –

-	Connected mode –

-	Inactive mode or sub-state: Should support at least on these functions: UE based mobility (e.g. cell selection/reselection), UE context and identification and energy efficient for both UE and NW.  Further study fast transitions, data transfer, further improvements to UE identification, and RNA management and paging/wake-up simplifications/enhancements.   We can consider the pros/cons of different modelling, after having some discussions on the functionality.

Agreements on Spectrum aggregation

- 1	Study spectrum aggregation of multiple pieces of spectrum and understand the issues that need to be addressed.

- 2	Study how to use UL and DL spectrum more efficiently (e.g. decoupling of UL and DL, etc).  Understand the deployment scenarios and problems to address

Discussion

RRC modeling

This contribution discusses the evolution of RRC modes from 4G/5G toward 6G, highlighting lessons learned from commercial deployment. Based on the analysis, this document proposes a simplified and flexible RRC model for 6G, which ensures energy efficiency, low latency, and scalable operation.

4G/5G networks have successfully commercialized RRC_CONNECTED and RRC_IDLE. The RRC INACTIVE was introduced in Rel-15 to improve signaling and latency trade-offs, but has not been widely deployed in 5G networks due to: RNA planning and architectural complexity, limited power saving benefit over IDLE, lack of use cases requiring low transition delay.

With 6G aiming at diverse service types (e.g., immersive, sensing-integrated, AI-driven), an optimized RRC model is needed to balance latency, energy efficiency, and mobility robustness.
In this regard, it becomes necessary to revisit the role of RRC INACTIVE mode or sub-states within the 6G RRC design.

In the previous RAN2 meeting, it was agreed that:

Inactive mode or sub-state: Should support at least on these functions: UE based mobility (e.g. cell selection/reselection), UE context and identification and energy efficient for both UE and NW.  Further study fast transitions, data transfer, further improvements to UE identification, and RNA management and paging/wake-up simplifications/enhancements.   We can consider the pros/cons of different modelling, after having some discussions on the functionality.

Therefore, in line with this consensus, the introduction of an RRC INACTIVE mode or sub-state in 6G is considered to achieve energy-efficient and flexible operation while supporting fast state transitions and robust UE identification.

As a starting point, RAN2 may first need to define what RRC INACTIVE and RRC sub-states represent in 6G, including their relationship with the well-defined CONNECTED and IDLE. In our understanding, the RRC INACTIVE defined in NR has several clear drawbacks, and the concept of a sub-state may serve as a complementary mechanism operating in parallel with the existing RRC modes to further enhance power efficiency.

We think that it would be preferable to define the sub-state as a new RRC mode in 6G that incorporates the beneficial functionalities of the RRC INACTIVE, such as small data transmission and fast transition (low latency and less signalling overhead), while ensuring power-saving efficiency.

We think that the following sub-state functionality can be considered for 6G.

- UE based mobility (cell selection/reselection): which has been well established in NR for cell selection and reselection in RRC IDLE and RRC INACTIVE, should be inherited and further applied to the 6G sub-state. This approach enables autonomous cell reselection by the UE without requiring network-triggered mobility signaling, thereby reducing signaling overhead and supporting energy-efficient mobility management.

- UE context and identification: the UE context and identification mechanism defined in NR should also be applied to the 6G sub-state with further enhancement. In the 6G sub-state, the UE may retain a minimal context in the RAN (e.g., QoS information, DRX configuration) and be identified using a lightweight context ID, instead of a full RNTI. This approach allows fast UE resume and paging with reduced signaling and enables efficient context management within the RAN cluster or anchor node.

- Energy efficient for both UE and NW: the UE can maintain deep sleep cycles while retaining minimal RAN context for fast activation, and the network can reduce control-plane signaling and context retrieval operations by avoiding frequent connection setups and RNA updates. Such a design enables both UE and network to minimize energy consumption during inactive periods while ensuring rapid service resumption when needed.

- (Further study) Data transfer: in order to support bursty and intermittent data traffic while maintaining high power efficiency, the 6G sub-state should include a small data transmission capability that allows the UE to send limited data without performing a full RRC resume procedure

- (Further study) Fast transition: In 6G, fast transition capability could be beneficial for scenarios such as event-driven IoT alerts and integrated sensing-and-communication (ISAC), where UEs may need to quickly switch from a low-power sub-state to the connected mode to transmit time-critical data.

In the 6G sub-state, the retained UE context can be utilized to enable fast connection resume, small data transfer, and efficient identification, while also supporting energy-efficient network operation.

Proposal 1: Introduce data transfer and fast transition in sub-state

In the 6G RRC sub-state, the UE is expected to maintain only a minimal set of context in the RAN to achieve fast transition, small data transmission, and energy-efficient operation. However, the amount and type of UE context required may differ depending on the sub-state functionality.

For example, small data transmission may require bearer or QoS information, while fast transition may need DRX or security parameters to be retained. A clear understanding of what information should be stored, where it resides, and how long it should be maintained is essential for optimizing both power efficiency and signaling complexity.

Proposal 2: Study of required UE context according to sub-state functionality

Number of Sub-states in RRC Modeling

The introduction of a sub-state in 6G aims primarily to enhance power saving and enable flexible state operation between RRC_CONNECTED and RRC_IDLE.
While the concept of a single sub-state may provide sufficient functionality through configurable parameters (e.g., DRX cycle, context retention level, or small data support), defining more than one sub-state could allow more fine-grained optimization for diverse service requirements.

From one perspective, a single sub-state simplifies state management, reduces signaling complexity, and maintains backward compatibility with the NR RRC model.
It could flexibly adapt to different use cases by parameter configuration, without introducing additional state transitions or network planning overhead.

From another perspective, multiple sub-states with different purposes (such as a light sub-state for deep sleep and a rich sub-state for fast transition or small data transmission) may provide better alignment with heterogeneous service requirements in 6G.

For example, latency-critical sensing or control applications may benefit from a sub-state retaining more context, while massive IoT devices may prefer a deeper sleep-oriented sub-state.

Further study is therefore needed to evaluate the trade-off between flexibility and complexity, including context retention, transition procedures, and signaling impact, to determine whether one or multiple sub-states would provide the optimal design for 6G.

Proposal 3: Study the number and functionality of sub-states in 6G

Spectrum aggregation

Multi-carrier serving cell

In 6G, the concept of a Multi-Carrier Serving Cell (MCSC) aims to support spectrum aggregation by treating multiple carriers as a single Serving Cell from a control and management perspective.
While resources are distributed across multiple frequency bands, the UE and the network can interact with them through unified Serving Cell procedures.

- In 5G NR, Bandwidth Part (BWP) allows the UE to monitor and operate on a portion of the carrier bandwidth to save energy.

- A UE can be configured with multiple BWPs, but only one DL BWP and one UL BWP can be active at a time per serving cell.

- When multiple carriers are aggregated (CA), each carrier (serving cell) still manages its own BWPs independently, which increases control overhead and limits coordination efficiency.

Proposal 4: Study the Multi-Carrier Serving Cell (MCSC) concept in 6G to enable efficient spectrum aggregation, considering the following aspects:

- Definition and scope of MCSC as an integrated serving cell framework across multiple carriers

- Impact of MCSC on HARQ operation, including entity configuration, process management, and timer operation

- Potential benefits in control simplification, energy efficiency, and cross-carrier coordination

HARQ entity/process in Multi-carrier serving cell

In NR, HARQ operation has been specified on a per-serving cell basis, with each serving cell maintaining its own HARQ entity and processes and associated timers. This design has proven effective in single-carrier serving cell environments, particularly for Carrier Aggregation (CA) operation, where independent HARQ management per serving cell provides robustness and flexibility.

Figure 1. Example of NR HARQ Entity for serving cell

In Figure 1, illustrates the structure of HARQ entities at the MAC layer in NR. Each serving cell (e.g., cell #X, #Y, #Z) is associated with its own independent HARQ entity, and within each HARQ entity multiple HARQ processes are maintained (e.g., Process 0 to Process 5). This representation highlights that HARQ operation is managed per serving cell, ensuring that retransmission processes, feedback timing, and timers are independently controlled even when multiple serving cells belong to the same cell group.

Figure 2. Example of Multi-carrier serving cell

Looking forward to 6G radio (6GR), the framework is expected to support tighter integration of multiple serving cells and carriers, possibly with unified scheduling and advanced aggregation concepts.

Figure 2 illustrates the difference between carrier-to-cell mapping in 5G NR and the envisioned multi-carrier serving cell (MCSC) concept in 6GR. In 5G NR, each carrier is associated with an individual serving cell (e.g., Carrier #A with Cell #X, Carrier #B with Cell #Y, Carrier #C with Cell #Z). In contrast, the MCSC approach, where multiple carriers (e.g., Carrier #A, Carrier #B, Carrier #C) can be integrated under a single serving cell (Cell #X). This unification allows for simplified cell management, reduced signaling overhead, and a more flexible HARQ operation framework across multiple carriers within one serving cell.

Together, these two illustrations show why the current per-serving cell HARQ entity, while effective in NR for CA scenarios, may need to be revisited for 6GR. With MCSC unifying multiple carriers under a single serving cell, HARQ process instantiation and timer operation could evolve from a strictly per-serving cell basis to group-based or unified structures, better aligned with the 6GR framework.

Observation 1: The NR approach of maintaining a separate HARQ entity per serving cell is not well suited for the 6GR MCSC environment, where multiple carriers are unified under a single serving cell

Therefore, in 6GR MCSC environments, a clear definition of how HARQ entities and processes should be instantiated and operated is required to ensure consistent timer alignment, process indexing, and feedback handling across carriers.

- How HARQ process indexing and entity instantiation should be adapted for MCSC.

- Methods for aligning HARQ timers across carriers when the serving cell boundary is unified.

- Mechanisms for efficient feedback signaling (ACK/NACK) across multiple carriers under one HARQ entity.

Several potential approaches can be considered for HARQ configuration and operation in multi-carrier serving cell environments:

- Option A: Maintain per-serving cell HARQ operation (NR approach)
– Each serving cell retains independent HARQ processes and timers.

- Option B: Group-based HARQ operation across serving cells
– Serving cells within a group share HARQ configuration and may pool HARQ processes.

- Option C: Unified HARQ operation across multiple serving cells
– Multiple serving cells are treated as a single HARQ entity with unified process indexing and timers.

The outcome of this study will provide a baseline for defining HARQ operation that is both efficient and robust in future 6GR deployments supporting MCSC.

Proposal 5: Study HARQ operation in the 6GR MCSC environment, considering the following aspects:

- Definition of HARQ entity instantiation and process management in MCSC

- Alignment of HARQ timers across multiple carriers within a single serving cell

- Efficient design of HARQ ACK/NACK feedback signaling in unified HARQ operation

Conclusion

In this contribution, we discussed the RRC modelling and spectrum aggregation aspects. According to the discussion in Section 2, we have the following observation and proposals:

Proposal 1: Introduce data transfer and fast transition in sub-state

Proposal 2: Study of required UE context according to sub-state functionality

Proposal 3: Study the number and functionality of sub-states in 6G

Proposal 4: Study the Multi-Carrier Serving Cell (MCSC) concept in 6G to enable efficient spectrum aggregation, considering the following aspects:

- Definition and scope of MCSC as an integrated serving cell framework across multiple carriers

- Impact of MCSC on HARQ operation, including entity configuration, process management, and timer operation

- Potential benefits in control simplification, energy efficiency, and cross-carrier coordination

Observation 1: The NR approach of maintaining a separate HARQ entity per serving cell is not well suited for the 6GR MCSC environment, where multiple carriers are unified under a single serving cell

Proposal 5: Study HARQ operation in the 6GR MCSC environment, considering the following aspects:

- Definition of HARQ entity instantiation and process management in MCSC

- Alignment of HARQ timers across multiple carriers within a single serving cell

- Efficient design of HARQ ACK/NACK feedback signaling in unified HARQ operation

Reference

RP-252912 revised SID: Study on 6G radio