3GPP TSG-RAN WG2 #132							R2-2508999

# Dallas, USA, 17th - 21st  November 2025

Agenda Item:	10.3.3.1

Source:  LG Uplus

Title: 	ANR for NTN-TN integration

Document for:	Discussion

Introduction

Regarding TN and NTN, mobility protocols are well-defined in 5G. However, basic and essential SON features such as ANR is not clear. In this contribution, an exemplary deployment scene and procedure for ANR between TN and NTN will be described and, correspondingly, requirements are proposed.

Discussion

As for NTN in transparent mode, a NTN gNB will be located far from a relevant TN gNB, wherein TN and NTN will require separate AMFs/MMEs for efficiently managing transport network address. This situation is different from legacy TN deployment, where all neighbor relations are sufficiently close within a MME or within neighbor MME. Hence, the neighbor relation request mechanism between TN and NTN should be developed.

Proposal 1: ANR mechanism shall be specified between TN and transparent mode NTN where NTN gNB is located far from a TN gNB.

For NTN in generative mode, the relationship between TN and NTN would be changed if NTN gNB is moving. NRT (Neighbor Relationship Table) in TN and NTN gNBs would be managed according to time and location. Correspondingly, novel ANR mechanisms should be developed.

Proposal 2: ANR mechanism shall be specified between TN and regenerative mode NTN where neighbour relationship between TN and NTN would be changed according to NTN gNBs transversing their orbit.

In commercial system, NRT usually is utilized in the gNA, the size of which is one of important design of gNB storage and capacity. If a NRT is fully filled, an additional neighbour would be rejected or a neighbour cell which has the least number of HO trials would be removed from the NRT.

For NTN in generative mode, the NTN gNB continue to move over the earth, which means NRT should consider all TN and NTN gNBs at least on the same orbit. However, it seems not realistic that NRT have the list of all TN and NTN gNBs at least for the same orbit. The sophisticated mechanism to manage NRT should be designed.

Proposal 3: In NTN/TN interoperation, the advanced NRT management scheme shall be designed which enable to maintain the reasonable size of NRT for sustaining searching efficiency.

Conclusion

In this contribution, the below proposals are introduced for 6G TN/NTN integration, which need to be realistic operation management.

Proposal 1: ANR mechanism shall be specified between TN and transparent mode NTN where NTN gNB is located far from a TN gNB.

Proposal 2: ANR mechanism shall be specified between TN and regenerative mode NTN where neighbour relationship between TN and NTN would be changed according to NTN gNBs transversing their orbit.

Proposal 3: In NTN/TN interoperation, the advanced NRT management scheme shall be designed which enable to maintain the reasonable size of NRT for sustaining searching efficiency.