3GPP TSG-RAN WG2 Meeting #132             			            R2-2509000

Dallas, USA, Nov. 17th - 21st

Agenda item:	10.3.3.1

Source:	TCL

Title:	Discussion on 6G general AI/ML framework

Document for:	Discussion and Decision

1	Introduction

At the RAN2#131bis meeting, the following agreements were reached on 6G AI/ML LCM [1].

Agreements on LCM Study 6G AI/ML LCM framework that supports at least the following functions/procedures: - UE AI/ML Capabilities Exchange; - Applicable Functionality Reporting; - Inference Configuration and Reporting; - Performance Monitoring Configuration and Reporting; - Functionality (De-)Activation and Fallback/Switching to the AI/ML/non-AI/ML Functionality;

In this contribution, we will show our views on the general AI/ML framework in 6G.

2. Discussion

2.1 LCM framework

In 5G, the SA and RAN working groups have independently defined functional frameworks for their respective AI/ML use cases. For example, RAN1 has explored a general AI/ML framework for the NR air interface based on the functional framework defined by RAN3, as illustrated in Figure 1 [2]. This framework provides technical support for use cases addressed by RAN1 and RAN2, such as CSI feedback enhancement, beam management, positioning accuracy enhancement, CSI compression, and AI/ML-based mobility, to cover a general functional architecture addressing both model-ID-based LCM and functionality-based LCM.

Figure 1: Functional framework for AI/ML for NR Air Interface

It is observed that several potential issues with the multiple AI/ML functional frameworks, including functional redundancy, a risk of specification conflicts, and low efficiency in end-to-end collaboration. Considering that the development of 6G native AI may require an efficient underlying architecture, it is proposed that, in the early phase of the 6G study, a future-proof AI/ML functional framework should be considered, which is intended to support a unified LCM framework including data collection, inference, fallback, etc. Hence, it is suggested that 6G should study a unified LCM framework designed to support diverse use cases across different WGs in a harmonized manner, using the 5G-Advanced AI/ML LCM framework for the NR air interface as a baseline.

In 5G, RAN and SA have respectively defined their functional frameworks for AI/ML to cover logical LCM procedures within their domains.

6G should study a unified LCM framework designed to support diverse use cases, and coordinate with other WGs if needed.

2.2 UE AI/ML Capabilities Exchange

In 5G, AI/ML capabilities are typically reported in a static manner (UECapabilityEnquiry-UECapabilityInformation procedure). In this mechanism, the gNB requires the UE to report its AI/ML capabilities based on the network configuration and its hardware capabilities. However, this static capability reporting mechanism has some limitations, particularly in terms of real-time responsiveness, redundancy, and flexibility, as it cannot promptly reflect the dynamic changes in the UE AI/ML capabilities. Moreover, “applicable functionality reporting” can be regarded as another capability reporting mechanism that allows the UE to actively report its AI/ML functionality changes under certain conditions. Although this approach offers some flexibility, it is still constrained by network configuration and typically relies on periodic reporting or event-triggered reporting methods. Therefore, while it can respond to changes under specific conditions, this mechanism cannot dynamically adjust in real-time, and fails to promptly adapt to changes in network or UE conditions, which impacts the efficiency of resource optimization and resource scheduling.

In 6G, we think that RAN2 should study a dynamic UE AI/ML capabilities exchange mechanism, which enables the network to obtain the UE AI/ML capability changes in real-time, and improve resource utilization efficiency.

In 6G, RAN2 is suggested to study a dynamic UE AI/ML capabilities exchange mechanism to improve the resource efficiency.

2.3 Applicable Functionality Reporting

In 5G, the applicable functionality reporting procedure is proposed for UE-side models, enabling the UE to indicate to the network whether the current UE’s AI/ML functionality is applicable or not. The basic procedure [3] of applicability reporting is illustrated in Figure 2. The UE reports its functionality applicability in the RRCReconfigurationComplete message, or it can report an updated applicability status in the UEAssistanceInformation message

Figure 2: Initial applicability and applicability status change reporting

However, whether the applicable functionality reporting is carried by the RRCReconfigurationComplete message or the UE Assistance Information (UAI) message, the reporting procedure is still controlled by the RRCReconfiguration by the network, and the reporting latency is relatively long. As a result, the network cannot promptly determine whether the configured AI/ML functionality remains applicable under changing UE or network conditions. In addition, this procedure was initially designed based on RAN1 use cases (e.g., AI-based BM), but for RAN2 use cases such as AI/ML-based mobility, the applicable functionality reporting procedure may differ. In other words, the current applicable functionality reporting mechanism is defined per use case, which increases the specification complexity. Therefore, in our view, it is considered that the applicable functionality reporting should be enhanced in 6G to support a unified and flexible mechanism.

In 6G, RAN2 is suggested to enhance the applicable functionality reporting to achieve a unified and flexible mechanism.

2.4 Inference Configuration and Reporting

In 5G, the inference configuration and reporting procedures were designed on a per-use-case basis, for example, separately specified for AI/ML-based CSI feedback enhancement, beam management, positioning enhancement, and AI/ML-based mobility. While this approach enabled early standardization of AI/ML functions, it also resulted in several structural drawbacks. Each use case introduces its own set of inference configuration parameters, reporting types, and so on, leading to fragmented procedures across different AI/ML-based use cases. Moreover, the current inference configuration is built upon existing 5G RRC and measurement signaling structures, which were originally designed for static configuration and periodic reporting. When reused for AI/ML inference, these procedures require additional control signaling to configure model-related parameters, trigger inference results, and collect feedback from the UE. As a result, the signaling overhead increases significantly.

Therefore, for 6G, it is necessary to establish a unified inference configuration and reporting mechanism that provides a common signaling structure for diverse AI/ML use cases, which would reduce redundant signaling and simplify configuration procedures.

In 6G, RAN2 is suggested to consider a unified inference configuration and reporting mechanism.

2.5 Data collection

Data collection is an essential aspect serving various purposes in LCM, e.g., model training, model inference, model monitoring, model selection, model update. Study on AI/ML data collection in 5G-Advanced have investigated both Control Plane (CP) based solution and User Plane (UP) based solution. It is observed that these approaches rely on the reuse of existing approaches, which in turn lead to the use of separate, use-case specific mechanisms for data collection. For example:

- For the AI/ML-based BM use case, data collection is based on the legacy CSI framework.

- For the AI/ML-based positioning use case, the LPP protocol is utilized for data collection.

- For the AI/ML-based mobility use case, legacy RRM measurement procedures are reused for data collection.

One the other hand, the same AI/ML data may be collected multiple times for different AI/ML use cases. For example, L1 beam measurement results are applicable to both AI/ML-based beam management and AI/ML-based mobility.

Based on above discussion, we think that in order to improve efficiency for data collection, it is important to design a harmonized data collection framework for 6G AI/ML. Hence, 6G should specify a unified data collection framework to streamline data collection, thereby improving efficiency and eliminating redundancy.

In 6G, RAN2 should specify a unified data collection framework to enhance the efficiency of data collection and prevent redundancy.

For data collection in 5G, UE can be configured by gNB to log L1 measurements in the AS buffer and report them via a RRC message(s) for NW-side model training. However, since each RRC message is encapsulated into PDCP SDUs over the SRB, its size is typically limited to about 9 kB, and with a maximum segmentation size of approximately 144 Kb. Obviously, the CP-based method is not suitable for transferring large-volume AI/ML training data. When multiple segments are required, the UE must perform repeated RRC transmissions, which significantly increases signalling overhead. Moreover, this method shares the same RRC resources as essential control signalling, and frequent AI/ML data transmission may cause congestion of the CP signalling. Alternatively, AI/ML data can be conveyed via Data Radio Bearers (DRBs) in UP-based method. The DRB is mapped to a QoS Flow within a PDU Session, which is terminated at the User Plane Function (UPF) in the core network. However, because DRB traffic is encrypted and transparent to the gNB, the RAN has no visibility over the AI/ML data. Consequently, the network cannot manage AI/ML data locally.

In summary, both the legacy CP- and UP-based solutions were designed for traditional control or user traffic and are not scalable for AI/ML data transfer in 6G. Therefore, it is necessary for RAN2 to study more efficient AI/ML data collection mechanisms, which may include enhancing SRB/DRB or introducing a new RB (e.g., AI/ML RB) to support large, flexible, and low-latency AI/ML data transmission.

In 6G, RAN2 is suggested to study how to efficiently transfer AI/ML data, e.g., enhance the SRB, DRB, or introduce a new RB.

2.6 AI agent

In 5G-Advanced, it has been confirmed through extensive evaluation that an AI/ML model can enhance communication efficiency. For example, in AI/ML-based mobility, the predicted RRM measurement results can improve handover performance and reduce measurement efforts.

Recently, it has been widely recognized that AI agents play a key role in enabling intelligent automation in wireless communication systems. SA2’s document [4] has provided the definition of the AI agent:

Al Agent: an automated intelligent entity capable of e.g interacting with its environment, acquiring contextual information, reasoning, self-learning, decision-making, executing tasks (autonomously or in collaboration with other Al Agents) to achieve a specific goal.

To realize the 6G native AI of a fully autonomous network with ubiquitous intelligence. AI agents, as intelligent entities with perception, learning, reasoning, and execution capabilities, are well-suited to fulfill this role. Therefore, it is proposed that 6G should natively support AI agents on both UE side and NW side, to enhance improve system performance.

6G should natively support AI agents on both UE side and NW side, to improve the system performance.

3	Conclusion

Based on the discussion in the previous sections, we propose the following:

- In 5G, RAN and SA have respectively defined their functional frameworks for AI/ML to cover logical LCM procedures within their domains.

- 6G should study a unified LCM framework designed to support diverse use cases, and coordinate with other WGs if needed.

In 6G, RAN2 is suggested to study a dynamic UE AI/ML capabilities exchange mechanism to improve the resource efficiency.

In 6G, RAN2 is suggested to enhance the applicable functionality reporting to achieve a unified and flexible mechanism.

In 6G, RAN2 is suggested to consider a unified inference configuration and rreporting mechanism.

In 6G, RAN2 should specify a unified data collection framework to enhance the efficiency of data collection and prevent redundancy.

In 6G, RAN2 is suggested to study how to efficiently transfer AI/ML data, e.g., enhance the SRB, DRB, or introduce a new RB.

6G should natively support AI agents on both UE side and NW side, to improve the system performance.

References

[1] RAN2#131bis, Chairman notes.

[2] TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface.

[3] TS 38.300, NR; NR and NG-RAN Overall Description.

[4] TR 22.870, Study on 6G Use Cases and Service Requirements.