3GPP TSG-RAN WG2 Meeting #132	R2-2509001

Dallas, USA, 2025，Nov. 17th - 21st

Agenda item:	8.1.2

Title:	Considerations on the remaining issue of UE side model

Source:	ZTE Corporation

Document for:	Discussion and Decision

Introduction

In RAN2#131bis meeting, following agreements were reached:

Agreements

1	[N021/H003/A105/S047] The parameters for beam management performance monitoring and CSI prediction performance monitoring are grouped in two separate fields.

2	[N021/H003/A105/S047] The parameters for beam management inference and beam management UE-side data collection are grouped in the same field. This field’s name should be general enough to cover both inference and data collection.

3	[N021/H003/A105/S047] (7/11 companies agree) A CHOICE structure is kept for the fields containing parameters introduced in CSI-ReportConfig for AIML PHY Rel-19.

4	[Z004/J008/J009] The UE logs a cell/configuration ID in VarCSI-LogMeasReport only when a measurement result for this cell/configuration is logged (and not upon receiving the logging configuration).

5	[Z004/J008/J009] The UE discards logged cell/configuration IDs from VarCSI-LogMeasReport, if there are no more logged measurements for them after reporting measurements in UEInformationResponse.

6	[H008/B204] RILs H008/B204 to be resolved in the CR, based on changes for N021/H003/A105/S047.

7	[S050] RIL S050 to be resolved after RAN2#131bis.

8	[H002] No specification changes are made to explicitly exclude or to enhance retaining logged data for LTM.

9	[Z005/H009] RILs Z005/H009 to be resolved after receiving a reply from RAN1.

10	[Z007] RIL Z007 to be resolved after RAN2#131bis.

Agreements

1	[X003]: Change the name of the “start” indication to a “request” indication. The condition of sending this “request” indication (if it is configured by NW to do so) is the UE has no candidate list. Update the procedural text in RRC to reflect this condition.

2	[O301]/[S045]/[N114]: Enhancing the “request” indication to be per-use case and/or per-serving cell will not be pursued in Release 19.

Agreements

- [B206][O300] A new IE in HandoverPreparationInformation message to convey complete applicability information.

This contribution provides consideration on some remaining issue and RIL for LCM of UE side model.

Discussion

In RAN2#131, RAN2 agreed that any previously configured inference configurations for which applicability/inapplicability has already been reported and whose applicability status has changed since the last report can be included in RRCReconfigurationComplete message.

However, it is still unclear whether UE can activate such configured inference configuration if it is configured with periodic CSI reporting and its applicability status is reported as applicable via RRCReconfigurationComplete.

Agreements on applicability/inapplicability

1	RRCReconfigurationComplete shall include applicability/inapplicability status for:

a)	All inference configurations included in the immediately preceding RRCReconfiguration message, and

b)	Any previously configured inference configurations for which applicability/inapplicability has already been reported and whose applicability status has changed since the last report.   [CB on how the UE handles previously configured periodic CSI config that becomes applicable]

2	Do not introduce a link, explicit or implicit, between a full inference configuration and a set of inference-related parameters. If the applicability of a full inference configuration changes and there is a corresponding set of inference-related parameters whose applicability changes at the same time, the UE shall report the applicability of both

Basically there are two options on the whether to activate the periodic CSI configuration when it becomes applicable from inapplicable and it is reported via RRCReconfigurationComplete message:

- Option 1: To follow handling for the initial configured periodic inference configuration that is included in the immediately preceding RRCReconfiguration. UE automatically activates the periodic CSI configuration upon transmitting applicability report via RRCReconfigurationComplete.

- Option 2: To follow the previous RAN2 agreements regarding the activation of periodic CSI configuration if the applicability report is transmitted via UAI. I.e. UE should not activate the periodic CSI configuration but only report applicability to network. Whether to activate the periodic CSI configuration is up to network decision.

From NW point of view, if option 1 is allowed, it has to handle the different activation status of the configured periodic CS configuration when it is reported as applicable from inapplicable via different messages:

- The periodic CSI configuration is activated when it is reported via RRCReconfigurationComplete

- The periodic CSI configuration is not changed when it is reported via UAI.

Observation 1: 	If the handling of periodic CSI configurations when reporting their applicability via RRCReconfigurationComplete and UAI is misaligned, the network will handle two distinct activation status for the periodic CSI configurations: activated when reported via RRCReconfigurationComplete, and unchanged when reported via UAI.

Further, NW has to keep providing the related resource for all configured periodic CSI configuration even if UE reports it as inapplicable because the periodic CSI Configuration may be activated by the UE automatically at any time. This is not efficient for radio resource utility and impose unnecessary restriction to network side implementation.

Observation 2:	If the UE is allowed to automatically activate a periodic CSI configuration when its applicability changes from inapplicable to applicable, the network must always provide the associated resources, even when the periodic CSI configuration is reported as inapplicable, since the UE may activate it automatically at any time.

Some companies may have concerns on the complexity in UE and NW implementation if UE is allowed to report the change of applicability for periodic CSI configuration via RRCReconfigurationComplete message.

More specifically, these companies think the UE has to classify received periodic CSI configuration into two sets: the set configured in immediate RRCReconfiguration message, which can be automatically activated if it is applicable, and the second set that is received in previous RRCReconfiguration message, which can not be automatically activated. And UE has to continuously update these two sets and track the activation status of the configured CSI configuration, which is actually contradictory to the intention of delta 	signaling..

Observation 3: 	Some companies may have concerns about the implementation complexity for both UEs and network if they are required to continuously track the activation status of periodic CSI configurations based on whether applicability reporting is an initial report or a change in applicability.

However, we have different views on the complexity issue. We understand the complexity to track the activation status is not caused by enabling reporting of the change of applicability via RRCReconfigurationComplete message. It already exists when change of applicability can only be reported via UAI.

This is because we currently have two kind mechanisms to activate the periodic CSI configuration: automatically activation if it is configured in immediate RRCReconfiguration, and activation via RRCReconfiguration if it is previous configured and becomes applicable from inapplicable.

Further, the tracking of activation status becomes more complicated if we take into account the behavior when UE report inapplicable to NW.

- If UE indicates release preference when report inapplicability, the NW may release the periodic CSI configuration.

- if UE does not indicate release preference wen report inapplicability, NW may have two options:

- option 1: NW may do nothing but just keep the periodic CSI configuration in UE side and keep its  activation status (activated or not activated depending on whether the configuration is already submitted to lower layer).

- option 2: NW may perform release and add to configure a new periodic CSI configuration in UE side and keep it not in activated state, to let UE perform applicability reporting later.

Following is a summary of the activation status of periodic CSI configuring in various cases:

| Activation status of periodic CSI config | Upon initial applicability reporting | Upon reporting change of applicability |
| --- | --- | --- |
| Report as applicable | Is automatically activated | Case 1: is in activated state, if it was activated before and not released by NW when it once became inapplicable. Case 2: is not in activated state, if was is not activated before, e.g. not applicable upon initial report and now become applicable |
| Report as inapplicable | Is not activated | Case 3: is in activated state, if it was activated before and was not released, e.g. applicable upon initial report and now becomes inapplicable Case 4: is not in activated state, if it was not activated by NW before, e.g. UE reported inapplicable but not reconfigured by NW before, and now it becomes inapplicable |

From the summary above, we can see the activation status is not very related to which message is used to report the change of applicability of a periodic CSI configuration. The complexity is more related to the activation/deactivation mechanism for the periodic CSI configuration.

Observation 4: 	The complexity of tracking the activation status of configured periodic CSI configurations is not related to the RC message used to report the change of applicability. Instead, it is more related to the activation and deactivation mechanisms for periodic CSI configurations:

- The activation mechanism for the periodic CSI configuration are different upon initial reporting and reporting for change of applicability.

- Periodic CSI configuration cannot be automatically deactivated by the UE upon it becomes inapplicable.

We think such complexity is not a real issue since UE and NW can track the activation status according to the  rule. For example, UE implementation can follow following rule:

- Upon initial reporting for a periodic CSI configuration, UE automatically activate it if it is applicable. Otherwise, UE keep it not activated.

- In all other cases (applicability change to applicable/inapplicable), UE just report the applicability, and perform configuration according to RRCReconfiguration message.

Based on above discussion, we propose RAN2 to firstly agree to keep previous RAN2 agreement, i.e. UE can report the change of applicability for periodic CSI configuration via RRCReconfigurationComplete message.

proposal 1:	RAN2 to confirm previous RAN2 agreement: “RRCReconfigurationComplete shall include applicability/inapplicability status for b)Any previously configured inference configurations for which applicability/inapplicability has already been reported and whose applicability status has changed since the last report”.

If reporting the change of applicability for configured periodic CSI configuration via RRCReconfigurationComplete is still allowed, we have following two proposals.

Proposal 2: 	The UE's behavior when reporting the change of applicability for a previous configured periodic CSI configuration via RRCReconfigurationComplete messages should be aligned with that via UAI messages.

Proposal 3:     If reporting the change of applicability for a periodic CSI configuration is transmitted via RRCReconfigurationComplete message, and the applicability is changed from inapplicable to applicable, UE shall NOT activate the periodic CSI config automatically.

After all, if companies really have concern on the complexity of such activation status tracking, we propose RAN2 to consider one of following options :

  - Option 1: To unify the activation/deactivation mechanism for periodic CSI configuration in all cases, e.g. UE automatically activate periodic CSI configuration when it is/becomes applicable and automatically deactivate it when it is/becomes inapplicable. thus UE does not need to track the activation status, but only perform activation/deactivation according to the applicability status.

  - Option 2: To not support periodic CSI configuration for AIML PHY. Instead, semi-persistent CSI configuration can be used which has no such complexity.

Considering now it is in late stage of Rel-19, we think such decision should be very carefully.

Proposal 4:     If companies have concern on the implementation complexity in tracking activation status of periodic CSI configuration, RAN2 is suggested to discuss following option:

- Option 1: UE automatically activate periodic CSI configuration when it is/becomes applicable and automatically deactivate it when it is/becomes inapplicable.

- Option 2: Periodic CSI configuration is not supported to be used for inference configuration.

In RAN1 LS[3], RAN1 has discussed when RRC layer submit periodic CSI inference configuration to layer and has reach conclusion:

RAN1 thanks RAN2 for the LS on when RRC layer submits periodic CSI inference configuration to lower layer. RAN 1 discussed the two options identified by RAN 2: - Option 1: Upon reception of RRC Reconfiguration message, UE’s RRC layer immediately submits inference configuration of periodic CSI to lower layer, regardless of whether the configuration is applicable/inapplicable. - Option 2: Upon reception of RRC Reconfiguration message, UE’s RRC layer submits inference configuration of periodic CSI to lower layer only if it is reported as applicable in RRCReconfigurationComplete.. From RAN1 perspective, Option 2 can resolve the issue. No additional RAN 1 impact was identified for Option 2.

According to the RAN1 LS,RAN1 suggest to adopt option 2, i.e. upon reception of RRCReconfiguration message, UE’s RRC layer submits inference configuration of periodic CSI to lower layer only if it is reported as applicable in RRCReconfigurationComplete.

We agree with the intention of the LS. The RRC layer shall not submit the inapplicable periodic CSI config to lower layer. Thus the physical layer don’t need to have activate/deactivate behavior for the periodic CSI configuration. The physical layer apply/activate the periodic CSI config once it is submitted to physical layer, and stop/remove the periodic CSI configuration once the periodic CSI configuration is deleted by NW via another RRCReconfiguration message.

Further, based on discussion related to whether UE automatically activate periodic CSI configuration when reporting the change of applicability for it, we think the RRC layer shall submit periodic CSI config to lower layer only if it is included/configured in the immediate RRC reconfiguration message and it is reported as applicable in RRCReconfigurationComplete. In other words, those periodic CSI configuration that is configured before and it is reported as applicable in the RRCReconfigurationComplete message because its applicability status changes from inapplicable to applicable should not be submitted to lower layer. This is aligned with the behavior of reporting applicability via UAI message.

proposal 5:	RAN2 to confirm that Upon reception of RRCReconfiguration message, UE’s RRC layer submits periodic CSI configuration to lower layer only if it is included in the immediate RRCReconfiguration message and it is reported as applicable in the RRCReconfigurationComplete.

a TP is provided in Annex A.

In our understanding, this proposal is more or less contradict with RAN1’s understanding. RAN1 may think as long as UE report applicability for a periodic CSI configuration as applicable via RRCReconfigurationComplete, UE will activate it automatically. Thus we suggest RAN2 to send LS to RAN1 to inform them of this and ask them whether they have any concerns.

proposal 6:	RAN2 to send LS to RAN1 to indicate the conclusion in RAN2 and ask whether they have any concerns if periodic CSI configuration configured in previous RRCReconfiguration message and is reported as applicable in RRCReconfigurationComplete message is not activated automatically by the UE.

In last RAN2 meeting, RAN2 reached some agreements related to applicability reporting for SCG inference configuration received in RRCResume. And at the same time, RAN2 agreed that for no NR-DC enhancements are considered and will not ask RAN3 work in Rel-19[1]

Agreements

- The procedures for UE data collection request for the CSI prediction use case are the same as for the beam management use case.

- The applicability reporting procedures for CSI prediction are the same as for beam management. RAN2 confirms that option B is not supported for CSI prediction, given no parameters were provided by RAN1.

- RAN2 understand that when network releases inference configurations of poor-performed applicable functionalities, network may also provide either non-AI/ML configuration in CSI-ReportConfig or may provide full inference configuration of other applicable functionalities, if previously not configured to UE. There is no spec impact and feedback from the NW to the UE to adjust the applicability determination procedure is not supported in Rel-19.

- No enhancements are pursued for reporting applicability in RRCReestablishmentComplete.

- The UE can report applicability via RRCResumeComplete for SCG inference configurations received in RRCResume, without specification impact beyond already agreed applicability reporting procedure.

- Applicability reporting is added in RRCResumeComplete for inference configurations that exist at the UE based on legacy procedures (restored or received in RRCResume).

- RAN2 assumes for no NR DC enhancements are considered and will not ask RAN3 work in Rel-19.

As the discussion held in last RAN2 meeting, during the RRC resume procedure, including inference configuration in RRCResume message and including applicability reporting in RRCResumeComplete for SCG cell comes for free. This is because the SCG configuration is included in RRCResume as a RRC container which includes a Reconfiguration for SCG, and the corresponding RRCReconfigurationComplete is included in RRCResumeComplete and transmitted via PCell. Thus nothing needs to be changed to support inference configuration for SCG cells.

With the similar reason, configuring inference configuration and/or inference related parameter set for SCG cells via RRCReconfiguration are already supported since the SCG RRCReconfiguration use the same IE as that for MCG cells. Thus all IEs introduced for these two use cases can be applied for SCG too.

On the other hand, there is no inter-node negotiation mechanism to negotiate UE's PU capability between MN and SN. Since the MN and SN can configure inference configuration independently, they may compete for the UE’s PU capability. Considering RAN2 has already agreed there will be no NR-DC enhancement and will not ask RAN3 work in Rel-19, it is up to UE decision on the applicability of inference configuration configured for MCG cell and SCG cell if they are configured at same time. I.e. UE determines applicability according to its PU capabilities.

Based on above consideration, we have following proposal:

Proposal 7: 	RAN2 to confirm that beam management and CSI prediction use case are supported for SCG cells in NR-DC configuration without need for NR-DC enhancement. When inference configurations for both MCG cell and SCG cell are included in same RRC message, It is up to UE implementation to decide the applicability.

Conclusion

Observation 1: 	If the handling of periodic CSI configurations when reporting their applicability via RRCReconfigurationComplete and UAI is misaligned, the network will handle two distinct activation status for the periodic CSI configurations: activated when reported via RRCReconfigurationComplete, and unchanged when reported via UAI.

Observation 2:	If the UE is allowed to automatically activate a periodic CSI configuration when its applicability changes from inapplicable to applicable, the network must always provide the associated resources, even when the periodic CSI configuration is reported as inapplicable, since the UE may activate it automatically at any time.

Observation 3: 	Some companies may have concerns about the implementation complexity for both UEs and network if they are required to continuously track the activation status of periodic CSI configurations based on whether applicability reporting is an initial report or a change in applicability.

Observation 4: 	The complexity of tracking the activation status of configured periodic CSI configurations is not related to the RRC message used to report the change of applicability. Instead, it is more related to the activation and deactivation mechanisms for periodic CSI configurations:

- The activation mechanism for the periodic CSI configuration are different upon initial reporting and reporting for change of applicability.

- Periodic CSI configuration cannot be automatically deactivated by the UE upon it becomes inapplicable.

Summary of the activation status of periodic CSI configuring in various cases

| Activation status of periodic CSI config | Upon initial applicability reporting | Upon reporting change of applicability |
| --- | --- | --- |
| Report as applicable | Is automatically activated | Case 1: is in activated state, if it was activated before and not released by NW when it once became inapplicable. Case 2: is not in activated state, if was is not activated before, e.g. not applicable upon initial report and now become applicable |
| Report as inapplicable | Is not activated | Case 3: is in activated state, if it was activated before and was not released, e.g. applicable upon initial report and now becomes inapplicable Case 4: is not in activated state, if it was not activated by NW before, e.g. UE reported inapplicable but not reconfigured by NW before, and now it becomes inapplicable |

proposal 1:	RAN2 to confirm previous RAN2 agreement: “RRCReconfigurationComplete shall include applicability/inapplicability status for b)Any previously configured inference configurations for which applicability/inapplicability has already been reported and whose applicability status has changed since the last report”.

Proposal 2: 	The UE's behavior when reporting the change of applicability for a previous configured periodic CSI configuration via RRCReconfigurationComplete messages should be aligned with that via UAI messages.

Proposal 3:     If reporting the change of applicability for a periodic CSI configuration is transmitted via RRCReconfigurationComplete message, and the applicability is changed from inapplicable to applicable, UE shall NOT activate the periodic CSI config automatically.

Proposal 4:     If companies have concern on the implementation complexity in tracking activation status of periodic CSI configuration, RAN2 is suggested to adopt one of following options:

- Option 1: UE automatically activate periodic CSI configuration when it is/becomes applicable and automatically deactivate it when it is/becomes inapplicable.

- Option 2: Periodic CSI configuration is not supported to be used for inference configuration.

proposal 5:	RAN2 to confirm that Upon reception of RRCReconfiguration message, UE’s RRC layer submits periodic CSI configuration to lower layer only if it is included in the immediate RRCReconfiguration message and it is reported as applicable in the RRCReconfigurationComplete.

proposal 6:	RAN2 to send LS to RAN1 to indicate the conclusion in RAN2 and ask whether they have any concerns if periodic CSI configuration configured in previous RRCReconfiguration message and is reported as applicable in RRCReconfigurationComplete message is not activated automatically by the UE.

Proposal 7: 	RAN2 to confirm that beam management and CSI prediction use case are supported for SCG cells in NR-DC configuration without need for NR-DC enhancement. When inference configurations for both MCG cell and SCG cell are included in same RRC message, It is up to UE implementation to decide the applicability.

References

- RAN2#131 chairman notes

- RAN2#131bis chairman notes

- R1-2508122, LS reply on RAN2 LS on periodic CSI inference configuration to lower layer

- R1-2508140, Reply LS on candidate data collection,

Annex A

NEXT CHANGE

#### 5.3.5.3	Reception of an RRCReconfiguration by the UE

<Text Omitted>

2>	if, for at least one serving cell, the RRCReconfiguration message includes in csi-ReportConfigToAddModList at least one CSI-ReportConfig including csi-InferencePrediction, or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19; or

2>	if the RRCReconfiguration message includes at least one entry in applicabilityConfigToAddModList within applicabilityReportConfig; or

2>	if, for at least one serving cell, the UE is configured with at least one reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction, or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19, for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (in RRCReconfigurationComplete or in UEAssistanceInformation or in RRCResumeComplete); or

2>	if the UE is configured with at least one entry in applicabilitySetConfigCSI-ToAddModList for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either in RRCReconfigurationComplete or in UEAssistanceInformation):

3>	for each serving cell associated with any of the configurations above, include an entry in the applicabilityReportList and set the content as follows:

4>	set the applicabilityCellId to the serving cell index of the cell;

4>	for each configured reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction, or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19, that is included in the RRCReconfiguration message or for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (in RRCReconfigurationComplete or in UEAssistanceInformation or in RRCResumeComplete):

5>	include an entry in the applicabilityInfoReportList and set the content as follows:

6>	set the csi-ReportConfigId within applicabilityInfoReportId to the corresponding reportConfigId;

6>	set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityInfoReportId;

6>	if the applicabilityStatus is set to inapplicable:

7>	if the UE prefers to release the concerned CSI-ReportConfig, include releaseConfigurationPreference;

6> if the CSI-ReportConfig is included in the RRCReconfiguration message, and the reportConfigType is periodic, and the applicabilityStatus is set to applicable:

7> submit the CSI-ReportConfig to lower layer;

4>	for each entry within applicabilitySetConfigCSI-ToAddModList associated with the concerned serving cell, that is included in the RRCReconfiguration message or for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either in RRCReconfigurationComplete or in UEAssistanceInformation):

5>	include an entry in the applicabilityInfoReportList and set the content as follows:

6>	set the applicabilitySetId within applicabilityInfoReportId to the corresponding applicabilitySetConfigId;

6>	set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityInfoReportId;

6>	if the applicabilityStatus is set to inapplicable:

7>	if the UE prefers to release the concerned configuration in ApplicabilitySetConfigCSI, include releaseConfigurationPreference;

<Text Omitted>

NEXT CHANGE