3GPP TSG-RAN WG2 Meeting #132					       R2-2509010

Dallas, USA, November 17th – 21st , 2025

Agenda item:	8.1.2

Source:	Samsung

Title:	Remaining RRC issues in LCM for BM use case

Document for:	Discussion and Decision

- Introduction

In this contribution, we would like to discuss remaining RRC issues.

- S050: UAI retransmission after HO/CHO/LTM cell switch

If the UE has sent a UAI and within 1 second a handover has occurred or if the UE has sent a UAI and a CHO or LTM cell switch has occurred, UE is required to resend the UAI. This is to avoid the transmission of UAI from source to target cell or to prepare CHO/LTM candidate cells, which can interrupt ongoing mobility. For applicability reporting, RAN2 has agreed that UE will send the new applicability in RRCReconfigurationComplete after handover. Thus, there is no need to retransmit the UAI.

This can be captured as below in the specification:

2> if reconfigurationWithSync was included in masterCellGroup or secondaryCellGroup: 3> if the UE initiated transmission of a UEAssistanceInformation message for the corresponding cell group during the last 1 second, and the UE is still configured to provide the concerned UE assistance information for the corresponding cell group; or 3> if the RRCReconfiguration message is applied due to a conditional reconfiguration execution or an LTM cell switch procedure, and the UE is configured to provide UE assistance information for the corresponding cell group, and the UE has initiated transmission of a UEAssistanceInformation message for the corresponding cell group since it was configured to do so in accordance with 5.7.4.2: 4> if the UE assistance information was not initiated to report assistance information about the applicability of configurations subject to the applicability determination procedure: 5> initiate transmission of a UEAssistanceInformation message for the corresponding cell group in accordance with clause 5.7.4.3 to provide the concerned UE assistance information; 5> start or restart the prohibit timer (if exists) associated with the concerned UE assistance information with the timer value set to the value in corresponding configuration; 4> start or restart the leave without response timer (if exists) or the wait timer (if exists) with the timer value set to the value in musim-CapabilityRestrictionConfig;

Proposal 1: [S050] UE does not resend the UAI for applicability reporting in below cases:

  - If the UAI was send 1s before the handover execution.

  - After LTM cell switch or CHO execution.

- E065: Required spec change to activate applicable periodic CSI report

RAN1 sent a reply LS on handling of activating applicable periodic CSI report.

- Option 1: Upon reception of RRC Reconfiguration message, UE’s RRC layer immediately submits inference configuration of periodic CSI to lower layer, regardless of whether the configuration is applicable/inapplicable. - Option 2: Upon reception of RRC Reconfiguration message, UE’s RRC layer submits inference configuration of periodic CSI to lower layer only if it is reported as applicable in RRCReconfigurationComplete.. From RAN1 perspective, Option 2 can resolve the issue. No additional RAN 1 impact was identified for Option 2.

Since RAN2 based option2 is more preferred by RAN1, it is natural for RAN2 to implement RRC based handling.

Regarding the spec change, it is noted that we don’t consider the transmission of RRCReconfigurationComplete message. As descried in Option 2, the main condition is “if it is reported as applicable in RRCReconfigurationComplete.”. It would mean that RRC applies configuration while it proceeds RRC reconfiguration message and generates the content of RRCRconfigurationComplete instead of applying configuration upon transmission of  RRCReconfigurationComplete message.

Proposal 2: [E065] RAN2 agree to apply RRC based handling to activate applicable periodic CSI report i.e RRC layer provide periodic CSI report configuration only if it is applicable and the required operation is applied when RRC layer proceeds RRC reconfiguration message.

Proposal 3: [E065] if proposal 2 is agreed, the proposed spec change shown in this document should be captured in the spec.

2> if the UE is configured with at least one entry in applicabilitySetConfigCSI-ToAddModList for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either in RRCReconfigurationComplete or in UEAssistanceInformation): 3> for each serving cell associated with any of the configurations above, include an entry in the applicabilityReportList and set the content as follows: 4> set the applicabilityCellId to the serving cell index of the cell; 4> for each configured reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction, or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19, that is included in the RRCReconfiguration message or for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (in RRCReconfigurationComplete or in UEAssistanceInformation or in RRCResumeComplete): 5> include an entry in the applicabilityInfoReportList and set the content as follows: 6> set the csi-ReportConfigId within applicabilityInfoReportId to the corresponding reportConfigId; 6> set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityInfoReportId; 6> if the applicabilityStatus is set to applicable and reportConfigType in CSI-ReportConfig is periodic: 7> configure lower layers to perform CSI report; 6> if the applicabilityStatus is set to inapplicable: 7> if the UE prefers to release the concerned CSI-ReportConfig, include releaseConfigurationPreference; 4> for each entry within applicabilitySetConfigCSI-ToAddModList associated with the concerned serving cell, that is included in the RRCReconfiguration message or for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either in RRCReconfigurationComplete or in UEAssistanceInformation): 5> include an entry in the applicabilityInfoReportList and set the content as follows: 6> set the applicabilitySetId within applicabilityInfoReportId to the corresponding applicabilitySetConfigId; 6> set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityInfoReportId; 6> if the applicabilityStatus is set to inapplicable: 7> if the UE prefers to release the concerned configuration in ApplicabilitySetConfigCSI, include releaseConfigurationPreference;

- When inapplicable periodic CSI report becomes applicable

Based on unofficial offline discussion, the following options are identified [x].

- Option 1: UE can report applicability change via RRCReconfigurationComplete, but UE can’t autonomously activate periodic CSI report configuration received in previous RRCReconfiguration (i.e., UE does not submit the configuration to lower layers).

- Option 2: RRCReconfigurationComplete can include the applicability status for any previously configured inference configurations, except previously configured periodic CSI config, for which applicability/inapplicability has already been reported and whose applicability status has changed since the last report  (i.e., changes of applicability for periodic CSI inference configuration are only transmitted via UAI).

- Option 3: When a UE receives a periodic CSI configuration, and it is inapplicable, the periodic configuration is not reported to the lower layers and the UE autonomously releases it. NW is notified of release via 'inapplicable' status in applicability report.

- Option 4: RRCReconfigurationComplete does not include the applicability status for any previously configured inference configurations, i.e., changes of applicability for all CSI inference configuration(s) are only transmitted via UAI

All options seem feasible. However, option 4 results in signaling overhead more than any other options because UE always send the applicability status in UAI if it is sent via RRCReconfigurationComplete message and also affect other CSI-ReportConfig cases (i.e. change from inapplicable to applicable and aperiodic/semi-persistent CSI-ReportConfig). Therefore, we would like to avoid option 4.

Observation 1: Option 4 causes signaling overhead unnecessarily by relying on UAI even if it is sent via RRCReconfigurationComplete message.

Regarding option 2, this has the same problem as option 4 although the final signaling overhead is less problematic given that only periodic CSI report is not included in RRCReconfiguraitonComplete message. However, given that we still have an option not to increase signaling overhead, there is not much motivation to support this option.

Observation 2: Option 2 causes signaling overhead unnecessarily by relying on UAI only for periodic CSI report config

Option 3 could be the cleanest solution from the spec point of view. However, it may give negative impact to overall operation in Option A in which gNB provides candidate CSI-ReportConfigs before gNB knows applicability status. It is noted that UE may not indicate applicability in RRCReconfigurationComplete message although model is available due to model downloading time or processing time to enable model. Instead of optimizing this case, RAN2 agreed to assume that we rely on the applicability status reporting via UAI. Option 3 actually preclude this benefit.

Observation 3: Option 3 cannot support the case where UE cannot report applicability in RRCReconfiguraitonComplete message due to model downloading or processing delay despite available model.

Proposal 4: RAN2 agree option 1 i.e.  UE can report applicability change via RRCReconfigurationComplete, but UE can’t autonomously activate periodic CSI report configuration received in previous RRCReconfiguration (i.e., UE does not submit the configuration to lower layers).

Proposal 5: if proposal 3 is agreed, the following change based on option 1 is captured in the spec.

The expected change for option 1 :

2> if the UE is configured with at least one entry in applicabilitySetConfigCSI-ToAddModList for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either in RRCReconfigurationComplete or in UEAssistanceInformation): 3> for each serving cell associated with any of the configurations above, include an entry in the applicabilityReportList and set the content as follows: 4> set the applicabilityCellId to the serving cell index of the cell; 4> for each configured reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction, or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19, that is included in the RRCReconfiguration message or for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (in RRCReconfigurationComplete or in UEAssistanceInformation or in RRCResumeComplete): 5> include an entry in the applicabilityInfoReportList and set the content as follows: 6> set the csi-ReportConfigId within applicabilityInfoReportId to the corresponding reportConfigId; 6> set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityInfoReportId; 6> if the applicabilityStatus is set to applicable and reportConfigType in CSI-ReportConfig that is included in RRCReconfiguration message is periodic: 7> configure lower layers to perform CSI report; 6> if the applicabilityStatus is set to inapplicable: 7> if the UE prefers to release the concerned CSI-ReportConfig, include releaseConfigurationPreference; 4> for each entry within applicabilitySetConfigCSI-ToAddModList associated with the concerned serving cell, that is included in the RRCReconfiguration message or for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either in RRCReconfigurationComplete or in UEAssistanceInformation): 5> include an entry in the applicabilityInfoReportList and set the content as follows: 6> set the applicabilitySetId within applicabilityInfoReportId to the corresponding applicabilitySetConfigId; 6> set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityInfoReportId; 6> if the applicabilityStatus is set to inapplicable: 7> if the UE prefers to release the concerned configuration in ApplicabilitySetConfigCSI, include releaseConfigurationPreference;

- Conclusion

In this contribution, we discussed the remaining RRC issues. Based on the discussion, we propose the following points.

Proposal 1: [S050] UE does not resend the UAI for applicability reporting in below cases:

  - If the UAI was send 1s before the handover execution.

  - After LTM cell switch or CHO execution.

Proposal 2: [E065] RAN2 agree to apply RRC based handling to activate applicable periodic CSI report i.e RRC layer provide periodic CSI report configuration only if it is applicable and the required operation is applied when RRC layer proceeds RRC reconfiguration message.

Proposal 3: [E065] if proposal 2 is agreed, the proposed spec change shown in this document should be captured in the spec.

Observation 1: Option 4 causes signaling overhead unnecessarily by relying on UAI even if it is sent via RRCReconfigurationComplete message.

Observation 2: Option 2 causes signaling overhead unnecessarily by relying on UAI only for periodic CSI report config

Observation 3: Option 3 cannot support the case where UE cannot report applicability in RRCReconfiguraitonComplete message due to model downloading or processing delay despite available model.

Proposal 4: RAN2 agree option 1 i.e.  UE can report applicability change via RRCReconfigurationComplete, but UE can’t autonomously activate periodic CSI report configuration received in previous RRCReconfiguration (i.e., UE does not submit the configuration to lower layers).

Proposal 5: if proposal 3 is agreed, the following change based on option 1 is captured in the spec.

- Reference

R1-2508122 – LS reply on RAN2 LS on periodic CSI inference configuration to lower layer (Samsung)

R1-250xxxx – Handling of inapplicable periodic CSI inference configuration, Interdigital et al.