3GPP TSG-RAN WG2 Meeting #132	                                                              R2-2509025

Dallas, USA, 17th – 21st Nov. 2025

Agenda item:	10.2.3

Source:	CEWiT

Title:	Forward compatible design considerations for 6GR User plane

Document for:	Discussion and Decision

# Introduction

In RAN#109 meeting, revised SID on 6G Radio (6GR) was approved [1]. The relevant SID topic related to this paper is

- User Plane architecture and protocol design

In this contribution, we discuss one of the forward compatible design considerations for 6G user plane related to multi-RAN connectivity supported by CORE.

# Discussion

In 6GR, future support for data flow involving multiple RAN co-ordinated by CORE may be required as discussed below.

In NR, to support high QoS services, lossless handover is supported during MBS multicast. This is achieved through the synchronisation of PDCP sequence numbers among source and target RAN. One method of achieving this synchronisation in NR is through derivation of the PDCP SNs from DL MBS QFI sequence numbers provided on NG-U [2]. Another approach is that NG-RAN nodes share a common user plane entity, denoted as shared NG-U termination, which allows the allocation of identical PDCP numbers to MBS users data when delivered to UEs in cells served by different NG-RAN nodes [3].

Another future use case relevant to 6GR, where such core level aggregation can be utilised is when the traffic distribution is non-uniform between adjacent cells and the surplus resources at a cell can be used for a UE in the adjacent cell. The target design in 6GR should lead to simpler network configuration and procedures compared to NR-DC. Instead of using the Xn interface to transfer RLC SDUs between the two gNBs as in NR-DC that increases the latency, schemes for DL data flow where the data is either duplicated or split from the core network among two gNBs (viz. gNB1 and gNB2) may be considered.

L2 and L3 protocol layer design should be forward compatible and not restrict any future possibility of supporting such use cases involving data flow through multiple RAN nodes supported by CORE.

Proposal 1: L2 and L3 protocol design should not restrict future possibility of supporting CORE level co-ordination/aggregation of data flows via multiple RAN nodes (e.g. to support MBS, load balancing, etc.)

# Conclusion

In this contribution, we discuss one of the forward compatible design considerations for 6G user plane related to multi-RAN connectivity supported by CORE. Following is the proposal:

Proposal 1. L2 and L3 protocol design should not restrict future possibility of supporting CORE level co-ordination/aggregation of data flows via multiple RAN nodes (e.g. to support MBS, load balancing, etc.)

# References

- RP-252912		Revised 	SID: Study on 6G Radio

- 3GPP TR 21.917

- 3GPP TS 23.247