3GPP TSG-RAN WG2 Meeting #132	                                                              R2-2509028

Dallas, USA, 17th – 21st Nov. 2025

Agenda item:	10.3.1.3

Source:	CEWiT

Title:	Retransmission enhancements for 6GR

Document for:	Discussion and Decision

# Introduction

In RAN#109 meeting, revised SID on 6G Radio (6GR) was approved [1]. The relevant SID topic related to this paper is

- User Plane architecture and protocol design

In RAN2#131bis following up on the UP design the following agreement was made.

Agreements on UP

- RAN2 first discuss/agree on the required functionalities for the protocol stack, and only afterwards discuss which protocol layer supports each functionality.  Discussions should focus on standalone architecture, until told otherwise by plenary.  When discussing functionalities companies should focus on identifying the problems being address or new requirements.

- UP design should aim to be hardware-processing friendly while keeping memory requirements low and minimize data movements across protocol layer.

- 6G user plane should aim to reduce the radio and end-to-end latency for general services (including eMBB).

- Study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic).  Understand the issues and shortcoming with the current NR QoS.   Aim to look at a general framework.

- Support at least the following scheduling schemes: dynamic grant and configured grant.  Further study configured grant.

- Study need for scheduling enhancements to address the SR/BSR/DSR latency.

- Study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ.  Study should identify enhancements needed for HARQ and L2 ARQ.

In this contribution, we discuss the possible directions for enhancing HARQ retransmission for improving latency.

# Discussion

ARQ based retransmission mechanisms embedded in the RLC layer is responsible to ensure reliability requirements of the packet, based on the demands of the service. HARQ retransmissions on the other hand provides reliability to the packet agnostic to the QoS flow to which the packet belongs to. For some applications demanding higher reliability along with small latency, ARQ based retransmission may not be able to satisfy latency requirement and small number of HARQ retransmissions may not be sufficient to satisfy required reliability requirement.

In the current HARQ scheme, even though HARQ failure to decode a MAC TB is known to the transmitter L2, ARQ will start retransmission based on the NACK reception at the transmitter RLC.

Observation 1. Existing ARQ retransmission mechanism that ensures reliability requirement for a service introduces delay in retransmission. For every ARQ retransmission, sequence number related header processing and its overhead is attached to it.

Observation 2. Existing HARQ retransmission mechanism does not differentiate between packets to achieve the QoS Flow’s demand of reliability.

Proposal 1: Study tighter ARQ and HARQ coordination for improved latency and efficiency by incorporating better service awareness in HARQ retransmission design compared to 5G NR

If the existing CBG based HARQ retransmission scheme can be enhanced with more service awareness, then it will be more flexible in general and more useful for such applications demanding higher reliability along with small latency.

A MAC TB may contain different packets with different service level requirements. The MAC entity cannot receive a successfully decoded CBGs of a MAC TB, i.e. only the entire MAC TB is forwarded from receiver PHY to receiver MAC. Assume the scenario where receiver MAC can decode the portions of the MAC TB. For example, on successful reception of a CBG, the MAC can decode the CBG. In such UL scenarios, when gNB is scheduling CBG based HARQ retransmissions, gNB is unaware about the retransmission requirement of the packets included inside the CBG. But if UE can request the gNB explicitly to continue the HARQ retransmissions for the CBGs those include packets demanding higher reliability, then service aware retransmissions can be achieved. One alternative method to achieve variable number of HARQ retransmissions for packets from different radio bearers is by dedicating HARQ Process IDs to a radio bearer (or to a set of radio bearers). In this way, UE can indicate gNB implicitly the required number of retransmissions if gNB can enforce different retransmission configurations to different HARQ Process IDs (or to the set of HARQ Process IDs).

Proposal 2. Study enhancements for UL CBG based HARQ retransmission by enabling gNB to determine CBG wise retransmission requirements with improved service awareness

# Conclusion

In this contribution, we discuss the possible directions for enhancing HARQ retransmission for improving latency. Following are the proposals:

Proposal 1: Study tighter ARQ and HARQ coordination for improved latency and efficiency by incorporating better service awareness in HARQ retransmission design compared to 5G NR

Proposal 2. Study enhancements for UL CBG based HARQ retransmission by enabling gNB to determine CBG wise retransmission requirements of UL data for improved service awareness

# References

- RP-252912		Revised 	SID: Study on 6G Radio