3GPP TSG-WG2 Meeting #132	R2-2509032

Dallas, USA, November 17 – 21, 2025

Agenda Item:	10.2.1

Source:	Deutsche Telekom

Title:	Discussion on UE capability framework

Document for:	Discussion, Decision

# 1	Introduction

In RAN2#131-bis, the following was agreed:

Agreements on UE capabilities for next meeting

For next meeting companies should focus on identifying the critical problems with UE capability framework and how to address them.  Identify what actions to be triggered with other WGs.

Companies can identify use cases to justify dynamic capability change in connected mode and analyse impacts to network side and network behaviour upon reception of updated capabilities.   Understand if there is impact to other WGs.

Companies can focus on the practical IODT problems in the field.   Possible ways to address them.

This contribution discusses the problems with existing UE capability framework and ways to address them.

# 2	Discussion

- The current 5G NR UE capability framework has become quite complex with the support of various UE capabilities defined per feature set and for different band combinations. This is reflected in the standards that have become really complicated leading to implementation complexity and commercialization delay. In terms of 6G study item, RAN2 should work on simplifying the current UE capability framework with the limitation of features/options. Specifically, RAN2, in collaboration with RAN1 and other relevant WGs, should identify as early as possible redundant/duplicated options and terminate them, merge possible subvariants and join dependent features where it is meaningful.

- Proposal 1:   RAN2 should work on minimizing the number of options/features by avoiding duplications, merging subvariants, joining dependent features, etc.

- In addition, in terms of UE capability signaling, RACS (Radio Capabilities Signaling Optimizations) feature, which was first specified in Release 16 aiming to optimize the signaling of a device's radio capabilities, should be further evaluated under 6G Study Item, as it could address some drawbacks of existing UE capability signaling. Via RACS, instead of sending the full capability information that has a high data volume due to supported bands and features, a UE receives via core network functionalities (AMF: Access and Mobility Management Function; UCMF: UE Radio Capability Management Function) a UE Radio Capability ID (Type 0/1), which is a shorter identifier that maps to its capabilities.

- In terms of 6G Study, RAN2 should evaluate RACS solution and identify possible enhancements focused on improving possible deficiencies of the existing solution in case of handover, where the retrieval of Radio capabilities from UCMF could add delay. For instance, one approach could be to evaluate the shift of the “UCMF-like” functionalities to the Base Station (Figure 1).

- Figure 1: Example of “UCMF-like” functionalities’ shift to the Base Station

- Proposal 2:     RAN2 should evaluate RACS solution and possible enhancements required for mobility case (i.e., shift of the “UCMF-like” functionalities to the Base Station).

# 3	Conclusion

Based on the discussion in the previous section, we propose the following:

- Proposal 1:   RAN2 should work on minimizing the number of options/features by avoiding duplications, merging subvariants, joining dependent features, etc.

- Proposal 2:     RAN2 should evaluate RACS solution and possible enhancements required for mobility case (i.e., shift of the “UCMF-like” functionalities to the Base Station).

# 4	References

[1]   R2-25xxxx, RAN2 #131bis meeting minutes.