3GPP TSG-RAN WG2 Meeting # 132	R2-2509037

# Dallas, USA, 17th – 21st November 2025

Source:	Google

Title:	Discussion on UE data collection and transfer

Agenda Item:	10.3.3.1

Document for:	Discussion and Decision

- Introduction

In the previous RAN2 meeting, RAN2 established several agreements for data collection and transfer as shown below. This contribution analyzes the technical considerations stemming from these agreements and provides focused proposals.

Agreements 1. Study the standardized data collection framework with these requirements as a baseline at least for AI/ML - The data collected is secured and data integrity and confidentiality for that data is ensured. - User data privacy, anonymity and user consent is respected. - The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE side data collection, without the need of Service Level Agreement (SLA) for this purpose - This includes initiating, terminating, and fully managing data transfer. - MNO has full visibility for standardized data. - The design is future-proof and extendable. - The UE data collection should minimize impact to the UE battery, UE processing and memory utilization. - UE data collection should minimize impact to user traffic transmission and power saving features These requirements can apply to both UE and NW sided data collection. FFS if some don’t apply to both. 2. Study further requirements of other non-AI/ML use cases/services and understand if we can have more common requirements across different use cases. 3. Study termination points (i.e. understand who are producers, consumers of the data, data collection points. Understand requirements of use cases sensing, aI/ml, son/mdt, QoE etc) and protocol used for data transfer (e.g. IP vs. non-IP). 4. Study model transfer/delivery requirements/functions

- Discussion

## 2.1 Transport of collected data

In current 5G NR systems, the RAN manages the configuration for these diverse data collection tasks, including SON, Logged MDT, various failure reports, QoE, and data collection for AI/ML model training. All configuration commands and the subsequent retrieval of the logged data rely on the RRC protocol, which operates in the control plane between the User Equipment (UE) and the RAN. Using RRC has two main issues described below.

- RRC is not suitable for substantial volume transfer: The primary issue is that the volume of data collected for purposes like AI/ML training is often substantial. The RRC protocol is fundamentally not suitable for handling such large-scale data transfers, leading to inefficiency.

- Increase cellular network load: Transmitting this large volume of data over the cellular network significantly increases the data load on the network infrastructure.

To address these inefficiencies, a new transport mechanism is required that satisfies two critical requirements:

- The protocol must be capable of efficiently handling the substantial volume of collected data.

- As per RAN2 agreements, any UE data collection mechanism must minimize impact on the UE battery.

Transmitting data over Wi-Fi consumes significantly less battery power than cellular transmission and offloads data load to WiFi. Leveraging this fact, the transport protocol should enable the UE to transmit the collected data via a Wi-Fi link in addition to a cellular connection.

Proposal 1: 	RRC is not suitable for transport of collected data. A transport protocol that efficiently transfers substantial volume of collected data is needed. The transport protocol should support transfer of the collected data via a cellular network and a WiFi network.

The termination points for this new transport protocol should be the UE and a dedicated new network entity (e.g., a Data Collection Function).

To facilitate transmission over diverse access technologies, this network entity must be placed in the core network, and the transport protocol itself must operate on top of IP. This IP-based architecture ensures the UE can efficiently transmit the substantial volume of collected data via a Wi-Fi link, achieving the necessary network offload and battery savings.

Proposal 2: 	Termination points are the UE and a new core network entity. This transport protocol operates on top of IP.

## 2.2 Data collection configuration

The configuration of data collection tasks (such as SON, MDT, and AI/ML data) is inherently linked to Radio Access Network (RAN) features that are themselves configured via the Radio Resource Control (RRC) protocol. This connection makes RRC a seemingly logical choice for providing the corresponding data collection configurations.

However, relying on RRC for configuring these data collection tasks introduces significant burdens as below:

- Mobility and Failure Management Overhead: The current RRC-based approach creates extra complexity for the UE and the RAN in managing configurations during mobility (e.g., handover) and failure recovery scenarios.

- Inter-Base Station Coordination: For instance, during handover preparation or UE context retrieval, the source base station must explicitly transfer the specific RRC data collection configurations to the target base station. This is a source of unnecessary signaling and overhead.

- RRC Specification Burden: Over time, incorporating diverse data collection configuration handling into the RRC specifications adds considerable complexity to the standard itself.

To alleviate these burdens, an alternative approach is to define a new, dedicated configuration protocol for configuring data collection.

- Protocol Location: This new configuration protocol would operate above the air interface protocol stack between the UE and the RAN, similar to protocols like LPP.

- New Network Entity: Configuration would be managed between the UE and a new network entity. This entity could reside either within the RAN or the core network.

- Key Benefit: By isolating the data collection configuration from the RRC, this approach successfully avoids the burdens associated with RRC configuration during mobility and failure recovery.

Proposal 3: 	Discuss the two options and agree one of them:

Option 1: RRC is used to configure data collections.

Option 2: A new configuration protocol is defined to configure data collections.

If RAN2 agrees to proceed with defining a new configuration protocol for data collection, the following two critical steps must be addressed:

- Determining Network Entity Location

RAN2 should discuss and reach an agreement on the optimal placement of the new network entity that will terminate this protocol. The options to be decided upon are location within the RAN or the CN.

- Coordination with Other WGs

Once the placement decision is finalized, RAN2 should formally communicate this crucial architectural agreement to the relevant working groups including at least RAN3 and SA2

Proposal 4: 	If RAN2 agrees to define a new configuration protocol to configure data collections, RAN2 should discuss and agree the new network entity is in the RAN or core network. RAN2 should send the agreement to RAN3 and SA2.

- Conclusion

Proposal 1: 	RRC is not suitable for transport of collected data. A transport protocol that efficiently transfer substantial volume of collected data is needed. The transport protocol should support transfer of the collected data via cellular and WiFi.

Proposal 2: 	Termination points are the UE and a new core network entity. This transport protocol operates on top of IP.

Proposal 3: 	Discuss the two options and agree one of them:

Option 1: RRC is used to configure data collections.

Option 2: A new protocol is defined to configure data collections.

Proposal 4: 	If RAN2 agrees to define a new configuration protocol to configure data collections, RAN2 should discuss and agree the new network entity is in the RAN or core network. RAN2 should send the agreement to RAN3 and SA2.

- References

[1] RAN2#131bis Chair note