3GPP TSG RAN WG2 #132	R2-2509055

Dallas, USA, Nov 17th-21st, 2025

Agenda Item:	8.8.4

Source: 	Xiaomi, Samsung, CATT, OPPO, Apple

Title:  	Discussion on the SMTC issues for the RRC redirection (V230)

Document for:	Discussion and Decision

Introduction

Int the last meeting, the SMTC issues for the RRC redirection from LTE-NR NTN were discussed and it agreed that the SMTC adjustment can be further discussed in this meeting. In this contribution, we further discussed the SMTC issue and provide our suggestions.

Discussion

According to TS36.331 [1], the nr-r19 is introduced in the RedirectedCarrierInfo and smtc-r19 is carried in the CarrierInfoNR-r19.

RedirectedCarrierInfo ::=			CHOICE {

eutra								ARFCN-ValueEUTRA,

geran								CarrierFreqsGERAN,

utra-FDD							ARFCN-ValueUTRA,

utra-TDD							ARFCN-ValueUTRA,

cdma2000-HRPD						CarrierFreqCDMA2000,

cdma2000-1xRTT						CarrierFreqCDMA2000,

...,

utra-TDD-r10						CarrierFreqListUTRA-TDD-r10,

nr-r15								CarrierInfoNR-r15,

nr-r17								CarrierInfoNR-r17,

nr-r19								CarrierInfoNR-r19

}

CarrierInfoNR-r19	::= SEQUENCE {

carrierFreq-r19					ARFCN-ValueNR-r15,

subcarrierSpacingSSB-r19			ENUMERATED {kHz15, kHz30, kHz120, kHz240},

smtc-r19							MTC-SSB-NR-r15				OPTIONAL,		-- Need OP

satAssistanceInfoList-r19			SEQUENCE (SIZE(1..maxSat-r17)) OF SatelliteId-r18

}

smtc The SSB periodicity/offset/duration configuration of the redirected target NR frequency. It is based on the timing reference of EUTRAN PCell. If the field is absent, the UE uses the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing.

When a UE is redirected from LTE to NR NTN, the network configures an SMTC to enable the UE to measure the NR NTN cells. However, LTE does not support PDD reporting, so the network cannot accurately determine the UE’s location or its propagation delay. This makes it difficult to configure a suitable SMTC. This challenge becomes more significant when the LTE cell has a large coverage radius (up to 100km). In such cases, the difference between the UE-to-TN propagation delay and the UE-to-satellite propagation delay can vary greatly depending on the UE’s position within the LTE cell. Therefore, considering that there is no PDD reporting in LTE cells, the UE may not be able to measure the NR NTN cells without SMTC offset adjustment, especially when the LTE cell has a large coverage radius.

Observation 1: Considering that there is no PDD reporting in LTE cells and the network can’t configure a suitable SMTC, the UE may not be able to measure the NR NTN cells without SMTC offset adjustment, especially when the LTE cell has a large coverage radius

Another point is that the network has the ability to configure multiple satellites for the RRC redirection. In the last meeting, E801 proposed to limit the number of satellite IDs to 1 in satAssistanceInfoList-r19. However, this proposal was not agreed upon. If multiple satellites are deployed on the frequency configured for RRC redirection, the distances between the UE and each satellite would be different. This is especially true for the case when these satellites are in the different orbit altitude. As a result, the propagation delays are different, and a single SMTC configuration cannot cover all satellites on that frequency. If the network configures multiple satellites in satAssistanceInfoList-r19, the UE cannot measure all the configured satellites unless it adjusts the SMTC offset. Without such adjustment, the UE may only be able to measure one satellite, as the fixed SMTC timing may not align with the propagation delays of the other satellites.

Observation 2:  When multiple satellites are configured by the network for RRC redirection, the UE cannot measure all of them using a single SMTC configuration unless it adjusts the SMTC offset according to the actual propagation delay between the UE and each configured satellite.

Based on the above discussion, we think that the UE should adjust the SMTC offset according to the actual propagation delay and the following TP is for the reference.

Smtc

The SSB periodicity/offset/duration configuration of the redirected target NR frequency. It is based on the timing reference of EUTRAN PCell. If the field is absent, the UE uses the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If smtc-r19 is configured, the offset (derived from parameter periodicityAndOffset) is based on the assumption that the UE's propagation delay difference between serving cell and neighbour cells equals to 0 ms, and UE can adjust the offset based on the actual propagation delay.

Proposal 1: We suggest accepting V230 as follows:

Smtc

The SSB periodicity/offset/duration configuration of the redirected target NR frequency. It is based on the timing reference of EUTRAN PCell. If the field is absent, the UE uses the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If smtc-r19 is configured, the offset (derived from parameter periodicityAndOffset) is based on the assumption that the UE's propagation delay difference between serving cell and neighbour cells equals to 0 ms, and UE can adjust the offset based on the actual propagation delay.

Conclusions

In this contribution, we discussed the smtc issues for the RRC redirection and provide the observations and proposal as follows：

Observation 1: Considering that there is no PDD reporting in LTE cells and the network can’t configure a suitable SMTC, the UE may not be able to measure the NR NTN cells without SMTC offset adjustment, especially when the LTE cell has a large coverage radius.

Observation 2: When multiple satellites are configured by the network for RRC redirection, the UE cannot measure all of them using a single SMTC configuration unless it adjusts the SMTC offset according to the actual propagation delay between the UE and each configured satellite.

Proposal 1: We suggest accepting V230 as follows:

Smtc

The SSB periodicity/offset/duration configuration of the redirected target NR frequency. It is based on the timing reference of EUTRAN PCell. If the field is absent, the UE uses the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If smtc-r19 is configured, the offset (derived from parameter periodicityAndOffset) is based on the assumption that the UE's propagation delay difference between serving cell and neighbour cells equals to 0 ms, and UE can adjust the offset based on the actual propagation delay

References

[1] 3GPP TS 36.331 V19.0.0, Radio Resource Control (RRC) protocol specification