3GPP TSG-RAN WG2 Meeting #132	R2-2509058

Dallas, USA, 17 – 21 November 2025

Agenda item:	8.7.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on remaining issues in RLC for R19 XR

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In this contribution, we address some open issues that remained after the previous meeting, as follows.

Agreements for RLC - … - (RLC-N01) Try to capture it in the procedure “If t-RxDiscard is configured and expires, the receiving side of an AM RLC entity consider the corresponding SDUs as received.”, if problem is identified or companies have strong concern during CR phase, it will be postponed. - (RLC-N01) Add “or t-RxDiscard expires” at the end of definition of RX_Next. - … - (RLC-H02) Confirm that when t-RxDiscard expires, RX_Next might be larger or equal than RX_Highest_Status. Companies are invited to bring contributions on the solutions to address it.

# 2	RLC

## 2.1	RLC-N01

In RAN2#131bis, the following was agreed for RLC-N01 concerning the behaviour of RX_Next variable on t-RxDiscard expiry.

- (RLC-N01) Try to capture it in the procedure “If t-RxDiscard is configured and expires, the receiving side of an AM RLC entity consider the corresponding SDUs as received.”, if problem is identified or companies have strong concern during CR phase, it will be postponed.

Accordingly, it was captured in RLC CR as follows:

##### 5.2.3.2.5 Actions when t-RxDiscard expires When t-RxDiscard expires, the receiving side of an AM RLC entity shall: - discard the AMD PDU(s) in the reception buffer with SN < RX_Next_Discard_Trigger, if any; - consider the RLC SDU(s) with SN < RX_Next_Discard_Trigger, as completely received; - update RX_Next to the SN of the first RLC SDU with SN >= RX_Next_Discard_Trigger for which not all bytes have been received; - if RX_Next_Highest > RX_Next +1; or - if RX_Next_Highest = RX_Next + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU: - start t-RxDiscard; - set RX_Next_Discard_Trigger to RX_Next_Highest.

The text added above seems to affect the existing normative text in the specification elsewhere, e.g., the clauses 5.3.4 and 6.2.3.10: For STATUS PDU, ACK_SN field indicates the SN of the next not received RLC SDU which is not reported as missing and is not discarded due to the expiry of t-RxDiscard. With the added text, all the discarded AMD PDUs will be considered as received, meaning that there would never be any discarded SDU which is “not received”. The ACK_SN to be set as per the procedural text in clause 5.3.4 and its description in clause 6.2.3.10 thus does not need to check whether the not received RLC SDU is discarded due to expiry of t-RxDiscard, i.e., they are always discarded. Therefore, if we want to specify update of RX_Next for t-RxDiscard in the procedural text in clause 5.2.3.2.5, it requires additional change in clauses 5.3.4 & 6.2.3.10:

### 5.3.4 Status reporting An AM RLC entity sends STATUS PDUs to its peer AM RLC entity in order to provide positive and/or negative acknowledgements of RLC SDUs (or portions of them). [text omitted] - set the ACK_SN to the SN of the next not received RLC SDU which is not indicated as missing in the resulting STATUS PDU.

#### 6.2.3.10 Acknowledgement SN (ACK_SN) field Length: 12 bits or 18 bits (configurable). The ACK_SN field indicates the SN of the next not received RLC SDU which is not reported as missing in the STATUS PDU. When the transmitting side of an AM RLC entity receives a STATUS PDU, it interprets that all RLC SDUs up to but not including the RLC SDU with SN = ACK_SN have been received by its peer AM RLC entity, excluding those RLC SDUs indicated in the STATUS PDU with NACK_SN, portions of RLC SDUs indicated in the STATUS PDU with NACK_SN, SOstart and SOend, RLC SDUs indicated in the STATUS PDU with NACK_SN and NACK_range, and portions of RLC SDUs indicated in the STATUS PDU with NACK_SN, NACK range, SOstart and SOend.

Observation 1: [RLC-N01] Specifying update of RX_Next for t-RxDiscard in the procedural text of clause 5.2.3.2.5 requires additional change in the clause 6.2.3.10 and 5.3.4.

From specification management perspective, it would not be a desirable way to specify the behaviour indirectly – possibly with more changes – especially when we can fix it in the right place. Therefore, we suggest to re-consider how to capture the update of RX_Next based on the text proposal in R2-2507159.

Proposal 1: [RLC-N01] The definition of RX_Next in clause 7.1 is updated as below:

a) RX_Next – Receive state variable This state variable holds the value of the SN of the first RLC SDU not delivered to upper layers, but still waited for and it serves as the lower edge of the receiving window. It is initially set to 0, and is updated whenever the AM RLC entity receives an RLC SDU with SN = RX_Next or t-RxDiscard expires.

## 2.2	RLC-H02

In RAN2#131bis, RAN2 discussed a scenario in R2-2507903 and confirmed that expiry of t-RxDiscard can cause RX_Next to advance past RX_Highest_Status, resulting in RX_Next being equal to or larger than RX_Highest_Status.

(RLC-H02) Confirm that when t-RxDiscard expires, RX_Next might be larger or equal than RX_Highest_Status. Companies are invited to bring contributions on the solutions to address it.

The current RLC specification defines the procedure for including NACK related information, e.g., NACK_SN and NACK range, in a STATUS PDU, applying it only to SNs that fall within the range RX_Next <= SN < RX_Highest_Status. This procedural text does not preclude a case where this range is empty. In fact (since Rel-15!), RX_Next can be equal to RX_Highest_Status when e.g., all data has been received in-sequence, and no gaps exist. In this specific case, the condition RX_Next <= SN < RX_Highest_Status is not met (i.e., the applicable range is empty), and consequently, no NACK_SN fields are included in the STATUS PDU. However, the ACK_SN is included by pointing to the SN of the next not received RLC SDU which is not indicated as missing.

Observation 2: [RLC-H02] In legacy, i.e., without t-RxDiscard, RX_Next can be equal to RX_Highest_Status, and in this case the STATUS PDU does not include NACK related SN information but includes ACK_SN which can be properly set according to the current specification.

When constructing a STATUS PDU, the AM RLC entity shall: - for the RLC SDUs with SN such that RX_Next <= SN < RX_Highest_Status that has not been completely received yet, in increasing SN order of RLC SDUs and increasing byte segment order within RLC SDUs, starting with SN = RX_Next up to the point where the resulting STATUS PDU still fits to the total size of RLC PDU(s) indicated by lower layer: - for an RLC SDU for which no byte segments have been received yet: - include in the STATUS PDU a NACK_SN which is set to the SN of the RLC SDU. - for a continuous sequence of byte segments of a partly received RLC SDU that have not been received yet: - include in the STATUS PDU a set of NACK_SN, SOstart and SOend. - for a continuous sequence of RLC SDUs that have not been received yet: - include in the STATUS PDU a set of NACK_SN and NACK range; - include in the STATUS PDU, if required, a pair of SOstart and SOend. - set the ACK_SN to the SN of the next not received RLC SDU which is not indicated as missing and is not discarded due to the expiry of t-RxDiscard in the resulting STATUS PDU.

This same logic in the above observation can extend to the new Rel-19 scenario where t-RxDiscard expiry results in RX_Next > RX_Highest_Status. In this case, the condition for including NACK information is also not met, as the applicable range is empty, hence, there is no RLC SDUs eligible for NACK reporting. The only remaining thing is to ensure the ACK_SN is correctly set to reflect the RX_Next, which is updated due to the discard.

In R2-2507903, one alternative (referred to as Alt-1) was proposed: separating the procedural text based on the trigger of the STATUS PDU (i.e., t-RxDiscard expiry or not) and explicitly specifying that ACK_SN is set to RX_Next.

In our view, separating the procedure would not be necessary as long as the existing text for ACK_SN setting is clear even for the case when RX_Next is higher than RX_Highest_Status. Currently, ‘next’ is used, which in general implies the one immediately following the previous one. Given that the intention is to search the lowest numbered not received RLC SDU satisfying the following conditions - not indicated as missing and not discarded due to t-RxDiscard expiry -, it would be clearer to use ‘first’ instead of ‘next’.

Proposal 2: [RLC-H02] In order to specify ACK_SN setting clearly for the case where RX_Next > RX_Highest_Status, change ‘next’ to ‘first’ in the procedure of ACK_SN setting in STATUS PDU construction (clause 5.3.4), without mentioning the trigger of STATUS PDU:

### 5.3.4 Status reporting An AM RLC entity sends STATUS PDUs to its peer AM RLC entity in order to provide positive and/or negative acknowledgements of RLC SDUs (or portions of them). [text omitted] - set the ACK_SN to the SN of the first not received RLC SDU which is not indicated as missing and is not discarded due to the expiry of t-RxDiscard in the resulting STATUS PDU.

# 3	Conclusion

This document has made the following observations:

Observation 1: [RLC-N01] The potential new text to consider missing SDUs as completely received when t-RxDiscard expires is incompatible with existing text elsewhere in the specification, which speaks of “not received RLC SDU which […] is not discarded due to the expiry of t-RxDiscard”.

Observation 2: [RLC-H02] In legacy, i.e., without t-RxDiscard, RX_Next can be equal to RX_Highest_Status, and in this case the STATUS PDU does not include NACK related SN information but includes ACK_SN which can be properly set according to the current specification.

When constructing a STATUS PDU, the AM RLC entity shall: - for the RLC SDUs with SN such that RX_Next <= SN < RX_Highest_Status that has not been completely received yet, in increasing SN order of RLC SDUs and increasing byte segment order within RLC SDUs, starting with SN = RX_Next up to the point where the resulting STATUS PDU still fits to the total size of RLC PDU(s) indicated by lower layer: - for an RLC SDU for which no byte segments have been received yet: - include in the STATUS PDU a NACK_SN which is set to the SN of the RLC SDU. - for a continuous sequence of byte segments of a partly received RLC SDU that have not been received yet: - include in the STATUS PDU a set of NACK_SN, SOstart and SOend. - for a continuous sequence of RLC SDUs that have not been received yet: - include in the STATUS PDU a set of NACK_SN and NACK range; - include in the STATUS PDU, if required, a pair of SOstart and SOend. - set the ACK_SN to the SN of the next not received RLC SDU which is not indicated as missing and is not discarded due to the expiry of t-RxDiscard in the resulting STATUS PDU.

And proposed the following:

Proposal 1: [RLC-N01] The definition of RX_Next in clause 7.1 is updated as below:

a) RX_Next – Receive state variable This state variable holds the value of the SN of the first RLC SDU not delivered to upper layers, but still waited for and it serves as the lower edge of the receiving window. It is initially set to 0, and is updated whenever the AM RLC entity receives an RLC SDU with SN = RX_Next or t-RxDiscard expires.

Proposal 2: [RLC-H02] In order to specify ACK_SN setting clearly for the case where RX_Next > RX_Highest_Status, change ‘next’ to ‘first’ in the procedure of ACK_SN setting in STATUS PDU construction (clause 5.3.4), without mentioning the trigger of STATUS PDU:

### 5.3.4 Status reporting An AM RLC entity sends STATUS PDUs to its peer AM RLC entity in order to provide positive and/or negative acknowledgements of RLC SDUs (or portions of them). [text omitted] - set the ACK_SN to the SN of the first not received RLC SDU which is not indicated as missing and is not discarded due to the expiry of t-RxDiscard in the resulting STATUS PDU.