3GPP TSG-RAN WG2 Meeting #132	R2-2509062

Dallas, USA, 17 – 21 November 2025

Agenda item:	9.3.3

Source:	Nokia

Title:	Views on Measurement Event Prediction

WID/SID:	NR_AIML_Mob - Release 20

Document for:	Discussion and Decision

# 1	Introduction

In the SI, the following too approaches for measurement event prediction were captured in TR 38.744:

There are two methods to predict measurement event, namely indirect and direct measurement event prediction as illustrated in Figure 4.3-1 and Figure 4.3-2 respectively. Figure 4.3-1: Indirect measurement event prediction In indirect measurement event prediction for intra-frequency temporal domain case A, temporal domain case B or spatial domain, measurement result(s) is predicted by a RRM measurement prediction model at first. Afterwards, predicted and optionally actual historical measurement result(s) of the same cell(s) are used to derive whether a measurement event at one future time instance occurs, without further involvement of an AI/ML model. In indirect measurement event prediction for frequency domain, measurement result(s) is predicted by a RRM measurement prediction model for frequency domain at first. Afterwards, predicted and optionally actual historical measurement result(s) of serving cell are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model. Figure 4.3-2: Direct measurement event prediction As illustrated in Figure 4.3-2, the input of the model with direct prediction is the same as indirect prediction as illustrated in Figure 4.3-1 and additional input is also allowed for both. Measurement event is predicted directly by an AI/ML model, i.e. the output of the model is the likelihood of an event occurrence. For measurement event prediction based on intra-frequency temporal domain case B, the 3 filtering options captured in section 4.2 also apply for the input of RRM sub-use case 2. For indirect prediction, the skipped result refers to L3 RSRP measurement result predicted previously by the RRM measurement prediction model. For direct prediction, the skipped result refers to skipped L1 filtered measurement result and filtering option 1 is not applicable. NOTE 1: The measurement event refers to measurement events A1-A6 defined in clause 5.5.4 in 38.331.

This contribution outlines our preliminary insights into the measurement event prediction framework, its configuration, and the essential AI-driven enhancements required for the feature.

# 2	Discussion

## 2.1	Measurement Event Prediction Framework

Event-based reporting conditions may be applied on predicted measurements, or the event condition fulfilment may be predicted directly.

Figure 2.1-1 Measurement Event Prediction

Elaborating further in Figure 2.1-1 shown in our companion t-doc [1],  shows RRM measurement prediction applied to 5G measurement framework (TS 38.300). The cell-level L3 measurement C is given as input into the measurement prediction, which predicts measurements CPred in the PW or in the target frequency. Depending on the NW configuration, reporting criteria may be applied to the predictions to determine if they should be reported. When predicting measurement events like A3 and A5, predicted measurements for serving and candidate cells may have to be compared and the measurement prediction should be configured accordingly. It can be also noted that a predicted measurement event condition evaluation can be based on both measurements and predictions, for example when a prediction is made during TTT.

Observation 1: RRM measurement predictions may be used in measurement event prediction by applying the reporting criteria on the predicted measurements.

Observation 2: When predicting measurement events like A3 and A5, predicted measurements for serving and candidate cells may have to be compared.

Observation 3: Measurement event condition evaluation can be based on both measurements and predictions, when a prediction is made during TTT.

In RAN2#131 it was agreed that:

2. For RRM measurement event prediction, the event prediction report can include: -RRM measurements (For normative phase to define what types of measurements) -Time-related information about the predicted event (e.g., the time/window of occurrence)

In RAN2#129bis it was agreed that:

- For RRM prediction, UE sided model, the UE can be configured with periodic or event triggered reporting of predicted and/or actual RRM measurements. FFS details - For event prediction, UE-sided model, the UE can be configured with event-triggered reporting based on prediction and/or actual measurements. FFS details - Baseline is that this applies to all A1-6 events, unless technical problems are identified. Baseline quantity is RSRP.

The UE reporting based on a predicted measurement event should be configurable by the network. The configured reporting based on a predicted measurement event can include:

- RRM measurements

- Predicted RRM measurements

- Properties of the predicted event, for example, time/window, probability, ML monitoring metrics

If predicted measurements are reported based on a predicted measurement event, the future time of the reported measurement predictions can indicate the time of the predicted measurement event. The reporting of either measurements and/or predictions based on either normal or predicted measurement events is performed in accordance with NW configuration..

Observation 4: Based on a predicted measurement event, the UE can report:

- RRM measurements

- Predicted measurements

- Properties of the predicted event, for example, time/window, probability, ML monitoring metrics

Proposal 1: The reporting of either measurements and/or predictions based on either legacy or predicted measurement events is performed in accordance with NW configuration.

Proposal 2: If predicted measurements are reported based on a predicted measurement event, the time of the predicted event can be indicated by the time of the reported measurement predictions.

## 2.2 	Measurement Event Prediction Configuration

In RAN2#131 it was agreed that:

1 For RRM measurement event prediction, the following are included in the inference configuration: - The length of PW for the associated AIML model - Event-related information (event type and event-related parameters)

Figure 2.2-1 RRM measurement prediction configuration

Figure 2.2-1 depicts an example of possible configurations for measurement prediction and measurement event prediction. The measurements used as input to the prediction can be configured as legacy. Measurement prediction configuration is discussed in our companion TDoc [1].

Measurement event prediction can be configured in connection with the reporting configuration for measurements and/or measurement predictions, i.e., a predicted measurement event can be configured as a condition in event-based reporting configuration for measurements and/or predicted measurements. Additionally, reporting of properties of the predicted measurement event, such as for example time/window, probability and ML monitoring metrics, can be configured.

Measurement event prediction configuration should include at least the PW, the predicted measurement event and if measurements and/or measurement predictions should be reported. Measurement identity configuration can be used to associate measurement, measurement prediction and measurement event prediction with reporting configurations.

Observation 5: Legacy framework can be reused to configure measurements used as input to the measurement event prediction feature.

Proposal 3: A predicted measurement event can be configured as a condition in event-based reporting configuration for measurements and/or predicted measurements.

Proposal 4: Measurement event prediction configuration should include at least the PW, the predicted measurement event and if measurements and/or measurement predictions should be reported.

In RAN2#131, there was an agreement:

3 For measurement event determined, when the UE is configured with temporal domain case B, and frequency domain prediction, UE reports measurement event by following an approach similar legacy procedure. This can be achieved without spec impact for measurement reporting. FFS for normative phase if additional information needs to be added.

According to the agreement, event-based reporting should be configurable for temporal domain case B RRM measurement prediction and inter-frequency measurement prediction. For these scenarios, classical measurement reporting configuration can be a starting point and any extensions are FFS.

An example of frequency domain measurement event prediction using inter-frequency measurement prediction is a scenario, where there is a larger coverage cell in frequency F1 and a smaller collocated capacity cell in frequency F2. When the UE is handed over to the coverage cell, it may be configured to predict measurements for the capacity cell in F2 and report if they are predicted to exceed configured threshold, by configuring A1 measurement event-based reporting of the inter-frequency measurement predictions. Based on the reported prediction, the NW may decide to configure the UE to measure F2.

Proposal 5: RAN2 to discuss event-based reporting of inter-frequency measurement predictions.

## 2.3	AI-related enhancements

During the SI, there was a clear understanding that we should aim to re-use the applicability framework developed for AI BM as much as possible. Such approach is seen as beneficial as it avoids lengthy and unnecessary discussions in RAN2 whenever new use-cases are introduced. While we agree with such intention, it's also important to recognize that there might be specific differences that need careful consideration for the measurement event prediction use case. Considering that a possible implementation of such feature is to extend the measurement prediction framework, we believe that detailed discussions about AI-related enhancements such as applicability determination, associated ID etc., can be started once the necessary enhancements for the measurement prediction use case are more clearly defined and understood.

Observation 6: While the AI BM applicability framework can be re-used to a large extent, detailed discussions on enhancements for measurement event prediction can be postponed until further progress with RRM measurement prediction use-case.

Proposal 6: Postpone discussion on applicability aspects for measurement event prediction use-case, until more progress is achieved for the measurement prediction use-case.

# 3	Conclusion

This document has made the following observations:

Observation 1: RRM measurement predictions may be used in measurement event prediction by applying the reporting criteria on the predicted measurements.

Observation 2: When predicting measurement events like A3 and A5, predicted measurements for serving and candidate cells may have to be compared.

Observation 3: Measurement event condition evaluation can be based on both measurements and predictions, when a prediction is made during TTT.

Observation 4: Based on a predicted measurement event, the UE can report:

- RRM measurements

- Predicted measurements

- Properties of the predicted event, for example, time/window, probability, ML monitoring metrics

Observation 5: Legacy framework can be reused to configure measurements used as input to the measurement event prediction feature.

Observation 6: While the AI BM applicability framework can be re-used to a large extent, detailed discussions on enhancements for measurement event prediction can be postponed until further progress with RRM measurement prediction use-case.

And proposed the following:

Proposal 1: The reporting of either measurements and/or predictions based on either legacy or predicted measurement events is performed in accordance with NW configuration.

Proposal 2: If predicted measurements are reported based on a predicted measurement event, the time of the predicted event can be indicated by the time of the reported measurement predictions.

Proposal 3: A predicted measurement event can be configured as a condition in event-based reporting configuration for measurements and/or predicted measurements.

Proposal 4: Measurement event prediction configuration should include at least the PW, the predicted measurement event and if measurements and/or measurement predictions should be reported.

Proposal 5: RAN2 to discuss event-based reporting of inter-frequency measurement predictions.

Proposal 6: Postpone discussion on applicability aspects for measurement event prediction use-case, until more progress is achieved for the measurement prediction use-case.

# 4	References

[1] R2-2509060, On RRM Measurement Prediction, Nokia, Nokia Shanghai Bell, 3GPP TSG-RAN WG2 Meeting #132, Dallas, Texas, USA, 17 – 21 November 2025.