3GPP TSG-RAN WG2#132                                        	R2-2509070

Dallas, USA, November 17 – 21, 2025

Agenda item:	10.3.3.1

Source: 	Kyocera

Title: 	Analysis of Data Flow Requirements for AI/ML and ISAC

Document for:	Discussion

Introduction

At the previous RAN2#131bis meeting, RAN2 agreed on the following statements for Data Collection/Management/Transfer. [1]

Agreements 1. Study the standardized data collection framework with these requirements as a baseline at least for AI/ML - The data collected is secured and data integrity and confidentiality for that data is ensured. - User data privacy, anonymity and user consent is respected. - The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE side data collection, without the need of Service Level Agreement (SLA) for this purpose - This includes initiating, terminating, and fully managing data transfer. - MNO has full visibility for standardized data. - The design is future-proof and extendable. - The UE data collection should minimize impact to the UE battery, UE processing and memory utilization. - UE data collection should minimize impact to user traffic transmission and power saving features These requirements can apply to both UE and NW sided data collection. FFS if some don’t apply to both. 2. Study further requirements of other non-AI/ML use cases/services and understand if we can have more common requirements across different use cases. 3. Study termination points (i.e. understand who are producers, consumers of the data, data collection points. Understand requirements of use cases sensing, aI/ml, son/mdt, QoE etc) and protocol used for data transfer (e.g. IP vs. non-IP). 4. Study model transfer/delivery requirements/functions For next meeting companies can consider at least the following aspects: - Diverse types of data and its services/use case scenarios (e.g., AI/ML related data, sensing data, QoE, SON/MDT, etc); - For each type of data, study: - Applicable use case(s) - End point pairs (i.e., producer and consumer), including UE and RAN, UE and CN, RAN and CN, RAN and OAM; - Data size in a single reporting and total data size - Frequency of data reporting; - QoS requirements (e.g., latency, priority, GBR, packet error rate, etc);

According to the above agreements [1], the analysis of diverse data endpoints, data sizes, and QoS requirements is requested for use cases such as AI/ML and sensing.

In this paper, the data characteristics required by AI/ML and ISAC and the applicability of existing data collection mechanisms are discussed.

Discussion

Characteristics of Data Flows for AI/ML and ISAC

Data Flows for AI/ML

6G is expected to integrate AI/ML functionalities into the network, which would involves the LCM operations such as control/configuration, inference, training, monitoring and model transfer. From the standardized data collection framework point of view, the training data, the monitoring data and the model transfer data should be considered since these cases have not been handled in 5G AI/ML framework.

The specific data flow characteristics related to these operations are summarized below in Table 1.

Table 1: Characteristics of Data Flows for AI/ML Use Cases

| Category | Data Content (Example) | Endpoint Pair (Source -> Consumer) *RAN2 point of view | Data Size | Max service latency | Transmission Interval |
| --- | --- | --- | --- | --- | --- |
| Training Data | L1-RSRP Beam ID Positioning | UE -> RAN/CN RAN -> OAM | Medium | Tolerant | Long |
| Monitoring Data | Statistics Failure Report | UE -> RAN/CN (e.g., LMF) RAN -> CN (e.g., LMF) | Small | Medium | Moderate |
| Model Transfer Data | AI/ML Model Parameters | RAN/CN -> UE OAM/CN -> RAN | Large (225 Mbyte) (*1) | Tolerant | Long |

(*1): Option 4-1: sharing {target CSI, CSI feedback} dataset [2]

Data Flows for ISAC

Another promising service for 6G is the Integration of Sensing and Communication (ISAC). For ISAC operation, the transmission of analysed data and raw data are considered.

The specific data flow characteristics related to these operations are summarized below in Table 2.

Table 2: Characteristics of Data Flows for ISAC Use Cases

| Category | Data Content (Example) | Endpoint Pair (Source -> Consumer) *RAN2 point of view | Data Size | Max service latency | Refreshing Rate |
| --- | --- | --- | --- | --- | --- |
| 3GPP Sensing Data | Timing of receiving reflected waves Frequency of receiving reflected waves | UE -> RAN RAN -> CN UE -> CN | Medium (Large potential) | Variable (Low for motion monitoring (5 to 50 msec) (*2)) | Variable (Low for motion monitoring (0.1 sec) (*2)) |

(*2): Table 6.2-1: Performance requirements for 5G Wireless sensing [3]

As shown in Table 1 and Table 2, the following data flow characteristics would be observed for AI/ML and ISAC use cases:
- Data Size (Small to Large)
- QoS Requirement (Tolerant to Tight Latency requirement)
- Transmission Interval/Refreshing Rate (Low to Very High)

The following endpoint pairs would be observed for AI/ML and ISAC data flows:
- RAN2 Scope:
 UE <-> RAN, UE <-> CN (which may need coordination with SA2)
- Related Scopes:
 RAN <-> CN (which may need coordination with RAN3)
 RAN <-> OAM (which may need coordination with RAN3 and SA5)
Furthermore, it may need to discuss whether the UE should be aware of these diverse endpoint pairs and whether the UE has to differentiate the corresponding data flows to be transmitted over the radio interface (e.g., to identify RAN-terminated vs. CN-terminated data flow in Uplink).

Analysis of the Existing Data Collection Mechanisms

This section assesses the capability of the existing 5G architecture to support the diverse data flows presented in Sections 2.1.

UE - RAN Data Collection Mechanisms (e.g., MDT, RAN-Visible QoE)

- MDT/RV-QoE: These mechanisms use RRC signalling (C-Plane) for data collection and are primarily specialized for reporting small-volume data or transferring medium-volume logged data (which may accumulate to a large total volume) with delay-tolerance.

The existing UE - RAN transmission mechanisms (e.g., MDT, RAN-Visible QoE) may not be capable of handling the diverse characteristics and requirements for AI/ML and ISAC data flows.

UE - CN Data Collection Mechanisms (e.g., NAS (C-Plane), U-Plane)

- NAS (C-Plane) architecture: The NAS message is transferred via RRC signalling (C-Plane). Therefore, the same observations for MDT/RV-QoE are applicable.

- U-Plane architecture: The data collection scheme from the UE to a CN NF is currently studied in SA2, which is potentially considered as the baseline.

NAS (C-Plane) may not be capable of handling the diverse characteristic requirements for AI/ML and ISAC use cases.

Consistency of Transmission Mechanisms between Endpoints

The diverse endpoint pairs (e.g., CN <-> RAN, CN <-> UE, RAN <-> OAM) are observed for the data flows as in Observation 2. In the current 5G architecture, different paths for different endpoint pairs have been specified individually for each specific purpose. Consequently, the 5G architecture may have limitations to handle these paths in combination for new use cases. So, the 6G data collection framework needs a careful consideration  to avoid potential system-wide inefficiency.

It’s FFS whether the 5G approach, which relies on the existing C-Plane (e.g., RRC signalling) and U-Plane mechanisms, can handle the diverse endpoint pairs efficiently.

RAN2 should study the standardized data collection framework to support the various QoS requirements and data sizes to support the AI/ML and ISAC use cases, and coordinate with RAN1 to identify the necessary design targets including QoS and data sizes.

Further Considerations on Potential Approaches

For the next step, the high-level approaches would be considered as follows:

- Option 1: Develop new dedicated mechanisms for AI/ML and Sensing, individually.

- Option 2: Develop a new mechanism for AI/ML and Sensing, which aims to being applicable for other data collection cases (e.g., MDT) if feasible.

- Option 3: Study a fully unified mechanism to cover all the use cases.

Conclusion

In this contribution, the analysis of data flow characteristics for AI/ML and ISAC use cases and the potential requirements for the data collection framework are provided.

RAN2 is kindly asked to take into account of the observations and proposals below:

Observation 1	As shown in Table 1 and Table 2, the following data flow characteristics would be observed for AI/ML and ISAC use cases: 
- Data Size (Small to Large) 
- QoS Requirement (Tolerant to Tight Latency requirement) 
- Transmission Interval/Refreshing Rate (Low to Very High)

Observation 2	The following endpoint pairs would be observed for AI/ML and ISAC data flows: 
- RAN2 Scope:
  UE <-> RAN, UE <-> CN (which may need coordination with SA2) 
- Related Scopes:
  RAN <-> CN (which may need coordination with RAN3)
  RAN <-> OAM (which may need coordination with RAN3 and SA5) 
Furthermore, it may need to discuss whether the UE should be aware of these diverse endpoint pairs and whether the UE has to differentiate the corresponding data flows to be transmitted over the radio interface (e.g., to identify RAN-terminated vs. CN-terminated data flow in Uplink).

Observation 3	The existing UE - RAN transmission mechanisms (e.g., MDT, RAN-Visible QoE) may not be capable of handling the diverse characteristics and requirements for AI/ML and ISAC data flows.

Observation 4	NAS (C-Plane) may not be capable of handling the diverse characteristic requirements for AI/ML and ISAC use cases.

Observation 5	It’s FFS whether the 5G approach, which relies on the existing C-Plane (e.g., RRC signalling) and U-Plane mechanisms, can handle the diverse endpoint pairs efficiently.

Proposal 1	RAN2 should study the standardized data collection framework to support the various QoS requirements and data sizes to support the AI/ML and ISAC use cases, and coordinate with RAN1 to identify the necessary design targets including QoS and data sizes.

References

- “RAN2 Chair’s Notes”, (InterDigital), RAN2#131bis

- “LS on signalling feasibility of dataset and parameter sharing”, R1-2410922

- “Service requirements for Integrated Sensing and Communication; Stage 1”, TS 22.137