3GPP TSG-RAN2#132	R2-2509092

Dallas, USA, 17th – 21st Nov. 2025

Agenda Item:	8.9.2

Source:	Ericsson

Title:	IoT NTN, Z001 and RRC issues and MAC issues

Document for:	Discussion, Decision

# 1	Introduction

Here we discuss some outstanding issues for IoT NTN phase 3.

# 2	Discussion

## 2.1	RRC: Support narrow band MPDCCH configuration for CB-Msg3-EDT

We received an LS from RAN1 [2] with

RAN2 ask RAN1 to check the specification impact in RAN1 to support 2 MPDCCH narrowbands for CB-Msg4 monitoring, and update the corresponding specification if needed.

RAN1 response for support MPDCCH narrowband configuration for CB-Msg3-EDT:

From RAN1 perspective, introducing 2 MPDCCH narrowbands requires additional RAN1 standard effort. RAN1 would like to inform RAN2 that there are no concerns with supporting two MPDCCH narrowbands, but RAN2 should decide on a mechanism to ensure the UE is monitoring a unique narrowband.

Thus, monitoring two MPDCCH has some drawbacks as we must in some way decide which narrowband a UE shall monitor, and that will be tricky this late in this WI.

Having two MPDCCHs enables sending more CB-Msg4, however, it seems not necessary as CB-Msg4 can be shared between 4 users, and eNB can time multiplex many more CB-Msg4 and at the low loads that we were looking at in the capacity evaluations, this is sufficient.

Remove the second MPDCCH for monitoring for CB-Msg4.

That is here

mpdcch-Narrowband-r19			SEQUENCE (SIZE(1..2)) OF INTEGER (1..maxAvailNarrowBands-r13),

Change to

mpdcch-Narrowband-r19			INTEGER (1..maxAvailNarrowBands-r13),

## 2.2	RRC: Z001 CB-Msg3 transmission window configuration

As a resolution for Z001, the following was proposed:

cb-Msg3-TxWindow-r19			SEQUENCE {

windowSize-r19						INTEGER {3..10

},

windowPeriodicity-r19				ENUMERATED {n1, n2, n4, n8, n16, n24, n32, n48, n64,

n96, n128, n160, n192, n224, n256, n288}

}																		OPTIONAL,	--Need OP

cb-Msg3-TxWindow CB-Msg3 transmission window configuration. The start time of the CB-Msg3 transmission window is aligned with the PUSCH start time indicated by pusch-StartSFN-r19 and pusch-StartSubframe-r19. When cb-Msg3-NumOfReplicas-r19 equals 1, this field is absent. For windowSize, value 3 corresponds to 3 PUSCH periods, 4 corresponds to 4 PUSCH periods and so on. For windowPeriodicity, value n1 corresponds to 10ms, n2 corresponds to 20ms and so on.

However, it is not clear what a PUSCH period is.

Can it be the pusch-Periodicity in cb-Msg3-TimeResource?

Last meeting we decided:

[Z001]:

9.	Change the unit of windowSize to PUSCH periodicity for IoT NTN CB-Msg3-EDT configuration. We keep the unit of windowPeriodicity in tens of ms (aim to decide on the actual range for both in the next round of ASN.1 review)

10.	Keep the PUSCH configuration separate from the window configuration as in the current specification.

11.	RAN2 understands that the PUSCH resource outside of the window is invalid for CB-Msg3-EDT (FFS if/how to clarify this in the spec)

We note that to lower collision probability, it is beneficial to spread the CB-Msg3 opportunities in time instead of in frequency, as we showed in Appendix A.2 [1].

Therefore, having only up to 10 consecutive PUSCH resources is too limiting.

Clarify how the windowSize(-NB) is defined in field description

To ensure low collision probability, by spreading spread CB-Msg3 opportunities in time instead of frequency, the maximum windowSize(-NB) is 100

We have

cb-Msg3-TimeResource-r19		SEQUENCE {

pusch-Periodicity-r19					ENUMERATED {sf2, sf4, sf8, sf16, sf32, sf64, sf128,

sf256},

pusch-StartSFN-r19					INTEGER (0..1023),

pusch-StartSubframe-r19				INTEGER (0..9)

},

that is, the pusch-Periodicity is sf2, but sf2 is not defined anywhere.

Also, there is no field description for any of the cb-Msg3-TimeResource fields.

Add field descriptions for cb-Msg3-TimeResource(-NB) fields

## 2.3	RRC: TBS limitations

Regarding TBS and CE levels we have agreed:

- RAN2 assumes that CB-msg3 EDT cell specific PUSCH resources are associated with number of repetitions, RSRP selection threshold to determine the CE level and largest TBS for Msg3 transmission, but this has to be confirmed by RAN1. FFS if there is an RSRP threshold that determines whether CB-msg3 EDT cannot be used (the UE will have to use 4-step RA)

- The UE triggers CB-Msg3 only if the size of pending UL data is less than the configured maximum TBS (FFS if the maximum TBS is same or different for different CE levels)

- The maximum TBS could be different for different CE levels.

6.	We don’t support multiple TBSs per CE level for CB-Msg3 EDT

7.	We introduce one additional value smaller than the smallest possible TBS size at the moment (328bits), i.e. 144bits (discuss further details when drafting the LS to RAN1).

- CB-msg3 EDT cell specific PUSCH resources for Msg3 transmission are provided per CE level (FFS whether we have a CE level specific configuration for DSA)

- The CB-Msg3-EDT configuration is configured per carrier (including anchor and non-anchor carrier). Within each carrier, the CB-Msg3-EDT could be configured per CE level

Thus, each CE level may have only one TBS on each carrier.

The UE knows the TBS for each CE level and each carrier.

In Rapporteur correction on IoT NTN Ph3:

Directory Listing /ftp/Email_Discussions/RAN2/[Misc]/ASN1 review/Rel-19 2025-09/36331/Spec ASN1 Review/Endorsed CRs after RAN2#131bis/IoT NTN

We have for eMTC:

numRUs-r19							BIT STRING (SIZE(2)),

prb-AllocationInfoSet-r19			SEQUENCE (SIZE(1..48)) OF BIT STRING (SIZE(10)),

mcs-r19								BIT STRING (SIZE(4)),

numRepetitions-r19					BIT STRING (SIZE(3)),

Now, those parameters are sufficient to point to one and only one TB size, freely from all possible TBSs.

Thus it seems the parameter

cb-Msg3-TBS-r19							ENUMERATED {b144, b328, b408, b504, b600, b712,

b808, b936},

Is too limiting compared to what can be configured.

The cb-Msg3-TBS unnecessarily limits the TBS

Further, as each UE knows the TBS for each CE level and carrier, the cb-Msg3-TBS is redundant.

Remove the redundant parameter cb-Msg3-TBS

That enables the operators to select larger or smaller TBSs depending on the deployment, service type, and coverage etc.

Similar for NB-IoT:

cb-Msg3-PhysicalConfig-r19		SEQUENCE {

npusch-NumRUsIndex-r19					INTEGER (0..7),

npusch-NumRepetitionsIndex-r19			INTEGER (0..7),

npusch-SubCarrierSetList-r19		CHOICE {

npusch-SubCarrierSetList-khz15		SEQUENCE (SIZE(1..12)) OF INTEGER (0..18),

npusch-SubCarrierSetList-khz3dot75	SEQUENCE (SIZE(1..48)) OF INTEGER (0..47)

},

npusch-MCS-r19						CHOICE {

singleTone							INTEGER (0..10),

multiTone							INTEGER (0..13)

},

Now, those parameters are sufficient to point out one unique TBS, freely from all possible TBSs.

Thus also for NB-IoT it seems the parameter

cb-Msg3-TBS-NB-r19						ENUMERATED {b144, b328, b408, b504, b584, b680, b808,

b936},

Is too limiting compared to what can be configured.

Remove the redundant parameter cb-Msg3-TBS-NB

## 2.4	MAC: Backoff for CB-Msg3 attempts

This was discussed last meeting and the endorsed R2-2507656 MAC CR has:

Backoff parameter value for CB-Msg3-EDT procedure is computed according to the backoff multification factor in Table 7.2-x.

Table 7.2-x: Backoff Multification Factor for CB-Msg3-EDT.

| Index | Backoff multification factor |
| --- | --- |
| 0 | 0 |
| 1 | 3 |
| 2 | 6 |
| 3 | 9 |
| 4 | 12 |
| 5 | 15 |
| 6 | 18 |
| 7 | 21 |
| 8 | 24 |
| 9 | 27 |
| 10 | 30 |
| 11 | 33 |
| 12 | 36 |
| 13 | Reserved |
| 14 | Reserved |
| 15 | Reserved |

The backoff parameter values for CB-Msg3-EDT is the the backoff multification factor multiplied by the configured CB-Msg3 transmission window periodicity provided by windowPeriodicity in ms.

The reserved values of the backoff multification factor if received by the current release version UEs shall be taken as 36.

A long back off introduces long delays, but that is natural and necessary at a high peak in load. Considering that for collision to occur there must be very many users that collide, and there will also be new users arriving in the future CB-Msg3 transmission windows – therefore we need even higher numbers than in this table.

It must be possible to spread failing users access attempts among future CB-Msg3 transmission windows. The back off values must cover many multiples of CB-Msg3 periodicity to not cause collision with new users arriving in those future CB-Msg3 transmission windows.

For Backoff Multiplication Factor for CB-Msg3-EDT, use the values 0, 3, 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 55, reserved, reserved, reserved

## 2.5	MAC header for CB-Msg4

We have in the endorsed R2-2507656 MAC CR

Figure 6.1.7-5: Example of MAC PDU consisting of a MAC header, MAC CMRs, MAC SDUs and padding

And we have:

-	T: The Type field indicates the type of the MAC subheader. The T field is set to "00" to indicate the presence of a CB Backoff Indicator field in the subheader (BI). The T field is set to "01" to indicate the presence of a CMR in the subheader. The T field is set to "10" to indicate the presence of a Logical Channel ID (LCID) field in the subheader. The value "11" is reserved for future use. The size of the Type field is 2 bits;

A UE must look through all sub-headers and when it finds a header addressed to it (matching UE CR ID), it needs to decode that sub-header and find the corresponding MAC payload.

This can be quite complex for a UE because each MAC header can have a different size, and each MAC payload MAC CMR + (MAC SDU) can have a different size.

UEs must know the size of every sub-header to be able to find the start of the MAC payload.

This is also true for all UEs if the reserved T value “11” is used in the future, that is all legacy R19 UEs must (at least) be able to know the size of the reserved MAC header to know where the first MAC payload starts. If the reserved MAC header is not placed at the end of the MAC header, the legacy UEs must also know the size of the corresponding MAC payload for the reserved MAC header.

To be future proof, all UEs must know the size of the reserved T value “11” subheader.

RAN2 to discuss future proofing T Type field value 11. For example, deciding that the T Type 11 subheader size is two octets.

# 3	Conclusion

In this paper we have made these observations:

Observation 1	The UE knows the TBS for each CE level and each carrier.

Observation 2	The cb-Msg3-TBS unnecessarily limits the TBS

Observation 3	It must be possible to spread failing users access attempts among future CB-Msg3 transmission windows. The back off values must cover many multiples of CB-Msg3 periodicity to not cause collision with new users arriving in those future CB-Msg3 transmission windows.

Observation 4	UEs must know the size of every sub-header to be able to find the start of the MAC payload.

Observation 5	To be future proof, all UEs must know the size of the reserved T value “11” subheader.

We propose:

Proposal 1	Remove the second MPDCCH for monitoring for CB-Msg4.

Proposal 2	Clarify how the windowSize(-NB) is defined in field description

Proposal 3	To ensure low collision probability, by spreading spread CB-Msg3 opportunities in time instead of frequency, the maximum windowSize(-NB) is 100

Proposal 4	Add field descriptions for cb-Msg3-TimeResource(-NB) fields

Proposal 5	Remove the redundant parameter cb-Msg3-TBS

Proposal 6	Remove the redundant parameter cb-Msg3-TBS-NB

Proposal 7	For Backoff Multiplication Factor for CB-Msg3-EDT, use the values 0, 3, 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 55, reserved, reserved, reserved

Proposal 8	RAN2 to discuss future proofing T Type field value 11. For example, deciding that the T Type 11 subheader size is two octets.

# 4	References

R2-2502886, “UL capacity enhancements for IoT NTN”, Ericsson, RAN2#129bis, April 2025

R2-2508014, “Reply LS on power ramping and RRC configuration for CB-Msg3-EDT”, From: RAN1 To: RAN2, RAN2#132, Nov. 2025