# 3GPP TSG-RAN WG2 #132	R2-2509154

# Dallas, US, 17 – 21 Nov 2025

Agenda Item:	8.0.1

Source: 	Ericsson

Title: 	38.331 ASN.1 Review, Class 0 issues, v024

Document for:	Discussion and decision

# Guidelines

- This file is used to log NR 38.331 ASN.1 Review Class 0 issues.

  - - Typo, minor wording improvement etc.

  - - ASN.1 field not following naming rules (e.g. incorrect suffix, capitalization, “-“, etc).

- Fill in the columns, see example.

  - Make sure the inserted specification text is unique, such that the location of the issue is simple to find.

  - Avoid indicating duplicated issues by checking if the concerned specification text is already reported in the table.

  - Step the file name v(x) -> v(x+1) and upload to ftp server.

- Try to identify related WI/TEI for the issue.

- The “Status” column is filled in by the WI CR editor/RRC Spec Rapporteur when the issue is implemented in a CR. Use the following:

  - WI-code (when implemented in a WI CR

  - REJ (when issue is rejected)

  - DUPL (for duplicate issues)

# Class 0 issues

| Issue | ASN1? Y/N | Copied existing specification text. Text should be unique, so that it can be easily found in the specification. If needed, add also the new text. | Comment/description/ correction | Related WI | Email address | Status |
| --- | --- | --- | --- | --- | --- | --- |
| Ex 1 | N N | 2> derive the KUPint key associated with the integrityProtAlgorithm indicated in the SecurityModeCommand message, as specified in TS 33.501 [11]; | Missing italics. | Mob | hakan.l.palm@ericsson.com |  |
| Ex 2 | N | PUSCH scheduled by RAR UL grant (see 38.213 clause 8.3 and 38.214 clause 6.1.2.2) and uses interlaced PUCCH Format 0, 1, 2, and 3 for cell-specific PUCCH (see TS 38.213 [13], clause 9.2.1). | Incorrect reference, should be 9.2.101. | SDT | hakan.l.palm@ericsson.com |  |
| Ex 3 | Y | RbSetGroup, rbSetGroups | RB-SetGroup, rb-SetGroups |  | hakan.l.palm@ericsson.com |  |
| 4 | N | 1> if the received OtherConfig includes gapOccasionCancelRatioReportConfig: 2> if gapOccasionCancelRatioReportConfig is set to setup: | It should be otherConfig. | XR | xuhao@catt.cn |  |
| 5 | N | UE-TimersAndConstantsRemoteUE field descriptions t300-RemoteUE Indicates the timer value of T300 used by L2 U2N Remote UE. If the field is absent, the timer value indicated in t300 applies to L2 U2N Remote UE. The effective T300 value for the L2 U2N Remote UE, accounting for both the Uu and PC5 hop components,, is obtained by multiplying the base T300 timer value by the Hop Count. For a single-hop scenario involving one Relay UE, the Hop Count is 1. For multi-hop scenarios involving two or three Relay UEs, the Hop Count is 2 or 3, respectively. t319-RemoteUE Indicates the timer value of T319 used by L2 U2N Remote UE. If the field is absent, the timer value indicated in t319 applies to L2 U2N Remote UE. The effective T319 value for the L2 U2N Remote UE, accounting for both the Uu and PC5 hop components,, is obtained by multiplying the base T319 timer value by the Hop Count. For a single-hop scenario involving one Relay UE, the Hop Count is 1. For multi-hop scenarios involving two or three Relay UEs, the Hop Count is 2 or 3, respectively. | Redundant punctuation should be deleted. | SLRelay | xuhao@catt.cn |  |
| 6 | N | rach-Less This field is set if the successful HO report is trigged by RACH-less LTM cell switch. | Typo, it should be “triggered” | SONMDT | tangxun@catt.cn | PropAgree |
| 7 | N | 3> set pSCellId to the the global cell identity and tracking area code, if available, and otherwise the physical cell identity and carrier frequency of the source PSCell (in case of PSCell change) or PSCell (in case of no PSCell change); | Duplicate “the” | SONMDT | tangxun@catt.cn | PropAgree |
| 8 | N | 3> set the measResultListNR in measResultNeighCells to include all the available measurement quantities of the best measured cells, other than the source PCell (in case HO failure) or PCell (in case RLF), and other than the source PSCell (in case of PSCell change) or PSCell (in case of no PSCell change) if the UE was configured with condExecutionCond and condExecutionCondPSCell and if the UE supports RLF-Report for conditional handover with candidate SCG ordered such that the cell with highest SS/PBCH block RSRP is listed first if SS/PBCH block RSRP measurement results are available, otherwise the cell with highest SS/PBCH block RSRQ is listed first if SS/PBCH block RSRQ measurement results are available, otherwise the cell with highest SS/PBCH block SINR is listed first, based on the available SS/PBCH block based measurements collected up to the moment the UE detected failure; | A comma should be added between “SCG” and “ordered”. | SONMDT | tangxun@catt.cn | PropAgree |
| 9 | N | fulfilledConfigWhenChoOnly This field indicates if the execution conditions for conditional handover or conditional PSCell change/addition was fulfilled at the time of receiving a complementary conditional reconfiguration i.e., a conditional reconfiguration for a candidate PCell for which atleast one CHO with conditional SCG is already configured. | A space should be added between “at” and “least”. | SONMDT | tangxun@catt.cn | PropAgree |
| 10 | N | 1> if configured to provide its preference on time offset for LP-WUS monitoring of a cell group: 2> if [the UE has a preference on time offset for LP-WUS monitoring of the cell group and] the UE did not transmit a UEAssistanceInformation message with lpwus-OffsetPreference for the cell group since it was configured to provide its preference on time offset for LP-WUS monitoring of the cell group for power saving; or | Typo “[“ and “]” | LPWUS | rama.kumar@huawei.com |  |
| 11 | Y | lpwus-OverlaidSeqRoots in “lpwus-MvalueAndSeqConfigFR1-r19” and “lpwus-MvalueAndSeqConfigFR2-r19” | Missing “-r19” for “lpwus-OverlaidSeqRoots” in 3 places | LPWUS | rama.kumar@huawei.com |  |
| 12 | Y | lpwus-LoFrameOffsetList-r19, lpwus-MoNumPerLo-r19, lpwus-PoNumPerLo-r19 | lpwus-LO-FrameOffsetList-r19, lpwus-MO-NumPerLO-r19, lpwus-PO-NumPerLO-r19 | LPWUS | rama.kumar@huawei.com |  |
| 13 | Y | lpss-StartSymbol-r19 SEQUENCE { startSymbol1-r19 INTEGER (0..10), startSymbol2-r19 INTEGER (0..10) OPTIONAL } OPTIONAL, -- Need R | Need code is missing TODO: this should be Class 1 | LPWUS | rama.kumar@huawei.com |  |
| 14 | Y | lp-SubgroupConfig-r19 LP-SubgroupConfig-r19 | Better to rename to “lpwus-SubgroupConfig” to align with others | LPWUS | rama.kumar@huawei.com |  |
| 15 | Y | CSI-LogMeasInfo-r19 ::= SEQUENCE { refCSI-LoggedMeasurementConfigId-r19 CSI-LoggedMeasurementConfigId-r19, csi-RS-MeasResultList-r19 SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF CSI-RS-MeasResult-r19 OPTIONAL, ssb-MeasResultList-r19 SEQUENCE (SIZE (1..maxNrofSSBs-r16)) OF SSB-MeasResult-r19 OPTIONAL, timeGap-r19 ENUMERATED {true} OPTIONAL, ... } | The extension marker should be in the next row. | AIML | tangxun@catt.cn |  |
| 16 | N | timeGap Indicates that there was a time gap, longer than the logging periodicity, between the reported measurement results in this instance of CSI-LogMeasInfoList and the previous instance of CSI-LogMeasInfoList with the same refCSI-LoggedMeasurementConfigId for the same serving cell. | Should be csi-LogMeasInfoList. | AIML | tangxun@catt.cn |  |
| 17 | N | Missing italics 5.2.2.3.1 : 4> if the UE is in RRC_CONNECTED while T311 is running: 5> if the SIB1 acquisition is upon receiving an indication that the system information has changed or upon receiving a PWS notification: 6> acquire the SIB1 (see clause 5.2.2.2.2), which is scheduled as specified in TS 38.213 [13]; Just to NOTE that we don’t need to have italics for “OD-SIB1 acquisition” as we don’t have “OD-SIB1” ASN.1 definition. This is correctly implemented in the CR. So no worries. | SIB1 | NES | Jarkko.t.koskela@nokia.com |  |
| 18 | N | In 5.2.2.3.1 in the note at the end of section first “SIB1” is missing italics. Second SIB1 in the note is regarding “SIB1 request procedure”. That does not need to be changes I guess as it is name of procedure and not ASN.1. But up to rapporteur to decide on that (like everything else). | SIB1 | NES | Jarkko.t.koskela@nokia.com |  |
| 19 | N | In 5.2.2.3.3x multiple instances of SIB1 don’t have italics – Note that not all need those possibly when we refer to “SIB1 request procedure” but we need use italics when we refer to SIB1 message e.g. in case “acquire SIB1 message” | SIB1 | NES | Jarkko.t.koskela@nokia.com |  |
| 20 | N | t-RxDiscard Timer for the AMD RLC PDU(s) discard at the Rx side of the RLC entity, see TS 38.322 [4]. For the value of the IE T-RxDiscard, value ms10 means 10 milliseconds, value ms20 means 20 milliseconds, and so on. The value of the field should not be lower than that configured by the field t-Reassembly or t-ReassemblyExt. | T-RxDiscard should be t-RxDiscard | XR | zhangyujian@xiaomi.com |  |
| 21 | N | sbfd-RSRP-ThresholdRO-TypeUsage Indicate how the SBFD aware UE chooses RACH occasion type using sbfd-RSRP-ThresholdRO-Type for the initial PRACH preamble transmissions and is always configured together with sbfd-RSRP-ThresholdRO-Type. With value above, the SBFD aware UE chooses the second PRACH occasions if the measured downlink pathloss reference RSRP is above sbfd-RSRP-ThresholdRO-Type and chooses the first PRACH occasions otherwise. With value below, the SBFD aware UE chooses the second PRACH occasions if the measured downlink pathloss reference RSRP is below sbfd-RSRP-ThresholdRO-Type and chooses the first PRACH occasions otherwise. If all of sbfd-RO-Type, sbfd-RSRP-ThresholdRO-Type and sbfd-RSRP-ThresholdRO-TypeUsage are absent, it is up to UE implementation to determine the RO type. | The highlighted sbfd-RSRP-ThresholdRO-Type should be italic. | SBFD | zhangyujian@xiaomi.com |  |
| 22 | N | sbfd-Config2-Transmission Indicates that the PUCCH and PUSCH transmissions can be in SBFD symbols and non-SBFD symbols in different slots in a given UL BWP (see TS 38.213 [13], clause 11 and TS 38.214 [19], clause 6).If not enabled, the restriction that the transmissions are restricted to SBFD symbols only or non-SBFD symbols only is applied for PUCCH and PUSCH transmissions in the given UL BWP. | Space is needed before “If not enabled”. | SBFD | zhangyujian@xiaomi.com |  |
| 23 | N | cli-RSSI-PeriodicityAndOffset Indicates the periodicity and slot offset for this CLI-RSSI-MeasResource, with the same value range as CSI-ResourcePeriodicityAndOffset. qcl-InfoPeriodic-CLI-RSSI-MeasResource Indicates a reference to one TCI-State in TCI-States for providing the QCL source and QCL type for a target periodic CLI-RSSI-MeasResource startSymbol Indicates the starting symbol of the CLI-RSSI-MeasurementResource within a slot | Highlighted part should be in italic. Period is missing at the end of description for qcl-InfoPeriodic-CLI-RSSI-MeasResource and startSymbol. | SBFD | zhangyujian@xiaomi.com |  |
| 24 | N | symbolType Configures the valid symbol type for PUCCH carrying P-CSI or SP-CSI when the transmissions are restricted to SBFD symbols only or non-SBFD symbols only for the UL BWP. The network does not configure this field if the transmissions can be in SBFD symbols and non-SBFD symbols in different slots for the UL BWP. (see TS 38.214 [19], clause 7.2) | Move the period to the end as in “UL BWP (see TS 38.214 [19], clause 7.2).” | SBFD | zhangyujian@xiaomi.com |  |
| 25 | N | 2> if the UE is configured in this RRCReconfiguration message to provide location information for assisted SMTC configuration in RRC_CONNECTED state: 3> include referenceLocationReport; | It seems that the UE provides its location, but actually the UE provides the reference location. Suggest using ‘provide reference location information’ instead of ‘provide location information’. | NTN | lixiaolong1@xiaomi.com |  |
| 26 | N | 1> if the received otherConfig includes the assisted-SSB-MTC-Config: 2> if the assisted-SSB-MTC-Config is set to setup: 3> consider itself to be configured to provide location information for assisted SMTC configuration in RRC_CONNECTED state in accordance with 5.7.4; 2> else: 3> consider itself not to be configured to provide location information for assisted SMTC configuration in RRC_CONNECTED state. | It seems that the UE provides its location, but actually the UE provides the reference location. Suggest using ‘provide reference location information’ instead of ‘provide location information’. | NTN | lixiaolong1@xiaomi.com |  |
| 27 | N | 1> if configured to provide location information for assisted SMTC configuration in RRC_CONNECTED state: 2> if the current closest reference locations are different from the ones indicated in the last transmission including referenceLocationReport: 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide location information for assisted SMTC configuration; | It seems that the UE provides its location, but actually the UE provides the reference location. Suggest using ‘provide reference location information’ instead of ‘provide location information’. | NTN | lixiaolong1@xiaomi.com |  |
| 28 | N | 1> if transmission of the UEAssistanceInformation message is initiated to provide location information for assisted SMTC configuration in RRC_CONNECTED state according to 5.7.4.2; 2> include the referenceLocationReport with a number of closest reference locations to the current UE’s position determined by closestLocsToReport; | It seems that the UE provides its location, but actually the UE provides the reference location. Suggest using ‘provide reference location information’ instead of ‘provide location information’. | NTN | lixiaolong1@xiaomi.com |  |
| 29 | N | 1> if the value of ltm-NoSecurityChangeID contained in the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoSecurityChange within VarLTM-ServingCellNoSecurityChange: | Based on the ASN.1 structure of VarLTM-ServingCellNoSecurityChange VarLTM-ServingCellNoSecurityChange UE variable -- ASN1START -- TAG-VARLTM-SEVINGCELLNOSECURITYCHANGE-START VarLTM-ServingCellNoSecurityChange-r19 ::= SEQUENCE { ltm-ServingCellNoSecurityChangeID-r19 INTEGER (1..maxNrofLTM-Configs-plus1-r18) OPTIONAL, ltm-SK-Counters-r19 SEQUENCE (SIZE (1..maxSecurityCellSet-r18)) OF SK-CounterConfigLTM-r19 OPTIONAL } -- TAG-VARLTM-SEVINGCELLNOSECURITYCHANGE-STOP -- ASN1STOP It should be ltm-ServingCellNoSecurityChangeID. | MOB | xiongyi3@xiaomi.com |  |
| 30 | N | 2> else if the LTM cell switch is triggered on the SCG: 3> consider the first sk-Counter value in the ltm-SK-Counters within the VarLTM-ServingCellNoSecurityChange associated to the the field ltm-NoSecurityChangeID as the selected sk-Counter value, and update the secondary key by performing security key update procedure as specified in 5.3.5.7; 3> remove the selected sk-Counter value from the ltm-SK-Counters within the VarLTM-ServingCellNoSecurityChange; | The Font Color of ”ltm-SK-Counters ” shall be black. | MOB | xiongyi3@xiaomi.com |  |
| 31 | N | 1> else (LTM cell switch triggered upon cell selection performed while timer T311 was running or upon the fulfilment of LTM cell switch execution conditions (as specified in clause 5.3.5.18.x): 2> apply the RRCReconfiguration message in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config related to the LTM candidate configuration identity for the selected cell (i.e., in accordance with 5.3.5.18.x or 5.3.7.3) according to clause 5.3.5.3; | The Font Color of “upon the fulfilment of LTM cell switch execution conditions (as specified in clause 5.3.5.18.x” shall be black. And there is no Underline for “upon the fulfilment of LTM cell switch execution conditions (as specified in clause 5.3.5.18.x” | MOB | xiongyi3@xiaomi.com |  |
| 32 | N | ##### 5.3.5.18.z LTM sk-Counter configuration relese The UE shall: 1> for each ltm-NoSecurityChangeID value included in the ltm-SK-CounterConfigToRemoveList that is part of the current ltm-SK-Counters in VarLTM-ServingCellNoSecurityChange: 2> remove the entry with the matching ltm-NoSecurityChangeID from the ltm-SK-Counters in VarLTM-ServingCellNoSecurityChange. | Correction 1: A typo: “relese” -> “release” Correction2: Based on the ASN.1 structure of LTM-ConfigNRDC-r19 LTM-ConfigNRDC-r19 ::= SEQUENCE { ltm-ConfigurationSCG-r19 LTM-Config-r18 OPTIONAL, -- Need M ltm-SK-CounterConfigToAddModList-r19 SEQUENCE (SIZE (1..maxSecurityCellSet-r18)) OF SK-CounterConfigLTM-r19 OPTIONAL, -- Need N ltm-SK-CounterConfigToReleaseList-r19 SEQUENCE (SIZE (1..maxSecurityCellSet-r18)) OF LTM-NoSecurityChangeId-r19 OPTIONAL, -- Need N ... ltm-SK-CounterConfigToRemoveList should be ltm-SK-CounterConfigToReleaseList. | MOB | xiongyi3@xiaomi.com |  |
| 33 | N | NOTE 2: It is up to UE implementation to use the cell/tracking area list and/or the Target Service Area in the USD or the ISA(s) in SIBXX to avoid acquiring the MCCH when the UE is outside the MBS service area of the MBS broadcast service. | Target Service Area is provided in the Service Announcement as one of the following Cell ID, TAI or geographical definition (ISA) (refer to SP-250930). The AF generates this input information (single/unique information source) which is the same regardless it is delivered either via USD or SIB to the UE. For the purpose of geofencing in NTN, the geographical definition will often be used. Provided Target Service Area is a general concept, the existing NOTE in the running CR needs to be ammended to maintain clarity in the specification and compatibility to existing TN behavior. Target Service Area is a general concept which include Cell ID, TAI and geographical description so the text referring especifically to cell/TAI list can be removed. NOTE 2: It is up to UE implementation to use the Target Service Area in the USD or the ISA(s) in SIBXX to avoid acquiring the MCCH when the UE is outside the MBS service area of the MBS broadcast service. | NTN | Ignacio.pascual.pelayo@ericsson.com |  |
| 34 | N | Clause 5.5a.1.3: 1> store the received AreaConfigurationNTN-List, if included, in VarLogMeasConfig; New Text: 1> store the received areaConfigurationNTN-List, if included, in VarLogMeasConfig; | Field name should start with lower case. | SONMDT | mani.thyagarajan@nokia.com | propAgree |
| 35 | N | Clause 5.5a.3.2: 1> if AreaConfigurationNTN-List is included in VarLogMeasConfig: 2> if location information is available, and is outside of all areas indicated by AreaConfigurationNTN-List; or New Text: 1> if areaConfigurationNTN-List is included in VarLogMeasConfig: 2> if location information is available, and is outside of all areas indicated by areaConfigurationNTN-List; or | Field name should start with lower case. | SONMDT | mani.thyagarajan@nokia.com |  |
| 36 | v | Clause 3.1 Applicable AI/ML configuration: Configuration according to which an AI/ML functionality is determined to be applicable by the UE, as defined in TS 38.300 [2]. | Definition not be referenced in the whole specification. Can be removed? [AIML WI CR rapporteur]: RIL N031 refers to modifying this definition. Deleting the definition is an alternative to modifying the definition, so it should be discussed together with RIL N031. | AIML | kimba@vivo.com | DUPL |
| 37 | v | Clause 4.2.2 - SRBx is for RRC messages which include logged measurement information for network-side data collection, all using DCCH logical channel. SRBx has a lower priority than SRB1 and can only be configured by the network after AS security activation. | Network-side data collection is a general description. Logged MDT can be treated as network-side data collection. It is better to add “for AI/ML” here. “- SRBx is for RRC messages which include logged measurement information for network-side data collection for AI/ML, all using DCCH logical channel. SRBx has a lower priority than SRB1 and can only be configured by the network after AS security activation. ” | AIML | kimba@vivo.com |  |
| 38 | v | Clause 5.3.5.3 6> if the applicabilityStatus is set to inapplicable | “inapplicable” Should be italic | AIML | kimba@vivo.com |  |
| 39 | v | Clause 5.3.5.3 6> if the applicabilityStatus is set to inapplicable: | “inapplicable” Should be italic | AIML | kimba@vivo.com |  |
| 40 | v | Clause 5.3.13 6> if the applicabilityStatus is set to inapplicable: | “inapplicable” Should be italic | AIML | kimba@vivo.com |  |
| 41 | v | Clause 5.7.4 7> if the applicabilityStatus is set to inapplicable: | “inapplicable” Should be italic | AIML | kimba@vivo.com |  |
| 42 | v | Clause 6.2.2 lowPowerState It is set to 'true' if the UE has entered a low power state. | :”'true'” Should be italic | AIML | kimba@vivo.com |  |
| 43 | v | Clause 6.2.2 CSI-LogMeasInfoCell-r19 ::= SEQUENCE { cellId-r19 CHOICE { cellGlobalId-r19 CGI-Info-Logging-r16, pci-arfcn-r19 PCI-ARFCN-NR-r16 }, csi-LogMeasInfoList-r19 SEQUENCE (SIZE (1..maxLogCSI-MeasReport-r19)) OF CSI-LogMeasInfo-r19, ... } | pci-arfcn-r19 “a” should Capital ltter | AIML | kimba@vivo.com |  |
| 44 | V | Clause 6.3.2 releaseConfigurationPreference Indicates the UE's preference to release the configuration associated to applicabilityReportConfigId (e.g. due to model unavailability), if the applicabilityStatus is set to 'inapplicable'. | “inapplicable” Should be italic | AIML | kimba@vivo.com |  |
| 44 | N | Clause 5.3.12 1> if the indicated serving cell is referred to by pucch-Cell included in CSI-ReportUE-IBR of an associated CSI-ReportConfig; 2> release pucch-Resource indicated in the associated CSI-ReportUE-IBR; | “if” statement should end in : instead of ; | MIMO | andrew.lappalainen@nokia.com |  |
| 45 | Y | CodebookConfig-r19 ::= SEQUENCE { codebookType CHOICE { type1 CHOICE { typeI-SinglePanel-r19 ... typeI-MultiPanel-r19 ... } type2 CHOICE { etypeII-r19 ... typeII-FePortSelection-r19 ... typeII-Doppler-r19 ... } } } | There seems to be unnecessary space in front of all the codebook configs (e.g. comparing them to the -r18 codebooks). They could be shifted to left. | MIMO | andrew.lappalainen@nokia.com |  |
| 46 | Y | resourcesForChannelCJTC-r19 SEQUENCE { resourceSet2CJTC-r19 …, resourceSet3CJTC-r19 …, resourceSet4CJTC-r19 … } mr-SelectedResources-r19 SEQUENCE { firstSelectedResource-r19 … secondSelectedResource-r19 … } | There seems to be unnecessary space in front of all these fields. They could be shifted left | MIMO | andrew.lappalainen@nokia.com |  |
| 47 | N | Conditional Presence; Explanation. CJTC; This field is absent if resourcesForChannel2 or resourcesForChannelTDCP is configured. It is optionally present, Need R, otherwise.. Conditional presence table for CSI-AperiodicTriggerStateList | resourcesForChannel2 and resourcesForChannelTDCP should be in italics | MIMO | andrew.lappalainen@nokia.com |  |
| 48 | Y | reportQuantity-r19 CHOICE { cjtc-Dd-r19 NULL, cjtc-F-r19 NULL, cjtc-P-r19 NULL, cjtc-Dd-F-r19 NULL } | There seems to be unnecessary space in front of all these fields. They could be shifted left | MIMO | andrew.lappalainen@nokia.com |  |
| 49 | Y | CSI-ReportUE-IBR-r19 <rest of config omitted> | The spacing for a lot of the fields within CSI-ReportUE-IBR is too far to the right or not aligned with one another (e.g. subfields of pusch-ResourceOfModeB-r19 are not aligned, and subfields of pucch-Resource-r19 are not aligned). Fields should be properly aligned. | MIMO | andrew.lappalainen@nokia.com |  |
| 50 | Y | minimumPucch-PuschOffset-r19 ENUMERATED { symb0, symb1, symb2, symb4, symb8, symb16, symb32, symb64, symb128, symb256, symb512} | Redundant space between “{ symb0” | MIMO | andrew.lappalainen@nokia.com |  |
| 51 | Y | nrofReportedRS-UE-IBR-r19 ENUMERATED {n1, n2, n3, n4}, tci-ServCellIndex-r19 ServCellIndex OPTIONAL, -- Need R | tci-ServCellIndex-r19 should be on a new line | MIMO | andrew.lappalainen@nokia.com |  |
| 52 | N | minimumPucch-PuschOffset Indicates the time offset in number of symbols for determining available transmission occasion of PUSCH in Mode-B from the PUCCH. Value symb0 corresponds to 0, value symb1 corresponds to 1 and so on. | To align with notation used elsewhere, “Mode-B” should be “mode-B”. Besides, it would be clearer to say: “…occasion of PUSCH from the PUCCH for mode-B UE initiated CSI reporting”. | MIMO | andrew.lappalainen@nokia.com |  |
| 53 | Y | additionalOneSlotOffsetDoppler-r19 CHOICE { resourceGroup4 BIT STRING(SIZE(4)), resourceGroup8 BIT STRING(SIZE(8)), resourceGroup12 BIT STRING(SIZE(12)) } OPTIONAL -- Need R | Fields are missing -r19 suffix; should be resourceGroup4-r19 resourceGroup8-r19 resourceGroup12-r19 | MIMO | andrew.lappalainen@nokia.com |  |
| 54 | N | tag2 This field is used to indicate the second TAG information for the serving cell, it is optionally configured in a serving cell if coresetPoolIndex for a BWP is configured with more than one value or if twoTA-Without-MultiDCI-MultiTRP is configured for a BWP. | These are different sentences. Should be “… for serving cell. It is optionally…” (Note: this also needs correcting in Rel-18 of TS 38.331.) | MIMO | andrew.lappalainen@nokia.com |  |
| 55 | N | fourPortSRS-3Tx Indicates whether port 1003 is disabled for all SRS resources in the SRS resource set with usage set to codebook or antennaSwitching, or whether 3Tx transmission is enabled for a configured SRS resource set with usage set to nonCodebook. This field can only be configured if nrofSRS-Ports for each SRS resources in the SRS resource set is set to ports4. | “SRS resource” should be singular, not plural | MIMO | andrew.lappalainen@nokia.com |  |
| 56 | N | srs-PortGrouping If configured, it indicates that SRS port grouping is enabled. This field can be configured only if reportQuantity is set to cri-RI-CQI and the usage of the SRS resource set is set to antennaSwitching. | “If configured” is redundant. Just say “Indicates that SRS port grouping is enabled.” | MIMO | andrew.lappalainen@nokia.com |  |
| 57 | N | pathlossOffset Indicates the pathloss offset applied to the UL only TCI or joint TCI state. Value dB-12 corresponds to -12 dB, dB-8 corresponds to -8 dB and so on. | “dB-12” and “dB-8” should be in italics | MIMO | andrew.lappalainen@nokia.com |  |
| 58 | N | QoS-FlowIdentity field descriptions. qfi Identifier of the QoS flow for which bit rate query or bit rate control is enabled.. pdu-SessionID Identifier of the PDU session to which the QoS flow idenfitied by the field qfi belongs.. | A typo: “idenfitied ” -> “identified ” | XR | eswar.vutukuri@zte.com.cn |  |
| 59 | N | RLC-Config field descriptions. .... t-RxDiscard Timer for the AMD RLC PDU(s) discard at the Rx side of the RLC entity, see TS 38.322 [4]. For the value of the IE T-RxDiscard, value ms10 means 10 milliseconds, value ms20 means 20 milliseconds, and so on. The value of the field should not be lower than that configured by the field t-Reassembly or t-ReassemblyExt.. | Correction: A typo: “discard ” -> “discarding” | XR | eswar.vutukuri@zte.com.cn |  |
| 60 |  | predictionConfiguration-r19 CHOICE { [RIL]: N021 AIML, [RIL]: H003 AIML, [RIL]: H008 AIML csi-InferencePrediction-r19 ENUMERATED {true}, [RIL]: N022 AIML, [RIL]: N023 AIML configurationForChannelPrediction-r19 SEQUENCE { resourcesForChannelPrediction-r19 CSI-ResourceConfigId OPTIONAL, -- Need R associatedIdForChannelPrediction-r19 AssociatedId-r19 OPTIONAL, -- Need R associatedIdForChannelMeasurement-r19 AssociatedId-r19 OPTIONAL, -- Need R nrofReportedPredicted-RS-r19 ENUMERATED {n1, n2, n3, n4} OPTIONAL, -- Need R nrofTimeInstance-r19 ENUMERATED {n1, n2, n4, n8} OPTIONAL, -- Need R timeGap-r19 ENUMERATED {ms10, ms20, ms40, ms80, ms160, spare3, spare2, spare1} OPTIONAL, -- Need R ... }, nrofReportedPredictedRS Indicates the number (K) of predicted RS resources to be reported per report setting, if nrofTimeInstance is not configured. Indicates the number (K) of predicted RS resources per time instance to be reported per report setting, if nrofTimeInstance is configured. This field is present only if reportQuantity-r19 is set to 'p-CRI-r19', 'p-SSB-Index’-r19, 'p-CRI-RSRP-r19' or 'p-SSB-Index-RSRP-r19'. | Parameter name in ASN.1 code is incorrect and misaligned from the name in the field description. It should be modified as: nrofReportedPredicted-RS-r19 nrofReportedPredictedRS-r19 | AIML | dawid.koziol@huawei.com |  |
| 61 | Y | maxNrofOD-SIB1-r19 INTEGER ::= 64 -- Max number of OD-SIB1 configurations maxPCI-OD-SIB1-r19 INTEGER ::= 8 -- Max number of PCIs sharing one OD-SIB1 configuration per ARFNC | Correction 1: Indentation mismatch. Remove extra space just before “- -“ Correction 2: A typo: “ARFNC” -> “ARFCN” | NES | shrivastava@samsung.com |  |
| 62 | N | 2> else: 3> consider itself not to be configured to provide UE's preference for gap occasion cancellation ratio; 3> stop the timer T346o, if running. | It is inconsistent description with two separate bullets. Follow legacy spec and specify as follows: 2> else: 3> consider itself not to be configured to provide UE's preference for gap occasion cancellation ratio and stop the timer T346o, if running. | XR | shrivastava@samsung.com |  |
| 63 | N | 1> if the received otherConfig includes the aerial-FlightPathAvailabilityConfig: 2> consider itself to be configured to indicate the availability of flight path information in accordance with 5.7.4; | Indentation for 2> bullet is not correct (i.e. it is at the level of 3>). Need to rectify | UAV | shrivastava@samsung.com |  |
| 64 | N | 1> if configured to provide its preference for gap occasion cancellation ratio: 2> if the UE did not transmit a UEAssistanceInformation message with gapOccasionCancelRatio since it was configured to do so and if the UE has the preference for gap occasion cancellation ratio for at least one measurement gap configuration; or 2> if the UE's preference for gap occasion cancellation ratio has changed for at least one measurement gap configuration since the last transmission of the UEAssistanceInformation message with gapOccasionCancelRatio and T346o is not running: 3> start the timer T346o with the timer's value set to gapOccasionCancelRatioProhibitTimer; 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide UE's preference for gap occasion cancellation ratio. | Inconsistent description Change timer's -> timer | XR | shrivastava@samsung.com |  |
| 65 | N | remaingTimeThresholdRLC-Polling Remaining time threshold used by the PDCP entity to notify the RLC entity to trigger remaining time-based polling as specified in TS 38.323 [4]. Value for the IE RLC-AM-RemainingTimeThreshold in milliseconds.. | A typo: “remaingTimeThresholdRLC-Polling” -> “remainingTimeThresholdRLC-Polling” Only there is issue in the field description. The ASN.1 is correct. | XR | eswar.vutukuri@zte.com.cn |  |
| 66 | N | 5.3.5.18.1 An ltm-ConfigNRDC included within an RRCReconfiguration message received via SRB1 is for LTM on the SCG. It includes the MCG configuration and may include ltm-ServingCellNoSecurityChangeID. | Considering other sentences, using "an" seems more natural. | MOB | kawano.takuma@mail.sharp |  |
| 67 | N | 5.3.5.18.6 3> consider the first sk-Counter value in the ltm-SK-Counters within the VarLTM-ServingCellNoSecurityChange associated to the the field ltm-NoSecurityChangeID as the selected sk-Counter value, and update the secondary key by performing security key update procedure as specified in 5.3.5.7; | Remove one extra 'the'. | MOB | kawano.takuma@mail.sharp |  |
| 68 | N | 1> else (LTM cell switch triggered upon cell selection performed while timer T311 was running or upon the fulfilment of LTM cell switch execution conditions (as specified in clause 5.3.5.18.x): | Add ‘)’ after the sentence: 1> else (LTM … (as specified in clause 5.3.5.18.x)): | MOB | kawano.takuma@mail.sharp |  |
| 69 | N | 1> if event(s) associated with all measId(s) for an ltm-CandidateId within the LTM-ExecutionConditionList IE are fulfilled: 2> inform lower layers that an event based on L3 measurements to perform an LTM cell switch procedure is fulfilled; 2> perform the LTM cell switch procedure for the LTM candidate configuration associated to the ltm-CandidateId according to the actions specified in 5.3.5.18.6. | An incorrect style is applied. Reapply the B2 style. | MOB | kawano.takuma@mail.sharp |  |
| 70 | N | #### 5.9.3.2 Initiation The UE applies the broadcast MRB establishment procedure to start receiving an MBS session of an MBS broadcast service it is interested in. The procedure may be initiated e.g. upon start of the MBS session, upon entering a cell providing an MBS broadcast service which is not associated with any ISA and the UE is interested in, upon becoming interested in the ongoing MBS broadcast service, upon removal of the UE capability limitations inhibiting reception of the ongoing MBS broadcast service UE is interested in, upon entering the ISA(s) associated with a MBS broadcast service the UE is interested in. | Typo, should be “an” MBS broadcast service | NTN | Jakob.buthler@nokia.com |  |
| 71 | N | 3.1 Definitions … Last U2N Relay UE: A U2N Relay UE having both Uu connection to the network and PC5 connection to a child UE for serving a U2N Remote UE in case of L2 U2N Relay communication. The child UE is the U2N Remote UE in case of single-hop L2 U2N Relay communication. Candidate configuration: A configuration part of an RRCReconfiguration message associated with a candidate cell, e.g., for LTM or subsequent CPAC. A candidate configuration can be a complete candidate configuration or a delta configuration relatively to a reference configuration. Reference configuration: A configuration provided by the network to the UE that is common, within the same cell group, to a group of configured non-complete candidate configurations. MBS Radio Bearer: A radio bearer that is configured for MBS delivery. | Incorrect order of terms, should be reordered alphabetically. | MOB | kawano.takuma@mail.sharp |  |
| 72 | N | 5.8.9.1.2 Actions related to transmission of RRCReconfigurationSidelink message … 1> if the UE is operating as a L2 U2N Relay UE: 2> if the destination UE is a L2 U2N Remote UEthat requested the SFN-DFN offset in a previous RemoteUEInformationSidelink message: | Space between UE and that is needed. | SLRelay, POS | kawano.takuma@mail.sharp |  |
| 73 | N | 5.8.XX.2 NR sidelink U2N Relay UE threshold conditions A UE capable of NR sidelink U2N Relay UE as an Last U2N Relay UE operation and is not having the PC5 connection with the Candidate Child UE shall: | “an Last” should be “a Last” | SLRelay | kawano.takuma@mail.sharp |  |
| 74 | Y | – RemoteUEInformationSidelink RemoteUEInformationSidelink-v1900-IEs ::= SEQUENCE { sl-PagingInfo-RemoteUE-List-r19 SetupRelease { SL-PagingInfo-RemoteUE-List-r19} OPTIONAL, -- Need M nonCriticalExtension SEQUENCE {} OPTIONAL } | “OPTIONAL” should be “OPTIONAL”, i.e. colour is wrong | SLRelay | kawano.takuma@mail.sharp |  |
| 75 | N | refLocList Indicates a reference location associated to an SMTC configuration in smtc5list. If present, it includes the same number of entries as smtc5list. The first entry in this list corresponds to the first entry in smtc5list, the second entry corresponds to the seccond entry in smtc5list, and so on. | Typo, second should be with only one c | NTN | jakob.buthler@nokia.com |  |
| 76 | N | fullfilled | fulfilled | SONMDT | qi.tao3@zte.com.cn |  |
| 77 | N | eventId Indicates an event used for measurement logging. If this field is set to eventA1, the UE starts performing logging of measurements when the entering condition as specified in 5.5.4.2 is met and stops logging when the corresponding leaving condition as specified in 5.5.4.2 is met. If this field is set to eventA2, the UE starts performing logging of measurements when the entering condition as specified in 5.5.4.3 is met and stops logging when the corresponding leaving condition as specified in 5.5.4.3 is met. | Suggest change “met” to “fulfilled” | AIML | yanxue@cictmobile.com |  |
| 78 |  |  |  |  |  |  |
| 79 |  |  |  |  |  |  |
| 80 |  |  |  |  |  |  |
| 81 |  |  |  |  |  |  |
| 82 |  |  |  |  |  |  |
| 83 |  |  |  |  |  |  |
| 84 |  |  |  |  |  |  |
| 85 |  |  |  |  |  |  |
| 86 |  |  |  |  |  |  |
| 87 |  |  |  |  |  |  |
| 87 |  |  |  |  |  |  |
| 88 |  |  |  |  |  |  |
| 89 |  |  |  |  |  |  |
| 90 |  |  |  |  |  |  |
| 91 |  |  |  |  |  |  |
| 92 |  |  |  |  |  |  |
| 93 |  |  |  |  |  |  |
| 94 |  |  |  |  |  |  |
| 95 |  |  |  |  |  |  |
| 96 |  |  |  |  |  |  |
| 97 |  |  |  |  |  |  |
| 98 |  |  |  |  |  |  |
| 99 |  |  |  |  |  |  |
| 100 |  |  |  |  |  |  |
| 101 |  |  |  |  |  |  |
| 102 |  |  |  |  |  |  |
| 103 |  |  |  |  |  |  |
| 104 |  |  |  |  |  |  |
| 105 |  |  |  |  |  |  |
| 106 |  |  |  |  |  |  |
| 107 |  |  |  |  |  |  |
| 108 |  |  |  |  |  |  |
| 109 |  |  |  |  |  |  |
| 110 |  |  |  |  |  |  |
| 111 |  |  |  |  |  |  |
| 112 |  |  |  |  |  |  |
| 113 |  |  |  |  |  |  |
| 114 |  |  |  |  |  |  |
| 115 |  |  |  |  |  |  |
| 116 |  |  |  |  |  |  |
| 117 |  |  |  |  |  |  |
| 118 |  |  |  |  |  |  |
| 119 |  |  |  |  |  |  |
| 120 |  |  |  |  |  |  |
| 121 |  |  |  |  |  |  |
| 122 |  |  |  |  |  |  |
| 123 |  |  |  |  |  |  |
| 124 |  |  |  |  |  |  |
| 125 |  |  |  |  |  |  |
| 126 |  |  |  |  |  |  |
| 127 |  |  |  |  |  |  |
| 128 |  |  |  |  |  |  |