3GPP TSG-WG2 Meeting #132	R2-2509170

Dallas, USA, November 17 – 21, 2025                              Revision of R2-2509157

Agenda Item:	8.11.3

Source:	Ericsson

Title:	E062 - Mandatory csi-RS-ResourceSetList in CSI-ResourceConfig

Document for:	Discussion

# 1	Introduction

In this document, we discuss E062.

# 2	Discussion

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E062 | SBFD | 2 | Mandatory csi-RS-ResourceSetList in CSI-ResourceConfig | R2-25xxxxx | Hakan.l.palm@ericsson.com |  |  | ToDo |

In IE CSI-ResourceConfig, SBFD WI added cli-MeasResourceSetList-r19.

In this IE, there is already a csi-RS-ResourceSetList. This list is mandatory present, meaning that at least one list element must be configured by Nw. It is not possible to include only a cli-MeasResourceSetList-r1 without also including csi-RS-ResourceSetList.

CSI-ResourceConfig ::=      SEQUENCE {

csi-ResourceConfigId        CSI-ResourceConfigId,

csi-RS-ResourceSetList      CHOICE {

nzp-CSI-RS-SSB              SEQUENCE {

nzp-CSI-RS-ResourceSetList  SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId

OPTIONAL, -- Need R

csi-SSB-ResourceSetList     SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSetsPerConfig)) OF CSI-SSB-ResourceSetId  OPTIONAL  -- Need R

},

csi-IM-ResourceSetList      SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSetsPerConfig)) OF CSI-IM-ResourceSetId

},

bwp-Id                      BWP-Id,

resourceType                ENUMERATED { aperiodic, semiPersistent, periodic },

...,

[[

csi-SSB-ResourceSetListExt-r17      CSI-SSB-ResourceSetId                                                  OPTIONAL  -- Need R

]],

[[

cli-MeasResourceSetList-r19       CHOICE {

cli-RSSI-MeasResourceSetList-r19  SEQUENCE (SIZE (1..maxNrofCLI-RSSI-MeasResourceSetsPerConfig-r19)) OF CLI-RSSI-MeasResourceSetId-r19,

srs-RSRP-MeasResourceSetList-r19  SEQUENCE (SIZE (1..maxNrofSRS-RSRP-MeasResourceSetsPerConfig-r19)) OF SRS-RSRP-MeasResourceSetId-r19,

...

}                                                                                                          OPTIONAL -- Need R

]]

}

We understand it should be possible to include a cli-MeasResourceSetList-r19 without also including csi-RS-ResourceSetList with “irrelevant/dummy content” that UE is expected to ignore. We have some examples of this in RRC. Usually, RAN2 solves this by stating in the field description that if cli-MeasResourceSetList-r19 is included, UE ignores csi-RS-ResourceSetList. A clean solution would require introduction of a new version of CSI-ResourceConfig where the csi-RS-ResourceSetList is OPTIONAL, and would require more complicated ASN.1 changes (we have not studied the ASN.1 impacts in details). We prefer to avoid this at this late stage.

With the existing ASN.1, it is also possible on purpose to include both csi-RS-ResourceSetList and cli-MeasResourceSetList-r19 in the CSI-ResourceConfig at the same time. Then, e.g. two different report config entities can point to the same CSI resource config, with each report quantity indicating which of the resource set lists (CSI or CLI) the UE shall use. There is a benefit to keep this signalling flexibility.

There is still the case only a single report config with a CLI report quantity points to the CSI-ResourceConfig. Then, UE need to ignore the csi-RS-ResourceSetList. One could argue the spec should describe this.

Below, we describe two options to resolve the ASN.1 problem.

Option 1: Do nothing.

Above, we note that a Nw must e.g. include csi-RS-ResourceSetList but Nw can omit the nzp-CSI-RS-ResourceSetList / csi-SSB-ResourceSetList. One can assume this would not harm the UE.

Option 2: Introduce a clarification in CSI-ResourceConfig field descriptions.

CSI-ResourceConfig field descriptions

bwp-Id The DL BWP which the CSI-RS or CLI measurement resources associated with this CSI-ResourceConfig are located in (see TS 38.214 [19], clause 5.2.1.2.

cli-MeasResourceSetList List of references to SRS-RSRP measurement resource set(s) or CLI-RSSI measurement resource set(s) used for L1 SRS-RSRP measurement and reporting or L1 CLI-RSSI measurement and reporting (see TS 38.214 [19], clause 5). Value cli-RSSI-MeasResourceSetList contains up to maxNrofCLI-RSSI-MeasResourceSetsPerConfig resource sets if resourceType is 'aperiodic' and 1 otherwise. Value srs-RSRP-MeasResourceSetList contains up to maxNrofSRS-RSRP-MeasResourceSetsPerConfig resource sets if resourceType is 'aperiodic' and 1 otherwise.

csi-RS-ResourceSetList UE ignores this field if the linked CSI-ReportConfig indicates report quantities cli-RSSI or cli-SRS-RSRP.

csi-IM-ResourceSetList List of references to CSI-IM resources used for CSI measurement and reporting in a CSI-RS resource set. Contains up to maxNrofCSI-IM-ResourceSetsPerConfig resource sets if resourceType is 'aperiodic' and 1 otherwise (see TS 38.214 [19], clause 5.2.1.2).

csi-ResourceConfigId Used in CSI-ReportConfig to refer to an instance of CSI-ResourceConfig.

csi-SSB-ResourceSetList, csi-SSB-ResourceSetListExt List of references to SSB resources used for CSI measurement and reporting in a CSI-RS resource set (see TS 38.214 [19], clause 5.2.1.2). The csi-SSB-ResourceSetListExt provides additional references and can only be configured if csi-SSB-ResourceSetList is configured and groupBasedBeamReporting-v1710 is configured in the CSI-ReportConfig that indicates this CSI-ResourceConfig as resourcesForChannelMeasurement. If groupBasedBeamReporting-v1710 is configured in the IE CSI-ReportConfig that indicates this CSI-ResourceConfig as resourcesForChannelMeasurement, the network configures 2 resource sets, which may be two CSI SSB resource sets (see TS 38.214 [19], clause 5.2.1.2 and 5.2.1.4.2). In this case, in TS 38.212 [17] Table 6.3.1.1.2-8B, the first resource set is indicated by a resource set indicator set to 0 and the second resource set by a resource set indicator set to 1.

nzp-CSI-RS-ResourceSetList List of references to NZP CSI-RS resources used for beam measurement and reporting in a CSI-RS resource set. If resourceType is set to 'aperiodic', the network configures up to maxNrofNZP-CSI-RS-ResourceSetsPerConfig resource sets. If resourceType is is set to 'periodic' or 'semiPersistent' and groupBasedBeamReporting-v1710 is not configured in IE CSI-ReportConfig, the network configures 1 resource set. If resourceType is set to 'periodic' or 'semiPersistent' and groupBasedBeamReporting-v1710 is configured, the network configures 2 resource sets, which may be two NZP CSI-RS resource sets (see TS 38.214 [19], clause 5.2.1.2 and 5.2.1.4.2). In this case, in TS 38.212 [17] Table 6.3.1.1.2-8B, the first resource set is indicated by a resource set indicator set to 0 and the second resource set by a resource set indicator set to 1. If resourceType is set to 'periodic' and reportQuantity is set to 'TDCP', the network configures up to 3 resource sets, see TS 38.214 [19] clause 5.2.1.2.

resourceType Time domain behavior of resource configuration (see TS 38.214 [19], clause 5.2.1.2). It does not apply to resources provided in the csi-SSB-ResourceSetList. If the associated csi-ResourceConfigId is included in CSI-LoggedMeasurementConfig, it is set to 'periodic'.

RAN2 to select between Option 1 and Option 2 above.

# Conclusion

Proposal 1	RAN2 to select between Option 1 and Option 2 above.