3GPP TSG-RAN WG2 Meeting #131-bis	R2-2507724

Prague, Czech Republic, 13th - 17th October 2025

Agenda Item:	8.6.2

Source:	Huawei, HiSilicon

Title:	38.331 TP for LTM execution conditions

Document for: Discussion and Decision

# 1	Introduction

This document provides a TP for C-LTM execution condition configuration, following issue H150.

# 2	Proposed changes

In 6.3.2 Radio resource control information elements:

LTM-Config-r18 ::=   SEQUENCE {

-- field omitted here

ltm-ServingCellNoResetID-r18      INTEGER (1..maxNrofLTM-Configs-plus1-r18)OPTIONAL, -- Need N

-- field omitted here

...,

[[

ltm-ServingCellNoSecurityChangeID-r19   LTM-NoSecurityChangeId-r19       OPTIONAL,   -- Need N

ltm-ServingCellExecutionCondition-r19   SetupRelease {LTM-ExecutionConditionList-r19}

OPTIONAL    -- Need N

]]

}

In 7.4 UE variables:

#### –	VarLTM-ExecutionConditionList

The UE variable VarLTM-ExecutionConditionList is used to store the LTM execution conditions for MCG LTM currently used by the UE.

VarLTM-ExecutionConditionList UE variable

-- ASN1START

-- TAG-VARLTM-EXECUTIONCONDITIONLIST-START

VarLTM-ExecutionConditionList-r19 ::=     SEQUENCE {

ltm-ExecutionConditionList-r19            LTM-ExecutionConditionList-r19

}

-- TAG-VARLTM-EXECUTIONCONDITIONLIST-STOP

-- ASN1STOP

In 5.3.5.18.3 LTM candidate configuration addition/modification:

##### 5.3.5.18.1	LTM configuration

[...]

The UE shall perform the following actions based on the received LTM-Config IE:

[...]

1>	if the received LTM-Config includes the field ltm-ServingCellExecutionCondition:

2>	perform LTM cell switch execution conditions modification as specified in 5.3.5.18.1a

##### 5.3.5.18.1a	LTM cell switch execution conditions modification

The UE shall:

1>	clear VarLTM-ExecutionConditionList;

1>	request lower layers to stop any LTM cell switch conditions evaluation based on L1 measurements for all the LTM candidate configurations;

1>	stop any LTM cell switch conditions evaluation based on L3 measurements for all the LTM candidate configurations as specified in 5.3.5.18.8;

1>	if this procedure is triggered by LTM cell switch execution as specified in 5.3.5.18.6 and if ltm-ExecutionCondition is configured in the LTM-Candidate to which LTM cell switch is performed:

2>	store ltm-ExecutionCondition in VarLTM-ExecutionConditionList;

1>	if this procedure is triggered by LTM configuration as specified in 5.3.5.18.1 and if ltm-ServingCellExecutionCondition is set to setup:

2>	store ltm-ServingCellExecutionCondition in VarLTM-ExecutionConditionList;

1>	for each LTM-ExecutionCondition in VarLTM-ExecutionConditions:

2>	if l3-Conditions is included in the LTM-ExecutionCondition:

3>	perform the LTM cell switch conditions evaluation based on L3 measurements as specified in 5.3.5.18.8 according to the LTM-ExecutionCondition;

2>	else if l1-Conditions is included in the LTM-ExecutionCondition:

3>	request lower layers to initiate the LTM cell switch conditions evaluation based on L1 measurements according to the LTM-ExecutionCondition.

In 5.3.5.18.6	LTM cell switch execution:

##### 5.3.5.18.6	LTM cell switch execution

Upon the indication by lower layers that an LTM cell switch procedure is triggered, or upon performing LTM cell switch following cell selection performed while timer T311 was running, as specified in 5.3.7.3, or upon the fulfilment of LTM cell switch execution conditions, the UE shall:

[...]

1>	perform LTM cell switch execution conditions modification as specified in 5.3.5.18.1a.

In 5.3.5.18.8	LTM cell switch conditions evaluation based on L3 measurements:

##### 5.3.5.18.8	LTM cell switch conditions evaluation based on L3 measurements

The UE shall:

1>	for each entry in VarLTM-ExecutionConditionList which has the l3-Conditions configured:

2>	for each measId indicated in the l3-Conditions which has a corresponding measId in the VarMeasConfig associated with the MCG measConfig:

3>	if the condEventId related to this measId is associated with condEventA3 or condEventA5, and if the entry condition applicable for this event is fulfilled for the ltm-CandidatePCI related to the ltm-CandidateId for all measurements after layer 3 filtering taken during the corresponding timeToTrigger defined for this event:

4>	consider the event associated to this measId to be fulfilled for the ltm-CandidateId associated to the measId;

3>	if the condEventId related to this measId is associated with condEventA3 or condEventA5, and if the leaving condition applicable for this event is fulfilled for the ltm-CandidatePCI related to the ltm-CandidateId for all measurements after layer 3 filtering taken during the corresponding timeToTrigger defined for this event:

4>	consider the event associated to this measId to be not fulfilled for the ltm-CandidateId associated to the measId;

1>	if event(s) associated with all measId(s) for an ltm-CandidateId within the LTM-ExecutionConditionList IE are fulfilled:

2> inform lower layers that an event based on L3 measurements to perform an LTM cell switch procedure is fulfilled;

2>	perform the LTM cell switch procedure for the LTM candidate configuration associated to the ltm-CandidateId according to the actions specified in 5.3.5.18.6.