3GPP TSG-RAN WG2 Meeting #131bis	R2-2507744

Prague, Czech Republic, Oct. 13th-17th 2025

Title:	Reply LS on Broadcasting Information on Disaster Condition of a PLMN from E-UTRAN in Case of Disaster Condition

Response to:	 R2-2506707 (C1-255678)

Release:	Release 19

Work Item:	MINT_Ph2

Source:	TSG RAN WG2

To:	CT1

Cc:	SA1, SA2

Contact Person:

Name:	SungHoon Jung

E-mail Address:	sunghoon.jung@lge.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

RAN2 would like to thank CT1 for the LS.

To support MINT in EPS, RAN2 agreed on the following:

- 1) For broadcasting disaster roaming information, disaster roaming information for MINT in EPS is introduced in existing SIB30 that is used for broadcasting disaster roaming information for MINT in 5GS.

- 2) For access control on disaster roaming access attempts, separate access barring parameters based on barring factor and barring time are introduced. Then, any disaster roaming access attempt in EPS as indicated by upper layers only applies the disaster roaming access barring parameters but not other existing access control mechanisms defined for LTE.

RAN2 understands that during access control for disaster roaming access in 5GS, Access Identity 3 related access control only applies and access category related access control is bypassed. Therefore RAN2 made similar changes to the EPS case. As a consequence, RAN2 understands that disaster roaming access for emergency call is not treated differently from other disaster roaming access for non-emergency call in terms of access control. RAN2 would like to ask if this is in line with the intention of access control toward disaster roaming access.

2. Actions:

To CT1 group.

ACTION: 	RAN2 respectfully asks CT1 to take the RAN2 agreements into account and provide feedback, if any, on access control on disaster roaming access for emergency call.

3. Date of Next TSG-RAN WG2 Meetings:

TSG RAN WG2 Meeting #132	November 17 – 21, 2025	Dallas, US

TSG RAN WG2 Meeting #133	February 9 – 13, 2026	Gothenburg, SE