3GPP TSG-RAN WG2 Meeting #131bis	R2-2507924

# Prague, Czech Republic, Oct 13th - 17th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.306 | CR | 1934 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the cyclic shift capability for PMCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_terr_bcast_Ph2-Core |  |  |  |  |  | Date: |  |  | 2025-10-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In RAN1#122, the below agreement is reached specifying a third value alpha3 for the cyclic shift for PMCH: Agreements: The input to the cyclic shift is the block of bits b0, …, bMbit-1 defined in clause 6.3.1 and the ouput is a block of bits b0, …, bMbit-1. The block of bits b0, …, bMbit-1 shall be cyclically shifted by Xi bits to form the block b0, …, bMbit-1 according to bn=b(n-Xi mod Mbit) where - Xi is given by Xi=r=C-SiC-1Er Si=iα mod C - i∈0, 1, …, N-1 denotes the ith subframe to which the transport block is mapped - N is the number of subframes to which the transport block is mapped - Er is the number of bits in the rth codeblock within a subframe as defined in TS 36.212 [3] - C is the number of codeblocks in the transport block - α is given by - if the higher-layer parameter pmch-CyclicShiftAlpha is set to alpha1 α=1 - if the higher-layer parameter pmch-CyclicShiftAlpha is set to alpha2 α=CNL - if the higher-layer parameter pmch-CyclicShiftAlpha is set to alpha3 Xi= Ainf ,nSF×floor(NSC ×QmN), Ainf ,nSF ∈{0,…N-1} where Ainf ,nSF= m=07c8nf mod 12810+nSF+m2mmod N where nf is the system frame number and nSF is subframe number within the frame. The pseudo-random sequence ci is defined by clause 7.2 and shall be initialized with cinit=NIDMBSFN at the beginning of each radio frame for which nf mod 128=0, where NSC is the number of subcarriers available in 1 OFDM symbol for PMCH, NSC =Mbit/(QmL), Qm is the modulation order. where L is the number of OFDM symbols used for the PMCH transmission. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a third value as following for the capability of cyclic shift for PMCH: pmch-cyclicShiftAlpha3-r19 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The capability of cyclic shift for PMCH is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.17.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.331 CR 5168 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

4.3.17	MBMS parameters

4.3.17.x	pmch-cyclicShiftAlpha3-r19

This parameter defines, for the corresponding E-UTRA band, whether the UE supports cyclic shift of PMCH with alpha3 for the bit sequence in clause 6.3.1 of TS 36.211 [17] for the ith subframe of the time-interleaved TB by X_i bits. A UE which supports this feature shall also support timeInterleaving-r19 as specified in TS 36.331 [5].

End of Change