3GPP RAN WG2 Meeting #131bis	R2-2507925

Prague, Czech Republic, October 13th – 17th 2025

Agenda Item:	8.1.2.2

Source:	InterDigital

Title:	Offline discussion summary on AIML RILs [X003] and [O301]/[S045]/[N114]

Document for:	Discussion, Decision

# Introduction

From online discussion, the following RILs could not be concluded:

[X003] Redundant indication of dataCollectionStart – [Proposed Status: ToDo]

R2-2507652   [X003][O301/S045/N114][X004][E040]Discussion on open issues of UE data collection          Xiaomi  discussion

- Proposal 1: [X003] is marked as PropAgree. TP in section 4.1 is adopted.

R2-2507476   Discussion on UE-sided data collection-related RILs: [E040], [X003], [X004]      InterDigital        discussion         Rel-19   NR_AIML_air-Core

- Proposal 2: [X003] RAN2 understands UE may send a start indication alone to request the dataCollectionCandidateconfiglist (if not already provided). UE does not re-send start indication when subsequently indicating the preferred configuration.

[O301]/[S045]/[N114] Missing purpose for UE-side data collection request – [Proposed Status: ToDo]

R2-2506764   [O301] Missing purpose for UE side data collection request   OPPO  discussion         Rel-19  NR_AIML_air-Core

- Proposal 1: [O301] RAN2 to discuss which solution below is preferred to address the issue:

  - Solution 1: Along with ‘start’ indicator, UE should also indicate the data collection use case type, e.g. {BM, CSI prediction, spare 2, spare 1}.

  - Solution 2: ‘start’ indicator is signaled per use case.

R2-2507090   Remaining issues in LCM for BM use case Samsung   discussion         Rel-19  NR_AIML_air-Core

- Proposal 6: (S045) the start indication (i.e., dataCollectionStart-r19) is introduced per serving cell, and the TP proposed in S045 is adopted.

This contribution sumamrizes offline discussion on those topics, and provides a set of proposals to resolve this.

# Remaining Issues marked ‘ToDo’

Based on initial discussions, the following three options have been considered for [X003]:

1) Start indication can be sent by itself as a request for data collection configuration. UE can send this request if there is no candidate list/data collection configuration [Huawei, Apple, OPPO, QC, Samsung]

- If start indication is included, then the use case may need to be included so network can provide suitable configuration [LG, Xiaomi]

- There is no way to prevent UE sending this frequently as we agreed to not have prohibit mechanism [ZTE]

- This is the agreed behaviour from RAN2#131 [Apple]

- This is only a 1-bit indication, so the request can save overhead compared to network always needing to provide the candidate list  [Apple, QC]

2) Start indication is not supported. UE requests data collection configuration implicitly via indication of a preferred data collection configuration, and NW is always expected to provide configuration if UE is capable and supported by NW [Xiaomi, ZTE, LG, Nokia, Ericsson]

- If NW doesn't provide indication there is no way to indicate UE wants to start data collection and the system could be broken [Apple]

- If start indication is removed, then NW always has to provide the candidate configuration regardless of whether the UE wants to start data collection or not which increases overhead [OPPO, Apple, QC]

- This would revert a RAN2 agreement from RAN2#131 [Apple]

- No need to differentiate per use case since start indication removed [Xiaomi]

3) Don't do anything, and leave interpretation to NW implementation [ZTE]

Based on the currently agreed behaviour from RAN2#131 Option 1 is considered the default, that the like the current system is not fundamentally broken. However, there do seem to be some missing pieces related to whether the use case needs to be provided with the request, UE conditions to send the request, and whether the request is per cell which may need further clarification. Subsequent discussion used the following proposal as a starting point:

Proposal [X003]: Change the name of the “start” indication to a “request” indication. The condition of sending this “request” indication (if it is configured by NW to do so) is the UE has no candidate list and the UE has no actual UE-side data collection configuration. Update the procedural text in RRC to reflect these conditions.

For the following enhancements related to [O301]/[S045]/[N114]:

1) including a use case with the start/request indication?

- Support: Xiaomi, LG, OPPO, vivo

- Don’t support: Samsung, ZTE, Apple, Ericsson, QC, ASUSTek

2) Making the start/request indication per cell?

- Support: Samsung, OPPO, vivo

- Don’t support: Ericsson, QC

- Neutral: Xiaomi, LG, ZTE, Apple, ASUSTek

Summary: There is some interest to do this and technical merit, however there is general agreement these enhancements can be considered in Release 20. Therefore, the following is proposed:

Proposal [O301]/[S045]/[N114]: Enhancing the “request” indication to be per-use case and/or per-serving cell will not be pursued in Release 19.

# Conclusion

In this contribution the following proposals are made regarding remaining open issues identified during the AIML ASN.1 review:

Proposal [X003]: Change the name of the “start” indication to a “request” indication. The condition of sending this “request” indication (if it is configured by NW to do so) is the UE has no candidate list and the UE has no actual UE-side data collection configuration. Update the procedural text in RRC to reflect these conditions.

Proposal [O301]/[S045]/[N114]: Enhancing the “request” indication to be per-use case and/or per-serving cell will not be pursued in Release 19.

# References

RP-243244 – WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface – Qualcomm

AIML Comments file v029 – Rel-19 CR ASN.1 Review (2025-09)

AIML Review file v029  – Rel-19 CR ASN.1 Review (2025-09)