3GPP TSG-RAN WG2 Meeting #132	R2-2509330

Dallas, USA, Nov. 17th – 21st, 2025

Agenda Item:	8.5.2

Source: 	Fujitsu

Title:	Report of [AT132][xxx][NES] Corrections to SIB1 request (Fujitsu)

Document for:	Discussion and Decision

# Introduction

This is summary report of the following offline:

R2-2509130 Corrections to SIB1 request [F001] [F002] Fujitsu discussion Rel-19 Netw_Energy_NR_enh-Core Intention is agreed. How to implement will be further discussed and TP in R2-2509330 will comeback in CB session (Fujitsu).

# Discussions

## 2.1 [F001]

RIL F001 indicates that one of the criteria to select SUL is not captured in the current SIB1 request procedure. In the legacy RRC, this criterion for SUL is defined in clause 5.2.2.4.2 of RRC specification [2], i.e., the actions upon reception of SIB1, as below:

4> if supplementaryUplink is present in servingCellConfigCommon; and 4> if the UE supports one or more of the frequency bands indicated in the frequencyBandList for the supplementaryUplink; and 4> if the UE supports at least one additionalSpectrumEmission in the nr-NS-PmaxList for a supported supplementary uplink band; and 4> if the UE is neither a RedCap nor an eRedCap UE, or for TDD if the UE is an (e)RedCap UE, or for FDD if the UE is an (e)RedCap UE and halfDuplexRedCapAllowed is present, or if the UE is an (e)RedCap UE and the (e)RedCap UE supports full-duplex FDD operation on the frequency bands indicated in the frequencyBandList for the supplementaryUplink; and 4> if the UE supports an uplink channel bandwidth with a maximum transmission bandwidth configuration (see TS 38.101-1 [15] and TS 38.101-2 [39]) which - is smaller than or equal to the carrierBandwidth (indicated in supplementaryUplink for the SCS of the initial uplink BWP), and which - is wider than or equal to the bandwidth of the initial uplink BWP of the SUL: 5> consider supplementary uplink as configured in the serving cell; 5> select the first frequency band in the frequencyBandList for the supplementaryUplink which the UE supports and for which the UE supports at least one of the additionalSpectrumEmission values in nr-NS-PmaxList, if present, and for (e)RedCap UEs in FDD, if the halfDuplexRedCapAllowed is not present, for which the UE supports full-duplex FDD operation;

As highlighted yellow, some conditions are defined whether the UE considers the cell is configured with SUL or not. If these conditions are skipped before OD-SIB1 request, the UE may transmit SIB1 request to the non-suitable cell, i.e., the UE does not satisfy the spectrum emission requirements for the cell or the uplink channel bandwidth of the cell is not supported by the UE. However, as the same resources are used for all type of UEs (including (e)RedCap UEs) for SIB1 request. Therefore, (e)RedCap related conditions are not needed

Observation 1: 	the UE needs to check the UL availability of OD-SIB1-Config in SIB26 before SIB1 request transmission via SUL as in SIB1 reception, except for (e)RedCap UE related conditions.

In addition, during the online session, several companies pointed out the same principle should be applied to normal uplink (NUL) as well. Because this is the common procedure between NUL and SUL.

Observation 2: 	the UE needs to check the UL availability of OD-SIB1-Config in SIB26 before SIB1 request transmission via NUL as in SIB1 reception, except for (e)RedCap UE related conditions.

According to the observations, rapporteur provides the TP below.

## 2.2 Proposed TP

To address the issues, the following three TPs in clause 5.2.2.3.3b for both NUL and SUL is required in RRC spec [2].

TP start

##### 5.2.2.3.3b	Request for on-demand SIB1

The UE shall, while SDT procedure is not ongoing:

- if od-SIB1-Config for this cell in stored valid version of SIB26 includes sib1-RequestConfig, and

1> if the UE supports one or more of the frequency bands indicated in the frequencyBandList in od-SIB1-Config of this cell for TDD, or one or more of the frequency bands indicated in the ul-frequencyBandList in sib1-RequestConfig of this cell for FDD, and they are not downlink only bands, and

1> if the UE supports at least one additionalSpectrumEmission in the nr-NS-PmaxList for a supported band in the frequencyBandList in the od-SIB1-Config for TDD, or a supported band ul-frequencyBandList in the sib1-RequestConfig for FDD, and

1> if the UE supports an uplink channel bandwidth with a maximum transmission bandwidth configuration (see TS 38.101-1 [15], TS 38.101-2 [39], and TS 38.101-5 [75]) which

-	is smaller than or equal to the carrierBandwidth in the sib1-RequestConfig, and which

-	is wider than or equal to the bandwidth indicated by locationAndBandwidth in the sib1-RequestConfig:

2> if od-SIB1-Config for this cell in stored valid version of SIB26 includes sib1-RequestConfigSUL; and

2>	if the UE supports one or more of the frequency bands indicated by the ul-frequencyBandList in the sib1-RequestConfigSUL; and

2> if the UE supports at least one additionalSpectrumEmission in the nr-NS-PmaxList for a supported supplementary uplink band; and

2> if the UE supports an uplink channel bandwidth with a maximum transmission bandwidth configuration (see TS 38.101-1 [15] and TS 38.101-2 [39]) which

- is smaller than or equal to the carrierBandwidth in the sib1-RequestConfigSUL and which

- is wider than or equal to the bandwidth indicated by locationAndBandwidth in the sib1-RequestConfigSUL:

3>	consider supplementary uplink as configured for this cell for SIB1 request.

1> else:

2>	consider the cell as barred in accordance with TS 38.304 [20]; and

2> perform cell re-selection to other cells on the same frequency as the barred cell as if intraFreqReselection is set to notAllowed as specified in TS 38.304 [20];

2> the procedure ends.

1>	if od-SIB1-Config for this cell in stored valid version of SIB26 includes sib1-RequestConfig and criteria to select normal uplink as defined in TS 38.321[3], clause 5.1.1 is met:

2>	trigger the lower layer to initiate the Random Access procedure on normal uplink in accordance with TS 38.321 [3] using the PRACH preamble(s) and PRACH resource(s) in sib1-RequestConfig included in od-sib1-Config for this cell in stored valid version of SIB26;

3>	if indication that maximum number of PRACH attempts as configured in sib1-RequestConfig is reached is received from lower layers as defined in TS 38.321 [3]:

4>	perform the actions as specified in clause 5.2.2.5.

3>	if acknowledgement for SIB1 request is received from lower layers:

4>	acquire the requested SIB1 message as defined in as specified in TS 38.213 [13], clause 23, immediately;

4>	upon acquiring SIB1, perform the actions specified in clause 5.2.2.4.2;

1>	if od-SIB1-Config for this cell in stored valid version of SIB26 includes sib1-RequestConfigSUL and criteria to select supplementary uplink as defined in TS 38.321[3], clause 5.1.1 is met:

2>	trigger the lower layer to initiate the Random Access procedure on supplementary uplink in accordance with TS 38.321 [3] using the PRACH preamble(s) and PRACH resource(s) in sib1-RequestConfigSUL included in od-sib1-Config for this cell in stored valid version of SIB26;

3>	if indication that maximum number of PRACH attempts as configured in sib1-RequestConfigSUL is reached is received from lower layers as defined in TS 38.321 [3]:

4>	perform the actions as specified in clause 5.2.2.5.

3>	if acknowledgement for SIB1 request is received from lower layers:

4>	acquire the requested SIB1 message as defined in as specified in TS 38.213 [13], clause 23, immediately;

4>	upon acquiring SIB1, perform the actions specified in clause 5.2.2.4.2;

1>	if cell reselection occurs while waiting for the acknowledgment for SIB1 request from lower layers:

2>	reset MAC;

1>	if the UE is unable to acquire the SIB1:

2>	perform the actions as specified in clause 5.2.2.5.

TP end

Q. Do you agree with the TP above (Yes/No)? Please provide comments and proposed changes if not.

| Company | Yes/No | Comments and proposed changes |
| --- | --- | --- |
| Samsung | Yes | The last “1>else” should be changed as shown below to have same procedure as specified for SIB1 processing. Also if the else part is executed, subsequent procedure in this section 5.2.2.3.3b needs to be skipped. 1> else: 2> consider the cell as barred in accordance with TS 38.304 [20]; and 2> perform cell re-selection to other cells on the same frequency as the barred cell as if intraFreqReselection is set to notAllowed as specified in TS 38.304 [20]; 2> the procedure ends. [Rapp] Revised in v07 |
| OPPO | Not sure whether this is a good idea to have such big normative change, but if we do this, the comment on text is provided | Besides the issue as pointed by Samsung, some additional change as indicated in the bubble. We do not take a strong view on the change, but since this normative change is rather long and affect the initial access, it may worth detailed check, e.g., continued in a short post. E.g., there could be additional question like whether this operation should be performed upon SIB26 reception (and thus captured in 5.2.2.4.28) to avoid instant processing upon reselection to the candidate OD-SIB1 target cell. [Samsung]: Since the text also involves barring, it cannot be done at SIB26 reception. |
| Apple | Acceptable, although it is not ideal | We have some sympathy with OPPO’s comment. The TP makes a lot of changes which are almost same as the following legacy conditions of SUL configuration in Section 5.2.2.4.2: 4> if supplementaryUplink is present in servingCellConfigCommon; and 4> if the UE supports one or more of the frequency bands indicated in the frequencyBandList for the supplementaryUplink; and 4> if the UE supports at least one additionalSpectrumEmission in the nr-NS-PmaxList for a supported supplementary uplink band; and 4> if the UE is neither a RedCap nor an eRedCap UE, or for TDD if the UE is an (e)RedCap UE, or for FDD if the UE is an (e)RedCap UE and halfDuplexRedCapAllowed is present, or if the UE is an (e)RedCap UE and the (e)RedCap UE supports full-duplex FDD operation on the frequency bands indicated in the frequencyBandList for the supplementaryUplink; and 4> if the UE supports an uplink channel bandwidth with a maximum transmission bandwidth configuration (see TS 38.101-1 [15] and TS 38.101-2 [39]) which - is smaller than or equal to the carrierBandwidth (indicated in supplementaryUplink for the SCS of the initial uplink BWP), and which - is wider than or equal to the bandwidth of the initial uplink BWP of the SUL: 5> consider supplementary uplink as configured in the serving cell; We think the ideal way is to touch the above base in Section 5.2.2.4.2 to include OD-SIB1 case as well. Otherwise, in future, if any new conditions on SUL configuration, we have to make corrections in both Section 5.2.2.3.3b and Section 5.2.2.4.2 to align the correction. However, since the TP is technically correct, we can accept the TP. |
| Fujitsu | Yes |  |

Summary:

- During the offline discussion, four companies provided the comments on the draft TP. Rapporteur revised the TP based on the comments. One company proposed to add the validity check for sib1-RequestConfig at the beginning, but one company thinks it is not needed. Rapporteur don’t see any technique issue therefore it was added in v11.

- There is no objection for the draft TP, but some companies think it may need more time to check because it requires a lot of changes due to including conditions for NUL on top of SUL. However, all companies think the TP is technically correct. Therefore, rapporteur suggests it should be agreed as baseline, and the conclusion can be made in CB session.

Proposal:	The revised TP should be agreed as baseline, and the conclusion can be made in CB session.

# Conclusion

Proposal:	The revised TP should be agreed as baseline, and the conclusion can be made in CB session.

Revised TP start

##### 5.2.2.3.3b	Request for on-demand SIB1

The UE shall, while SDT procedure is not ongoing:

1> if od-SIB1-Config for this cell in stored valid version of SIB26 includes sib1-RequestConfig, and

1> if the UE supports one or more of the frequency bands indicated in the frequencyBandList in od-SIB1-Config of this cell for TDD, or one or more of the frequency bands indicated in the ul-frequencyBandList in sib1-RequestConfig of this cell for FDD, and they are not downlink only bands, and

1> if the UE supports at least one additionalSpectrumEmission in the nr-NS-PmaxList for a supported band in the frequencyBandList in the od-SIB1-Config for TDD, or a supported band ul-frequencyBandList in the sib1-RequestConfig for FDD, and

1> if the UE supports an uplink channel bandwidth with a maximum transmission bandwidth configuration (see TS 38.101-1 [15], TS 38.101-2 [39], and TS 38.101-5 [75]) which

-	is smaller than or equal to the carrierBandwidth in the sib1-RequestConfig, and which

-	is wider than or equal to the bandwidth indicated by locationAndBandwidth in the sib1-RequestConfig:

2> if od-SIB1-Config for this cell in stored valid version of SIB26 includes sib1-RequestConfigSUL; and

2>	if the UE supports one or more of the frequency bands indicated by the ul-frequencyBandList in the sib1-RequestConfigSUL; and

2> if the UE supports at least one additionalSpectrumEmission in the nr-NS-PmaxList for a supported supplementary uplink band; and

2> if the UE supports an uplink channel bandwidth with a maximum transmission bandwidth configuration (see TS 38.101-1 [15] and TS 38.101-2 [39]) which

- is smaller than or equal to the carrierBandwidth in the sib1-RequestConfigSUL and which

- is wider than or equal to the bandwidth indicated by locationAndBandwidth in the sib1-RequestConfigSUL:

3>	consider supplementary uplink as configured for this cell for SIB1 request.

1> else:

2>	consider the cell as barred in accordance with TS 38.304 [20]; and

2> perform cell re-selection to other cells on the same frequency as the barred cell as if intraFreqReselection is set to notAllowed as specified in TS 38.304 [20];

2> the procedure ends.

1>	if od-SIB1-Config for this cell in stored valid version of SIB26 includes sib1-RequestConfig and criteria to select normal uplink as defined in TS 38.321[3], clause 5.1.1 is met:

2>	trigger the lower layer to initiate the Random Access procedure on normal uplink in accordance with TS 38.321 [3] using the PRACH preamble(s) and PRACH resource(s) in sib1-RequestConfig included in od-sib1-Config for this cell in stored valid version of SIB26;

3>	if indication that maximum number of PRACH attempts as configured in sib1-RequestConfig is reached is received from lower layers as defined in TS 38.321 [3]:

4>	perform the actions as specified in clause 5.2.2.5.

3>	if acknowledgement for SIB1 request is received from lower layers:

4>	acquire the requested SIB1 message as defined in as specified in TS 38.213 [13], clause 23, immediately;

4>	upon acquiring SIB1, perform the actions specified in clause 5.2.2.4.2;

1>	if od-SIB1-Config for this cell in stored valid version of SIB26 includes sib1-RequestConfigSUL and criteria to select supplementary uplink as defined in TS 38.321[3], clause 5.1.1 is met:

2>	trigger the lower layer to initiate the Random Access procedure on supplementary uplink in accordance with TS 38.321 [3] using the PRACH preamble(s) and PRACH resource(s) in sib1-RequestConfigSUL included in od-sib1-Config for this cell in stored valid version of SIB26;

3>	if indication that maximum number of PRACH attempts as configured in sib1-RequestConfigSUL is reached is received from lower layers as defined in TS 38.321 [3]:

4>	perform the actions as specified in clause 5.2.2.5.

3>	if acknowledgement for SIB1 request is received from lower layers:

4>	acquire the requested SIB1 message as defined in as specified in TS 38.213 [13], clause 23, immediately;

4>	upon acquiring SIB1, perform the actions specified in clause 5.2.2.4.2;

1>	if cell reselection occurs while waiting for the acknowledgment for SIB1 request from lower layers:

2>	reset MAC;

1>	if the UE is unable to acquire the SIB1:

2>	perform the actions as specified in clause 5.2.2.5.

Revised TP end

# Reference

- R2-2509130

- R2-2509093