# 3GPP TSG-RAN WG2 Meeting #132															R2-2509350

# Dallas, USA, Nov. 17th - 21st, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.306 | CR | 1934 | rev | 2 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the capability of LTE-based 5G Broadcast |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Samsung, Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_terr_bcast_Ph2-Core |  |  |  |  |  | Date: |  |  | 2025-11-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - In RAN1#122bis, an updated RAN1 feature list has been approved and sent to RAN2 in R1-2507982, where there are some updates related to the capability of LTE-based 5G Broadcast. This needs to be captured in TS 36.306. - There are some editorials in the descriptions of LTE-based 5G Broadcast capability. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the description of the capability related to time interleaving by adding “up to a maximum TBS”. - Introduce a new capability as following related to the cyclic shift for PMCH: pmch-CyclicShiftAlpha3-r19 - Editorial changes |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The capability of LTE-based 5G Broadcast is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.17.13, 4.3.17.14, 4.3.17.15, 4.3.17.16 (new), 4.3.17.17 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.331 CR 5168 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

4.3.17	MBMS parameters

4.3.17.13	timeInterleavingKhz15-r19, timeInterleavingKhz7dot5-r19, timeInterleavingKhz2dot5-r19, timeInterleavingKhz1dot25-r19

This parameter defines, for the corresponding E-UTRA band, whether the UE supports MBMS reception from an MBMS-dedicated cell configured with time interleaving. With time interleaving, one TB is mapped to N non-consecutive subframes and two transmissions of the same TB are separated by (M-1) subframes. A UE that supports this feature shall support the following components:

1)	PMCH transmission pattern, excluding MCCH and MSI, with time interleaving for the corresponding PMCH numerology

2)	TBS determination for the scaled TB up to a maximum TBS;

3)	determining the starting point for reading from the circular buffer (k0) for each subframe;

4)	extended MSI periodicities.

A UE which supports this feature shall also support fembmsDedicatedCell-r14 as specified in TS 36.331 [5].

4.3.17.14	pmch-CyclicShiftAlpha1-r19

This parameter defines, for the corresponding E-UTRA band, whether the UE supports cyclic shift of PMCH with alpha1 for the bit sequence in clause 6.3.1 of TS 36.211 [17] for the ith subframe of the time-interleaved TB by X_i bits. A UE which supports this feature shall also support timeInterleaving-r19 as specified in TS 36.331 [5].

4.3.17.15	pmch-CyclicShiftAlpha2-r19

This parameter defines, for the corresponding E-UTRA band, whether the UE supports cyclic shift of PMCH with alpha2 for the bit sequence in clause 6.3.1 of TS 36.211 [17] for the ith subframe of the time-interleaved TB by X_i bits. A UE which supports this feature shall also support timeInterleaving-r19 as specified in TS 36.331 [5].

4.3.17.16	pmch-CyclicShiftAlpha3-r19

This parameter defines, for the corresponding E-UTRA band, whether the UE supports cyclic shift of PMCH with alpha3 for the bit sequence in clause 6.3.1 of TS 36.211 [17] for the ith subframe of the time-interleaved TB by X_i bits. A UE which supports this feature shall also support timeInterleaving-r19 as specified in TS 36.331 [5].

4.3.17.17	freqInterleavingKhz15-r19, freqInterleavingKhz7dot5-r19, freqInterleavingKhz2dot5-r19, freqInterleavingKhz1dot25-r19

This parameter defines, for the corresponding E-UTRA band, whether the UE supports MBMS reception from an MBMS-dedicated cell configured with frequency interleaving. A UE that supports this feature shall support frequency-interleaving for MTCH for the corresponding PMCH numerology.

A UE which supports this feature shall also support fembmsDedicatedCell-r14 as specified in TS 36.331 [5].

End of Change