3GPP TSG-RAN2 Meeting #132	R2-2509397

Dallas, USA, 17 - 21 November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.304 | CR | 0452 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Low mobility criterion for LP-WUS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LPWUS-Core |  |  |  |  |  | Date: |  |  | 2025-11-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | When the entry condition for Rel-19 further RRM relaxation is met, then the UE may use a scaling factor 16 for intra-frequency measurements, see KLPW in TS 38.133. This means that with a DRX cycle of 1.28 seconds the UE is not required to measure neighbour cells for 20 seconds. The latency to detect and evaluate the measurements adds further delay to the cell reselection. This may impact the cell reselection and paging performance. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The low mobility criterion is added to the Relaxed measurement criterion for UE supporting LP-WUS in section 5.2.4.12.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The cell reselection and paging performance may be impacted for UE supporting LP-WUS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.4.12.2, 5.2.4.12.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 38.300 CR 1061 TS/TR 38.331 CR 5628 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

#### 5.2.4.12	Relaxed measurement and measurement offloading for UE supporting LP-WUS

In multi-beam operations, measured cell RX level value and measured cell quality value of the serving cell based on LR is up to UE implementation.

##### 5.2.4.12.1	Relaxed measurement rules

UE supporting LP-WUS may choose to perform relaxed serving cell and neighbouring cell measurements on MR according to requirements specified in TS 38.133 [8] if the relaxed measurement criterion in clause 5.2.4.12.2 is fulfilled.

##### 5.2.4.12.2	Relaxed measurement criterion

The relaxed measurement criterion for serving cell and neighbouring cell measurement relaxation on MR is fulfilled when:

-	Srxlev > SLP_WUS_RelaxThresholdP_MR, and,

-	Qrxlevmeas_lr > QLP_WUS_RelaxThresholdP_LR, if QLP_WUS_EntryThresholdP_LR is configured, and,

-	Squal > SLP_WUS_RelaxThresholdQ_MR, if SLP_WUS_RelaxThresholdQ_MR is configured, and,

-	Qqualmeas_lr > QLP_WUS_RelaxThresholdQ_LR, if QLP_WUS_EntryThresholdQ_LR is configured, and,

-	Low mobility criterion in clause 5.2.4.12.x is fulfilled, if lowMobilityEvaluationLPWUS is configured.

Where:

-	Srxlev = current Srxlev value of the serving cell (dB).

-	Squal = current Squal value of the serving cell (dB).

-	Qrxlevmeas_lr = current measured cell RX level value of the serving cell based on LR (RSRP).

-	Qqualmeas_lr = current measured cell quality value of the serving cell based on LR (RSRQ).

-	SLP_WUS_RelaxThresholdP_MR: The Srxlev threshold for the criterion for serving cell and neighbouring cell measurement relaxation based on MR.

-	SLP_WUS_RelaxThresholdQ_MR: The Squal threshold for the criterion for serving cell and neighbouring cell measurement relaxation based on MR.

-	QLP_WUS_RelaxThresholdP_LR: The cell RX level threshold for the criterion for serving cell and neighbouring cell measurement relaxation based on LR.

-	QLP_WUS_RelaxThresholdQ_LR: The cell quality threshold for the criterion for serving cell and neighbouring cell measurement relaxation based on LR.

These thresholds can be configured separately for LR measurements based on LP-SS and LR measurements based on SSB if a cell supports both measurement types as specified in TS 38.331 [3]. SLP_WUS_RelaxThresholdP_MR is the parameter SSearchThresholdP3 or SSearchThresholdP4  in TS 38.331 [3]. SLP_WUS_RelaxThresholdQ_MR is the parameter SSearchThresholdQ3 or SSearchThresholdQ4 in TS 38.331 [3]. QLP_WUS_RelaxThresholdP_LR is the parameter SRSRPThresholdLR or SRSRPThresholdLR2 in TS 38.331 [3]. QLP_WUS_RelaxThresholdQ_LR is the parameter SRSRQThresholdLR or SRSRQThresholdLR2 in TS 38.331 [3]. If UE supports both measurement types, it is up to UE implementation to choose LR measurements based on LP-SS or based on SSB for the determination of the relaxed measurement criterion for serving cell and neighbouring cell measurement relaxation on MR.

<End of modified section>

<Start of modified section>

##### 5.2.4.12.x	Low mobility criterion for UE supporting LP-WUS

The low mobility criterion for UE supporting LP-WUS is fulfilled when for a period of TSearchDeltaPLPWUS the condition below is fulfilled, otherwise the low mobility criterion is not fulfilled:

-	(QrxlevmeasRef_lr – Qrxlevmeas_lr) < SSearchDeltaP-LPWUS,

Where:

-	Qrxlevmeas_lr = current measured cell RX level value of the serving cell based on LR (RSRP).

-	QrxlevmeasRef_lr = reference measured cell RX level value of the serving cell based on LR (RSRP) set as follows:

-	After selecting or reselecting a new cell, or

-	When the entry condition for RRM relaxation is fulfilled and the UE chooses to start relaxed serving cell and neighbouring cell measurements on MR, or

-	When the entry condition for RRM relaxation is not fulfilled and the UE stops relaxed serving cell and neighbouring cell measurements on MR, or

-	If (Qrxlevmeas_lr - QrxlevmeasRef_lr) > 0, or

-	If the relaxed measurement criterion has not been met for TSearchDeltaP-LPWUS:

-	The UE shall set the value of QrxlevmeasRef_lr to the current measured cell RX level value of the serving cell based on LR (RSRP).

<End of modified section>