3GPP TSG RAN WG2 Meeting #131              	                	                          R2-250xxxx

Bangalore, India  Aug 25th – 29th , 2025

Title:	draft LS on candidate data collection

Release:	Release 19

Work Item:	NR_AIML_air-Core

Source:	Xiaomi, Ericsson (to be RAN2)

To:	RAN1

Cc:	RAN4

Contact person:	Ziyi Li

liziyi5@xiaomi.com

Andra Mihaela Voicu

andra.mihaela.voicu@ERICSSON.COM

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# Overall description

For AI/ML based beam management and CSI prediction UE-side data collection, before UE is being configured with CSI-ReportConfig for UE-side data collection measurement, RAN2 agreed to allow UE indicating its preferred UE-side data collection configuration(s) based on the candidate UE-side data collection configuration(s) provided by network.

RAN2 also agreed that UE is NOT expected to perform measurement(s) solely based on those candidate UE-side data collection configuration(s) and this UE behavior will be captured in RRC specification, i.e., no RAN1 specification impact.

For AI/ML based beam management, according to RAN1 agreement on UE-side data collection, for each candidate UE-side data configuration, RAN2 agreed at least the following IEs are included:

- An identifier of the candidate configuration

- CSI-ResourceConfigId for Set A

- CSI-ResourceConfigId for Set B

- Related associated IDs

RAN2 wonders if apart from the above four parameters is there any additional IEs need to be configured in the candidate data collection configuration for determining the preferred data collection configuration for beam management.

In addition, RAN2 agreed the same framework of candidate/preferred UE-side data collection configuration is also applicable to CSI prediction. However, RAN2 wonders what IEs are needed for CSI prediction as part of candidate UE-side data collection configuration.

# Action

To RAN1

ACTION:

RAN2 respectfully asks RAN1 if there’s any concern on the above agreements and provide the necessary higher layer parameters for candidate UE-side data collection configuration for CSI prediction and for AI/ML beam management if anything is missing.

# Dates of the next TSG RAN WG2 meetings

TSG RAN2 Meeting #131bis 	13th – 17th Oct, 2025	Prague, CZ

TSG RAN2 Meeting #132 	17th – 21st Nov, 2025	Dallas, US

# Annex of RAN2 agreements on candidate UE-side data collection configuration

RAN2 #129bis

- The UE can request measurement configuration for data collection of AI/ML based beam management. The request can contain one or more of the following: • An indication on start/stop of data collection • Preferred configuration from a list of candidate configurations provided by NW. Details of signaling are FFS. It is up to network what it configures at the end. - Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request.

RAN2 #130

The UE doesn’t need to measure the candidate data collection configuration(s). This will be specified in RAN2 specs. For beam management, candidate data collection configuration includes at least: - CSI-ResourceConfigId of Set A - CSI-ResourceConfigId of Set B - One/two associated IDs (up to whether Set B is equal/subset of Set A or not) according to RAN1 agreements FFS the details of how this is signalled (e.g. CSIReport config or simplified signaling)

RAN2 #131

Agreements on UE candidate data collection 1 Multiple preferred configurations within the list of candidate configurations provided by NW can be indicated by the UE via UAI. 2 No prohibit timer is needed for UE indicating its preferred data collection configuration. 3 On stop/start indication - The UE can send start indication (without a preferred list) to indicate preference to start data collection - The UE can send preferred list implying that it would like to start data collection on those configuration - The UE can send stop indication for all or a given actual CSI report config ID. - Rapporteur will determine best way of signaling. This doesn’t preclude merging 1 and 2. - Adopt below text in the field description of dataCollectionCandidateConfig: The UE is not expected to perform measurements solely based on the configurations provided by this IE. - Adopt the following solution: OtherConfig contains a list of candidate configurations as a list of a new IE, where each candidate configuration contains at least an identifier of the candidate configuration, CSI-ResourceConfigId for Set A, CSI-ResourceConfigId for Set B, and related associated IDs, as agreed in RAN2#130. Each candidate configuration is associated with a cell ID. We will also include individual IEs for CSI prediction case. - Ask RAN1 what IEs are needed for CSI prediction and inform them of our agreements on BM and confirm if anything else is needed.