3GPP TSG RAN WG2 Meeting #131bis	R2-2507906

Prague, Czech Republic, 13th – 17th October, 2025

# Title:	Draft LS on Security parameter in A-IoT paging

# Response to:	-

# Release:	Release 19

# Work Item:	Ambient_IoT_solutions

Source:	CMCC [to be RAN WG2]

To:	SA WG3

Cc:	CT WG1

Contact Person:

Name:	Ningyu Chen

E-mail Address:	chenningyu@chinamobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	-

# 1. Overall Description:

RAN2 thanks SA3 on security parameter in paging request message (R2-2505064/ S3-252392).

RAN2 would like to ask SA3 the following question:

Question: Whether the 128 bits security parameter has to be included in every paging message?

From RAN2 point of view, while it is feasible to include 128 bits security parameter in A-IoT paging message from signaling perspective, RAN2 has following concerns with making the 128 bits mandatory for every paging message:

- Deployment: secure indoor deployments, where it may not be needed to always keep re-authenticating a specific known device in every single-device CFA paging message if it has already completed the authentication.

- Overhead: high overhead to transmit 128 bit RANDAIOT_n in A-IoT paging message and to transmit 128 bit RANDAIOT_d and 32bit RESAIOT in D2R message. It should be noted there is no segmentation in R2D message.

- Complexity and time consuming: complexity and time consuming for devices to perform authentication calculations.

- Power consumption: power consumption overhead with authentication.

- Coverage reduction: coverage reduction due to larger payload size of the A-IoT paging message and D2R message.

RAN2 would like to clarify that RAN2 has defined different types of paging messages:

- Paging for one, multiple or all devices: A-IoT paging allows the A-IoT reader to trigger one, multiple or all A-IoT device(s) to perform A-IoT CBRA (Contention Based Random Access) or A-IoT CFA (Contention Free Access). The A-IoT paging message may include zero or one paging identifier, i.e., AIoT Device Permanent Identifier or Filtering Information as specified in TS 23.369. If a paging identifier is included, the A-IoT paging message may be addressed to a single A-IoT device or a group of A-IoT devices. If no paging identifier is included, the A-IoT paging message is addressed to all A-IoT devices.

- Subsequent paging: To support devices fail to receive the prior paging message or fail to complete access in the prior paging round, A-IoT reader may send the subsequent paging message containing the same transaction id as the previous paging message.

RAN2 also achieve the following agreements and the agreements may update according to the answer to the above question from SA3:

-	Add a 1 bit optionality bit for 128 bits security field in paging message.  For now, we state in RAN2 specification that this optionality bit is set to ‘present’ in this release according to SA3 TS 33.369.  If SA3 confirms that 128bits security field in paging message can be optional in SA3 reply LS, it shall be updated.

# 2. Actions:

# To SA3:

ACTION: RAN2 respectfully asks SA3 to take the above concerns into consideration and provide answers for the question above.

# 3. Dates of Next RAN2 Meetings:

RAN2#132				17th to 21st November 2025			Dallas, US

RAN2#133				9th to 13rd February 2026			Gothenburg, SE