# 3GPP TSG RAN WG2 Meeting #131b      	                           R2-2507768
Prague, Czech Republic, Oct. 13 – 17, 2025

Title:	LS on OD-SSB

Response to:

Release:	Release 19

Work Item:	Netw_Energy_NR_enh-Core– Release 19

Source:	RAN2

To:	RAN1

Cc:	RAN4

Contact person:	Peng Cheng

pcheng24@apple.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: None

# 1	Overall description

In RAN2#131b, RAN2 discussed the relationship between SSB-less SCell and OD-SSB Case 1 (i.e. only with OD-SSB but without Always-On SSB). The following RAN2 agreements were made:

Agreements:

1.	When AO-SSB in reference cell is present (i.e. absoluteFrequencySSB is present) and OD-SSB is not configured, the UE can obtain timing from SpCell or another SCell (i.e. SSB-less operation).

2.	When AO-SSB in reference cell is present (i.e. absoluteFrequencySSB is present) and OD-SSB is deactivated, RAN2 assume that the UE can obtain timing from SpCell or another SCell (i.e. SSB-less operation as legacy Rel-18 behavior).

3.	The field description of absoluteFrequencySSB is updated as follows:

Frequency of the SSB to be used for this serving cell. SSB related parameters (e.g. SSB index) provided for a serving cell refer to this SSB frequency unless mentioned otherwise. The CD-SSB of the PCell is always on the sync raster. Frequencies are considered to be on the sync raster if they are also identifiable with a GSCN value (see TS 38.101-1 [15] or TS 38.101-5 [75]). If the field is absent, the SSB related parameters should be absent, e.g. ssb-PositionsInBurst, ssb-periodicityServingCell and subcarrierSpacing in ServingCellConfigCommon IE. If the field is absent and od-ssb-r19 is absent in SCellConfig, or if the field is absent and od-ssb-r19 is present in SCellConfig but OD-SSB is not activated, the UE obtains timing reference from the intra-band SpCell or intra-band SCell if applicable as described in TS 38.213 [13], clause 4.1, or from the SpCell or an SCell indicated by referenceCell, or from the reference serving cell defined in TS 38.133 [14]. This is supported in case the SCell for which the UE obtains the timing reference is in the same or different frequency band as the cell (i.e. the SpCell or the SCell, respectively) from which the UE obtains the timing reference. For PCell, this field corresponds to the CD-SSB.

# 2	Actions

To RAN1

ACTION: 	RAN2 respectfully asks RAN1 to take above agreements into account and provide feedback if any concern.

# 3	Dates of next TSG RAN WG2 meetings

TSG-RAN WG2 Meeting #132		             November 17 to 21, 2025	         Dallas, US

TSG-RAN WG2 Meeting #133		             February 9 to 23, 2026	   Goteborg, Sweden