3GPP TSG RAN WG2#132	R2-2508010

Dallas, USA, 17th - 21st, November 2025

3GPP TSG-RAN WG1 Meeting #122bis	R1-2508026

Prague, Czech, October 13th – 17th, 2025

Title: 	Reply LS on the implicit indication of TRP location coordinates via "Associated ID"

Response to:	R1-2506737 (R2-2506658)

Release:	Rel-19

Work Item:	NR_AIML_air-Core

Source:	RAN1

To:	RAN2

Cc:

Contact person

Name:	Yufei Blankenship

E-mail Address:	yufei.blankenship@ericsson.com

# Attachments:	None

# 1	Overall description

RAN1 thanks RAN2 for the LS sharing the work on Associated ID for Info #7. Regarding the questions sent by RAN2, the following response is provided.

Question 1:
Does a single Associated ID correspond to the location coordinates of one specific TRP within a cell, or does it represent the location information of a group of TRPs in the same cell, each having distinct location coordinates?

Response to Question 1:

As stated in RAN1#121 agreement, an associated ID is configured per cell (e.g., NCGI-r15). It is provided for a group of TRP(s) in the same cell, not for one specific TRP within a cell.

Note: A group of TRP(s) in the same cell can have only one TRP.

Question 2:

Is the Associated ID in any way related to the identification of the location of Antenna Reference Points (ARPs) associated with DL-PRS Resource Sets and DL-PRS Resources?

Response to Question 2:

It is up to network implementation whether the associated ID is related to the identification of the location of ARP(s).

Question 3:

Are Associated IDs unique across different cells? Can cells belonging to different Positioning Frequency Layers (PFLs) share the same Associated ID? Furthermore, if a single cell is part of multiple PFLs, should it be assigned a distinct Associated ID for each PFL?

Response to Question 3:

As captured in the RAN1#121 agreement, the associated ID is configured per cell, not per PFL. Specific to RAN2’s Question 3:

- Associated ID(s) are not required to be unique across different cells.

- As associated ID(s) are unique only per cell, sharing associated ID(s) for cells belonging to different PFLs is irrelevant.

- The associated ID is not PFL specific. The associated ID is specific only to geographical coordinates of a group of TRP(s) in a cell.

Question 4:

RAN2 observes that an Associated ID is configured "per-Cell". However, NR DL-PRS Assistance Data support also PRS-only Transmission Points (TPs), which are not associated to any specific cell (i.e., have no NCGI). Can an Associated ID also implicitly indicate the location coordinates of such PRS-only TPs?

Response to Question 4:

From RAN1’s perspective, in Rel-19, the associated ID cannot be used to implicitly indicate the consistency of location coordinates of such PRS-only TPs, as the PRS-only TPs are not associated with a cell.

# 2	Actions

To RAN2:

ACTION: 	RAN1 respectfully ask RAN2 to take the above into account in their future work.

# 3	Dates of next TSG RAN WG1 meetings

TSG RAN WG1 Meeting #123	17th Nov – 21th Nov, 2025	Dallas, US

TSG RAN WG1 Meeting #124	9th Feb – 13th Feb, 2026	Stor-Göteborg, Sweden