3GPP TSG-RAN WG3#129bis 	R3-256531

Prague, Czechia, 13-17 October 2025

Agenda item:	9.2.5. R19 Mobility enhancements

Source:	Nokia

Title:	Remaining Conditional LTM Issues

Document for:	Discussion and Decision

# 1	Introduction

This contribution describes remaining issues for Rel-19 Conditional LTM and provides a way forward to address them.

# 2	Conditional LTM Issues

## 2.1 	L3 based C-LTM candidate cell list

A remaining issue for L3 based C-LTM is on how to have the Source gNB-DU determine in which cases it is required to immediately provide to the UE a received TA value from gNB-CU via MAC CE message, and in which others it should store the TA value and provide it later together with the Cell Switch Command.

Consider the very basic scenario where there is a single L3 C-LTM cell configured at a candidate gNB-DU. In this case, the cell at Source gNB-DU is also NOT an LTM/C-LTM candidate cell.

The signaling follow would be of following format.

1.	gNB-CU issues a UE Context Setup procedure to candidate gNB-DU including LTM Indicator = C-LTM and Early Sync Information Request.

2.	gNB-CU issues a UE Context Modification procedure to Source gNB-DU including LTM indicator = C-LTM and Early Sync Candidate Cell Information List

3.	gNB-CU generates L3 execution conditions from the source toward the only candidate and includes them in RRC Reconfiguration message to the UE

4.	gNB-CU based on L3 measurements determines to trigger Early TA Acquisition

5.	gNB-CU toward the Source DU sends a CU-DU MOBILITY INITIATION REQUEST with First bit early UL Sync and second bit early DL sync

6.	Candidate gNB-DU sends to gNB-CU a DU-CU TA INFORMATION TRANSFER

7.	gNB-CU sends to Source gNB-DU a CU -DU TA INFORMATION TRANSFER

8.	Source gNB-DU sends Timing Advance Command MAC CE to the UE – issue occurs

The issue identified is that the Source gNB-DU is unable to determine whether the TA value should be send to the UE immediately or not.

The current RAN3 specification and IE structure seems to assume that the Source gNB-DU based on the C-LTM indication received as part of the UE CONTEXT MODIFICATION REQUEST message in step 2 above would lead the Source gNB-DU to provide any TA value that it had received immediately to the UE via Timing Advance Command MAC CE, since in the UE CONTEXT MODIFICATION REQUEST message from CU it understood this concerned L3 CLTM (via absence of request for L1 execution condition).

However, this is not the correct behavior. Consider a further scenario with two LTM cells at a Candidate gNB-DU(s), cell-1 which is L3 C-LTM and cell-2 which is just LTM (but not C-LTM). That is, the Source gNB-DU should signal a received TA immediately via Time Advance Command MAC CE only for cell-1 but not for cell-2.

The issue with the current specification is that at Step 2 in the sequence above the Source gNB-DU will be informed of (a) C-LTM, and (b) Early Sync Candidate Information List. However, it is not possible to differentiate which cells are C-LTM and which are not with the contents of this message.

A possible solution to resolve this issue is to introduce a new list similar to when L1-exeuction conditions are requested. This would clearly indicate to the Source gNB-DU which cells are L3 C-LTM and with that information it can signal received TA values accordingly via Time Advance Command MAC CE only for those cells.

Proposal 1: gNB-CU signals to gNB-DU a list of L3 C-LTM Candidate Cells in UE CONTEXT MODIFICATION REQUEST and UE CONTEXT SETUP REQUEST messages.

# 3	Conclusions

Proposal 1: gNB-CU signals to gNB-DU a list of L3 C-LTM Candidate Cells in UE CONTEXT MODIFICATION REQUEST and UE CONTEXT SETUP REQUEST messages.

# Annex	Text Proposal for TS 38.473

/*********************First change***********************/

### 9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| SpCell ID | M |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | reject |
| ServCellIndex | M |  | INTEGER (0..31,...) |  | YES | reject |
| <skip unchanged part> |  |  |  |  |  |  |
| LTM InformationSetup |  | 0..1 |  |  | YES | reject |
| >LTM Indicator | M |  | ENUMERATED (true, …, C-LTM) |  | - |  |
| >Reference Configuration | O |  | 9.3.1.292 |  | - |  |
| >CSI Resource Configuration | O |  | 9.3.1.330 |  | - |  |
| >Request for CSI-RS Resource Configuration for L1 measurements | O |  | ENUMERATED (true, …) |  | YES | reject |
| >Request for CSI-RS Resource Configuration for CSI acquisition | O |  | ENUMERATED (true, …) |  | YES | reject |
| >Request for L1 Execution Condition | O |  | 9.3.1.x |  | YES | reject |
| >L3 C-LTM Candidate List | O |  | 9.3.1.y |  | YES | reject |
| LTM Configuration ID Mapping List | O |  | 9.3.1.294 |  | YES | reject |
| Early Sync Information Request |  | 0..1 |  |  | YES | ignore |
| >Request for RACH Configuration | O |  | ENUMERATED (true, …) |  | - |  |
| >LTM gNB-DUs List |  | 0..1 |  | This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s). | YES | reject |
| >>LTM gNB-DUs Item IEs |  | 1..< maxnoofLTMgNBDUs> |  |  |  |  |
| >>>LTM gNB-DU ID | M |  | gNB-DU ID 9.3.1.9 |  |  |  |
| >>>LTM gNB ID | O |  | Global gNB ID 9.3.1.305 |  | YES | reject |
| Path Addition Information | O |  | 9.3.1.296 | This IE contains either the Indirect Path Addition IE or the N3C Indirect Path Addition IE. | YES | reject |
| NR A2X Services Authorized | O |  | 9.3.1.323 |  | YES | ignore |
| LTE A2X Services Authorized | O |  | 9.3.1.324 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.119 | This IE applies only if the UE is authorized for NR A2X services. | YES | ignore |
| LTE UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | LTE UE Sidelink Aggregate Maximum Bit Rate 9.3.1.118 | This IE applies only if the UE is authorized for LTE A2X services. | YES | ignore |
| DL LBT Failure Information Request | O |  | ENUMERATED (inquiry, …) |  | YES | ignore |
| Ranging and Sidelink Positioning Service Information | O |  | 9.3.1.331 | This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services. | YES | ignore |
| Non-Integer DRX Cycle | O |  | 9.3.1.344 |  | YES | ignore |
| LTM Information SN Addition |  | 0..1 |  |  | YES | reject |
| >LTM with SCG Indicator | M |  | ENUMERATED(true, …) |  | – |  |

/*********************Next change***********************/

### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SpCell ID | O |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | ignore |
| <skip unchanged part> |  |  |  |  |  |  |
| LTM Information Modify |  | 0..1 |  |  | YES | reject |
| >LTM Indicator | M |  | ENUMERATED (true, …, C-LTM) |  | - |  |
| >Reference Configuration | O |  | 9.3.1.292 |  | - |  |
| >CSI Resource Configuration | O |  | 9.3.1.330 |  | - |  |
| >Request for CSI-RS Resource Configuration for L1 measurements | O |  | ENUMERATED (true, …) |  | YES | reject |
| >Request for CSI Resource Configuration for CSI acquisition | O |  | ENUMERATED (true, …) |  | YES | reject |
| >Request for L1 Execution Condition | O |  | 9.3.1.x |  | YES | reject |
| >L3 C-LTM Candidate List | O |  | 9.3.1.y |  | YES | reject |
| LTM CFRA Resource Config List |  | 0..1 |  |  | YES | ignore |
| >LTM CFRA Resource Config Item IEs |  | 1 .. <maxnoofLTMCells> |  |  | EACH | ignore |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>LTM CFRA Resource Configuration | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. | - |  |
| >>LTM CFRA Resource Configuration for SUL | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. This IE applies for SUL carrier. | - |  |
| LTM Configuration ID Mapping List | O |  | 9.3.1.294 |  | YES | reject |
| Early Sync Information Request |  | 0..1 |  |  | YES | ignore |
| >Request for RACH Configuration | M |  | ENUMERATED (true, …) |  | - |  |
| >LTM gNB-DUs ID List |  | 1 |  | This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s). | YES | reject |
| >>LTM gNB-DUs Item IEs |  | 1..< maxnoofLTMgNBDUs> |  |  | - |  |
| >>>LTM gNB-DU ID | M |  | gNB-DU ID 9.3.1.9 |  | - |  |
| >>>LTM gNB ID | O |  | Global gNB ID 9.3.1.305 |  | YES | reject |
| Early Sync Candidate Cell Information List |  | 0..1 |  |  | YES | ignore |
| >Early Sync Candidate Cell Information Item IEs |  | 1 .. <maxnoofLTMCells> |  |  | EACH | ignore |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TCI States Configurations List | O |  | OCTET STRING | Includes the LTM-TCI-Info IE, as defined in TS 38.331 [8]. | - |  |
| >>Early UL Sync Configuration | O |  | 9.3.1.328 |  | - |  |
| >>Early UL Sync Configuration for SUL | O |  | Early UL Sync Configuration 9.3.1.328 | This IE applies for SUL carrier. | - |  |
| >>TA Assistance Information | O |  | ENUMERATED (zero, …) | The value "zero" corresponds to TA value of the cell being equal to zero. | - |  |
| >>UE Based TA Measurement Configuration | O |  | OCTET STRING | Includes the ltm-UE-MeasuredTA-ID contained in the LTM-Candidate IE, as defined in TS 38.331 [8], for the LTM candidate cell identified by the Cell ID IE. | - |  |
| >>SSB Positions In Burst | C-ifEarlyUL |  | 9.3.1.138 | This IE applies to early TA acquisition. | YES | ignore |
| >>LTM Residual TA Information List | O |  | 9.3.1.xyz | This IE indicates the TA value and the remaining TA timers of the cell. | YES | ignore |
| Early Sync Serving Cell Information |  | 0..1 |  |  | YES | ignore |
| >UE Based TA Measurement Configuration | O |  | OCTET STRING | Includes the ltm-ServingCellUE-MeasuredTA-ID contained in the LTM-Config IE, as defined in TS 38.331 [8], for the current serving cell. | - |  |
| LTM Cells To Be Released List | O |  | 9.3.1.291 |  | YES | reject |
| Path Addition Information | O |  | 9.3.1.296 |  | YES | reject |
| NR A2X Services Authorized | O |  | 9.3.1.323 |  | YES | ignore |
| LTE A2X Services Authorized | O |  | 9.3.1.324 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.119 | This IE applies only if the UE is authorized for NR A2X services. | YES | ignore |
| LTE UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | LTE UE Sidelink Aggregate Maximum Bit Rate 9.3.1.118 | This IE applies only if the UE is authorized for LTE A2X services. | YES | ignore |
| DL LBT Failure Information Request | O |  | ENUMERATED (inquiry, …) |  | YES | ignore |
| Ranging and Sidelink Positioning Service Information | O |  | 9.3.1.331 | This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services. | YES | ignore |
| Non-Integer DRX Cycle | O |  | 9.3.1.344 |  | YES | ignore |
| LTM Reset Information | O |  | 9.3.1.346 |  | YES | ignore |
| LTM Security Information | O |  | 9.3.1.XX |  | YES | reject |
| LTM Information SN Modification |  | 0..1 |  |  | YES | reject |
| >LTM with SCG Indicator | M |  | ENUMERATED(true, …) |  | – |  |

/*********************Next change***********************/

### 9.3.1.y		L3 C-LTM Candidate Cell List

This IE indicates the list of LTM candidate cells requested for generating conditional LTM L1 execution conditions.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| L3 C-LTM Candidate Cell List Item IEs |  | 1..< maxnoofLTMCells> |  |  |
| >Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured LTM allowed towards one UE, the maximum value is 8. |

/*********************End of changes***********************/