3GPP TSG-RAN WG3 Meeting #129bis	R3-256541

Prague, Czech Republic, October 13-17, 2025

Agenda Item:	10.4

Source:	ZTE Corporation

Title:	Discussion on internal functional split and interfaces

Document for:	Discussion

- Introduction

In the SID of 6G Radio [1], the objective related to RAN architecture and interface is given as below.

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2] - Overall RAN architecture aspects [RAN3, RAN2] - RAN-CN functional split, interface, protocol stack and procedures [RAN3] - RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

In this contribution, we provide the initial consideration on internal functional split and interfaces in 6G.

- Discussion

  - Split Architecture

In real deployment, there are transport networks with performance that varies from high transport latency to low transport latency. In order to cater for these various types of transport networks and realize multi-vendor operation, the RAN architecture for NR is split into two kinds of entities, i.e., the centralized unit (CU) and the distributed unit (DU). The latency-tolerant network function resides in the CU entity, and the latency-sensitive network function resides in the DU entity. In addition, the flexible HW implementations allows scalable cost effective solutions, and the split allows for coordination for performance features, load management, real-time performance optimization, and enables NFV/SDN.

During the SI phase, the possible CU-DU split options are shown as below [2].

Figure 1. Function split between centralized and distributed units

The choice of how to split functions in the 5G RAN architecture should offer good performance of services. Then, 3GPP agreed that there shall be normative work for a single HLS option (Option 2 or Option 3), and finally Option 2 for high layer RAN architecture split is selected because better performance with more high throughput and less latency restriction can be provided by Option 2 compared with Option 3.

And the overall RAN architecture which supports CU-DU split is given as below [3].

Figure 2: Overall architecture with CU-DU split in 5G

A gNB may consist of a gNB-CU and one or more gNB-DU(s). A gNB-CU and a gNB-DU is connected via F1 interface. One gNB-CU can connect to multiple gNB-DUs, and the maximum number of connected gNB-DUs is only limited by implementation. one gNB-DU is supported to connect only one gNB-CU. However, a gNB-DU can be connected to multiple gNB-CUs by appropriate implementation for resiliency. Meanwhile, one gNB-DU can support one or more cells. The internal structure of the gNB is not visible to the core network and other RAN nodes, the gNB-CU and connected gNB-DUs are only visible to other gNBs and the 5GC as a gNB.

In addition, to achieve better integration of the ng-eNB with gNB and easier network upgrading with new service nodes, the CU-DU split has been also introduced in the ng-eNB, following the same split option (i.e. Option 2) as CU-DU split for gNB. The interface between the ng-eNB-CU and ng-eNB-DU is named as W1 interface. One ng-eNB may consist of an ng-eNB-CU and ng-eNB-DUs. An ng-eNB-CU and an ng-eNB-DU is connected via W1 logical interface. One ng-eNB-DU supports one or multiple cells. One cell is supported by only one ng-eNB-DU. And The general principle of F1 interface also applies for W1 interface.

On the top of the CU-DU split architecture, in order to provide the possibility of optimizing the location of different RAN functions based on the scenario and desired performance, the gNB-CU can be separated further into CU-CP and CU-UP. The gNB-DU hosts the RLC/MAC/PHY protocols, the CU-CP hosts the control plane instance of PDCP and RRC protocols and the CU-UP hosts the user plane instance of PDCP (and SDAP) protocols. The overall RAN architecture with CU-CP and CU-UP separation is shown as below.

Figure 3. Overall architecture for sep aration of gNB-CU-CP and gNB-CU-UP

A gNB may consist of a CU-CP, multiple CU-UPs and multiple DUs. The CU-CP is connected to the DU through the F1-C interface, while the CU-UP is connected to the DU through the F1-U interface. The CU-UP is connected to the CU-CP through the E1 interface. Furthermore, one gNB-DU is connected to only one CU-CP and one CU-UP is connected to only one CU-CP. A gNB-DU or a CU-UP may be connected to multiple CU-CPs by appropriate implementation for resiliency. One gNB-DU can be connected to multiple CU-UPs under the control of the same CU-CP and one CU-UP can be connected to multiple gNB-DUs under the control of the same CU-CP.

2.1.1 RAN3 Standardization Effort for Split architecture

In 5G standardization phase, RAN3 has taken lots of effort on introducing the above split architecture and published three sets of technical specifications related to F1 interface (38.47x series), E1 interface (38.46x series for R15 and R16  and switched to 37.48x series since R17) and W1 interface (37.48x series).

For F1 interface, the SI phase starts from April 2016 to March 2017 with the TR 38.801 as the output. And the first stable set of technical specifications was achieved in Release 15.

For E1 interface, a dedicated SI phase starts from June 2017 to December 2017 with the TR 38.806 as the output. And the first stable set of technical specifications was achieved in Release 15.

For W1 interface, a dedicated phase starts from October 2017 to March 2018 with the TR 37.876 as the output. And the first stable set of technical specifications was achieved in Release 16.

| Interface | Final Solution | Output of SI Phase | Output of WI Phase |
| --- | --- | --- | --- |
| F1 | Option 2 | TR 38.801 From 2016.04 | TS 38.470/471/472/473/474/425 From 2017.04 |
| E1 | Solution 2 | TR 38.806 From 2017.06 | TS 38.460/461/462/463 From 2018.02 TS 37.480/481/482/483 From 2022.02 |
| W1 | Option 2 | TR 37.876 From 2017.10 | TS 37.480/481/482/483/425 From 2018.05 |

Table 1. Summary of RAN3 work on the different interfaces for split architecture

Based on the above summary, it takes 3 SIs and 18 Technical Specifications for RAN3 to finalize the stable progress of the split architecture. In addition, after Release 15, almost all newly introduced features of each release have standard impact on F1 or E1 interface, and it also takes lots of time for RAN3 to work on the solution and impacts towards corresponding specifications. Therefore, the support of split architecture in 5G introduced quite high complexity on RAN3’s workload.

Observation 1: Since R15, RAN3 has taken tremendous effort on the standardization of split architecture in 5G.

2.1.2 Commercial Deployment for Split architecture

However, there seems to be no successful large-scale commercial 5G deployment with this split architecture so far. Only the North America area held a fair market share on the split architecture (e.g., Open RAN or V-RAN) deployment, and the market share of split architecture seems not sizable in the rest area of the world. As a reminder, Open RAN investments accelerated rapidly in the initial phase between 2019 and 2022. Open RAN-based investments then declined since 2023,  and short-term visibility is more uncertain.[4].

In addition, the power consumption is one key factor of the network performance evaluation. Base stations and related infrastructure that are part of access networks can use up to 50% of the total energy consumed by the network segment. For the split architecture, the density of RUs and the longest distance each DU and CU can support shall affect the power consumption of network. Longer distances and higher densities are typically associated with higher energy needs. In the case of the most stringent values of maximum supported distance, the split architecture has almost 8 times greater consumption than the RAN one [5]. So, the power consumption could be a barrier for the Operators to deploy the split architecture in practical.

Observation 2: The market share of commercial deployment for split architecture is not as sizeable as expected after 5G standardization.

During the 6G standardization phase, the requirement on the support of split architecture in 6G is still not clear, and RAN3 should focus on the interface with real needs on the inter-vendor interoperability. Considering the tremendous effort on the standardization of split architecture in 5G, RAN3 should also take the complexity and time budget into account. If the split architecture is still needed in 6G, the gap analysis is required to understand the real needs from the market.

Figure 4. Overall architecture in 5G

There is another incoming LS from RAN#109 on fronthaul interface [5], RAN3 should follow the decision made in RAN plenary, and no further WG level discussion is expected.

Proposal 1: Considering the trade off of the complexity, RAN3 workload, and the profit and issues from commercial deployment, whether to support the split architecture in 6G Day1 needs to be evaluated during SI phase.

- 2.2 Interface between 6G RAN nodes

Regarding the interface between the 6G RAN nodes, RAN3 has introduced the Xn interface in NR, similar as the X2 interface in LTE, which can be considered as the start point. And the possible deployment of 6G RAN nodes can be illustrated as below.

Figure 5. Possible deployment of 6G RAN nodes

- To achieve the connection between the 6G RAN nodes, the Xn+ interface, similar as Xn interface, can be applied, especially for the Standalone scenario in the Day 1 of 6G. In details, the connection between two 6G NodeB (6NB)s or one 6NB and one evolved gNB (gNB+) or two gNB+s can be achieved by means of the Xn+ interface.

Proposal 2: The Xn+ interface, similar as Xn interface, can be introduced to achieve the connection between the RAN nodes in 6G.

In the 5G standardization phase, the general principle of Xn interface is given as below [6].

4.2 Xn interface general principles The general principles for the specification of the Xn interface are as follows: - the Xn interface is open; - the Xn interface supports the exchange of signalling information between two NG-RAN nodes, and the forwarding of PDUs to the respective tunnel endpoints; - from a logical standpoint, the Xn is a point-to-point interface between two NG-RAN nodes. A point-to-point logical interface should be feasible even in the absence of a physical direct connection between the two NG-RAN nodes.

The Xn interface has already been applied in the 5G commercial market, and there seems to be no strong requirement to introduce principle enhancement on top of the existing Xn interface. Therefore, the general principles of Xn interface should be considered as the baseline of the interface between RAN nodes in 6G (e.g., Xn+ interface).

Proposal 3: The general principles of Xn interface defined in 5G should be considered as the baseline of Xn+ interface in 6G.

- Summary

Observation 1: Since R15, RAN3 has taken tremendous effort on the standardization of split architecture in 5G.

Observation 2: The market share of commercial deployment for split architecture is not as sizeable as expected after 5G standardization.

Proposal 1: Considering the trade off of the complexity, RAN3 workload, and the profit and issues from commercial deployment, whether to support the split architecture in 6G Day1 needs to be evaluated during SI phase.

Proposal 2: The Xn+ interface, similar as Xn interface, can be introduced to achieve the connection between the RAN nodes in 6G.

Proposal 3: The general principles of Xn interface defined in 5G should be considered as the baseline of Xn+ interface in 6G.

The corresponding TP for TR 38.760-3 is given in the Annex.

4	Reference

[1] RP-251881, New SID: Study on 6G Radio, NTT DOCOMO (Moderator)

- [2] TR 38.801, v14.0.0

- [3] TS 38.401, v19.5.0

- [4] BALDINI, G., BOLLA, R., BRUSCHI, R., CARREGA, A., DAVOLI, F. et al., Toward Sustainable O-RAN Deployment: an in-depth Analysis of Power Consumption, IEEE TRANSACTIONS ON GREEN COMMUNICATIONS AND NETWORKING, 9, 2, 2025, p. 429-444, IEEE-INST ELECTRICAL ELECTRONICS ENGINEERS INC, https://data.europa.eu/doi/10.1109/TGCN.2024.3426108, JRC135185.

[5] R3-256536, LS on 6G fronthaul, TSG RAN

- [6] TS 38.420, v19.1.0

5	Annex

TP for TR 38.760-3

6.2	RAN internal interfaces

6.2.1	Interface between RAN nodes

Editor’s Note: This chapter should also address the interface between 6G RAN nodes.

Figure 6.3.1. Deployment of 6G RAN nodes

The interface allowing to interconnect two 6G NodeB (6NB)s, or one 6G NodeB and one evolved gNB (gNB+), or two evolved gNBs with each other is referred to as the Xn+ interface.

NOTE:	The name of the interface can be further discussed in normative phase.

The general principles for the specification of the Xn+ interface are as follows:

-	the Xn+ interface shall be open;

-	the Xn+ interface shall support the exchange of signalling information between the endpoints, and the forwarding of user data to the respective tunnel endpoints;

-	from a logical standpoint, the Xn+ is a point-to-point interface between the endpoints. A point-to-point logical interface should be feasible even in the absence of a physical direct connection between the endpoints.

-	the Xn+ interface shall support control plane and user plane separation;

-	the Xn+ interface shall separate Radio Network Layer and Transport Network Layer;

-	the Xn+ interface shall be future proof to fulfil different new requirements, support new services and new functions.

6.2.2	Evaluation and interface(s) for split architecture

Editor’s Note: This chapter is used to evaluate and address the CU-DU and/or CP-UP aspects considering interim milestone.

Whether to continue to support the split architecture in 6G should evaluate the following factors:

-	Commercial deployment requirements

-	Complexity and benefit

-	CAPEX and OPEX

-	Energy Consumption