3GPP TSG-RAN WG3 Meeting #129bis       	   				    R3-256583

Prague, Czech Republic, 13 – 17 October 2025

Agenda Item:		10.4

Source:			China Telecom

Title:	Views on 6G RAN internal functional split and interfaces

Document for:	 	Discussion & Decision

- Introduction

In TR 38.914[1], the basic requirements of architecture of 6G RAT are captured, including the functions to be supported by 6G RAN.

The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements:

-	The 6G RAN architecture shall support standalone RAN architecture.

-	The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR.

-	The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR.

-	The 6G RAN architecture shall support connectivity through multiple TRPs, either collocated or non-collocated.

-	The 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs.

-	3GPP defined interfaces for 6G RAN shall be open for multi-vendor interoperability.

-	The 6G RAN architecture shall allow for control plane and user plane separation.

-	The 6G RAN architecture shall support sharing of the RAN between multiple operators.

-	The 6G RAN architecture shall allow for the operation of network slicing.

-	The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network.

-	The 6G RAN architecture shall support enhanced service awareness in RAN.

-	The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable.

-	The design of the 6G RAN shall enable lower CAPEX/OPEX with respect to current networks.

-	The 6G RAN architecture shall allow non-public networks.

In this contribution, we will provide some initial consideration on 6G RAN internal functional split and interfaces from the perspective of RAN3.

- Discussion

2.1 Inter 6G NodeB Interface

The Xn interface is a logical interface defined within the 5G RAN between gNBs. The existence of Xn interface is the key to achieving high performance and reliability of 5G. Its core function is to enable direct and rapid coordination and information exchange between gNBs.

6G should support multi-node access, cooperative transmission, and flexible handover. To fulfill these requirements, a direct connection interface between 6G NodeBs is necessary (equivalent to Xn).

The general principles for the specification of the inter 6G NB interface are as follows:

-	The inter 6G NB interface shall be open for multi-vendor interoperability;

-	The inter 6GNB interface shall support control plane and user plane separation;

-	The inter 6GNB interface shall be future proof to fulfil different new requirements, support new services and new functions.

Proposal 1: RAN3 should specify the inter 6G NB interface.

2.2 6G NodeB Architecture and Internal Interfaces

According to the RAN#109 plenary meeting's way forward for 6G NB low layer split (Fronthaul)[2]:

3GPP shall capture the following text in italic in a normative Annex of TSxx.401 or TSxx.300 (RAN3):

- 6G NodeB architecture includes a distinct logical Radio Unit (RU).

- The functions hosted by the RU as well as the interface between RU and other parts of 6G NodeB are not specified in 3GPP.

Note-1: This interface can be implemented according to O-RAN ALLIANCE specifications for multi-vendor interoperability.

Note-2: This interface can be implemented according to e.g. (e)CPRI-based or fully proprietary solution(s)

Therefore, whether to support LLS is not within the scope of discussion in RAN3, and we can focus on higher layer split in the study item for 6GR to ensure scalability and flexibility and enable coordination for performance features, load management, and real-time performance optimization in cooperative communication.

RAN3 is expected to study different functional splits between central and distributed units. NR protocol stack is taken as a basis for further discussion[3], with the understanding that the conclusions may need to be revisited, once RAN2 defines the protocol stack for NR. The following functional splits between central and distributed unit are possible, as illustrated in Figure 1.

Figure.1 The high layer split architecture

The CU/DU split architecture (i.e., split in PDCP-RLC layer) can be considered as the starting point. Similarly, the CU-CP and CU-UP split function should also be discussed in RAN3, and the actual gain and network complexity should be considered during the study phase.

Proposal 2: RAN3 should support the high-layer split architecture for 6G NB. The CU/DU split architecture (i.e., split in PDCP-RLC layer) can be considered as the starting point.

Proposal 3: RAN3 should discuss the separation between the CU-CP and CU-UP.

2.3 6G NodeB External Interfaces

Except for inheriting the general requirements of architecture of 5G RAN, we need to identify the new requirements/functions/scenarios of 6G RAN. For example, AI/ML Function Node, and Sensing Function Node (SF Node) deployed in RAN may be considered in 6G network architecture.

Proposal 4: It is proposed RAN3 to study the new interfaces between 6G NB and the new NFs.

- Conclusions

In this paper, we have the following proposal:

Proposal 1: RAN3 should specify the inter 6G NB interface.

Proposal 2: RAN3 should support the high-layer split architecture for 6G NB. The CU/DU split architecture (i.e., split in PDCP-RLC layer) can be considered as the starting point.

Proposal 3: RAN3 should discuss the separation between the CU-CP and CU-UP.

Proposal 4: It is proposed RAN3 to study the new interfaces between 6G NB and the new NFs.

- References

[1] 	pCR for 38.914 on requirements for architecture and migration for 6G, V0.1.1.

[2] 	RP‑252881, Way forward on fronthaul in 6G, Nokia.

[3] 	3GPP TR 38.801, Study on new radio access technology: Radio access architecture and interfaces.

- Text Proposal for TR 38.8XX

<<<<<<<<<<<<<<<<<< Begin of the changes >>>>>>>>>>>>>>>>>>>

6.3       RAN internal functional split and interfaces

Editor’s Note: This chapter should comprehensively address the CU-DU and CP-UP aspects considering interim milestone.

Editor’s Note: This chapter should also address the interfaces between 6G NBs (equivalent to Xn)

6.3.1	XX Interface

6.3.1.1	General

The interface allowing to interconnect two 6G NBs with each other is referred to as the XX interface.

6.3.1.2	General principles

The general principles for the specification of the XX interface are as follows:

-	the XX interface shall be open for multi-vendor interoperability;

-	the XX interface shall support control plane and user plane separation;

-	the XX interface shall be future proof to fulfil different new requirements, support new services and new functions.

<<<<<<<<<<<<<<<<<< End of the changes >>>>>>>>>>>>>>>>>>>