3GPP TSG RAN WG3#129bis	R3-256586

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:	13.2

Source: 	China Telecom

Title:	[TP for 38.765] Network architecture solution for gNB-based mono-static sensing

Document for:	Discussion and decision

- Introduction

The 3GPP RAN#108 has approved a new SID, “Study on Integrated Sensing And Communication (ISAC) for NR” [1] as part of the Rel-20 5G NR advanced. The detailed focus of the study is as follows:

Evaluate the performance of gNB-based mono-static sensing (i.e., single TRP with co-located sensing transmitter and receiver) for UAV use case [RAN1] - Identify and study metrics, measurements, and relevant measurement quantization for UAV use case - As baseline, existing DL NR waveform and DL NR reference signals are to be used for evaluations. - For other waveform and reference signals, companies are to share relevant information - No UE impacts - Deployment scenario and assumptions for channel model calibration for UAV sensing targets in the Rel-19 ISAC channel model SI [FS_Sensing_NR] are used as starting point for evaluation assumptions. - FR1 frequency range is prioritized. Study the procedures, signaling between RAN and CN to support ISAC [RAN3] Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

And at RAN1#122, evaluation aspects for ISAC have been discussed and some evaluation parameters have been agreed, the SI progress reached 15%. In this meeting, RAN3 start the discussion on the network architecture, procedures and signalling for gNB-based mono-static sensing.

In this paper, we will focus on the study of a network architecture to enable gNB-based mono-static sensing and fulfil key functional and procedural requirements.

- Discussion

2.1 Architecture Options for RAN-CN interface

Based on the scope of ISAC SI, the architecture supporting sensing should be extended based on the current 5GC architecture as defined in TR 38.300 [2] and TS 23.501 [3]. The sensing architecture shall support gNB-based mono-static sensing in 5G-A Rel-20 and gNB bistatic sensing support may be considered as part of this network architecture without additional architecture impacts. The architecture does not support inter-gNB coordination.

The architecture design should fulfil the whole sensing service provision requirement, such as

- Sensing gNB and sensing function discovery and (re-)selection;

- RAN-CN sensing interface setup and update;

- Sensing service session establishment, maintenance, and release;

- Sensing data and the associated information collection and transport;

For now, there are two ISAC architectures that have become the mainstream in SA2 and been captured in TR 23.700-14 [4]. We discuss the two architectures at following sections.

Terms used within the architecture discussion:

Sensing Entity: The Sensing Entity referring to a Sensing Transmitter and/or to a Sensing Receiver.

Sensing gNB: Indicating a gNB capable of serving as a sensing transmitter and/or as a sensing receiver.

Sensing Function: Indicating the logical function which is involved to support Sensing Service in 5GC.

2.1.1 Option 1: Direct interface architecture

Figure 1. the reference direct interface architecture

In the direct interface architecture, there is a direct connection interface between the sensing entity (i.e. sensing gNB) and the SF(Sensing Function) of 5GC. For support of sensing service, the sensing gNBs need firstly to establish the interface connection with SF.

After the connection established, when the Sensing Function receives a request for subscription to sensing service from the sensing service consumer, it discovers sensing gNBs and establishes the sensing service session with them to provide the sensing service configuration.

The sensing gNB receives the sensing service configuration from SF and perform the sensing operation. Then after getting the sensing date, the sensing gNB perform the report of 3GPP Sensing Data to 5GC SF directly.

The Sensing Function could further process the data, obtain the sensing result and expose the sensing result to the sensing service consumer.

2.1.2 Option 2: Indirect interface architecture

Figure 2. the reference indirect interface architecture

In the indirect interface architecture, there is a control signalling interface between the sensing gNB and the AMF of 5GC and a date transmission interface between the sensing gNB and the SF of 5GC. Thus, the sensing gNBs could reuse the NG-C interface to establish the control signalling connection with SF.

When the sensing consumer requests the sensing service, AMF discovers sensing gNBs and establishes the sensing service session with them to provide the sensing service configuration via NG interface.

The sensing gNB receives the sensing service configuration and performs the sensing operation. After getting the sensing date, the sensing gNB performs the report of 3GPP Sensing Data to 5GC SF directly. Then, the Sensing Function could further process the data, obtain the sensing result and expose the sensing result to the sensing service consumer.

2.1.2 Two architecture comparison

In both architectures, the RAN's data transmission plane has a direct connection interface with 5GC SF. However, for the control plane, one connects directly to the SF while the other connects to the AMF. This results in significant differences in the design of sensing control signalling between the two architectures. When the interface is directly connected to the AMF, we can enhance the existing NG interface to implement sensing functionality, requiring relatively fewer modifications to the signalling. When the interface is directly connected to the SF, since it is a new interface, more new signalling needs to be introduced to achieve sensing functionality. However, by eliminating the process of AMF forwarding SF signalling, the direct SF interface architecture offers a more simple and efficient transmission process for sensing signalling.

Observation: The direct SF interface architecture offers a more simple and efficient transmission process for sensing signalling.

2.2 Skeleton of Architecture Chapter

Document draft TR 38.765 [5] outlines the overall Integrated Sensing and Communication (ISAC) Technical Report (TR). Chapters 7 ("Network Architecture") and 8 ("RAN-CN Procedures and Signalling") fall under the responsibility of RAN3 for discussion and finalization. This contribution provides the proposed sections for Chapter 7. We recommend structuring this chapter according to the outline provided in the appendix.

Proposal: It is proposed to include the text in the Annex as part of TR 38.765.

- Conclusion

This paper has discussed two potential RAN architectures able to fulfil the gNB-based sensing requirement. Based on the above discussion on the RAN sensing architecture, we provide the following observations and proposals:

Observation: The direct SF interface architecture offers a more simple and efficient transmission process for sensing signalling.

Proposal: It is proposed to include the text in the Annex as part of TR 38.765.

- Reference

- RP-252819, Updated SID: Study on Integrated Sensing And Communication (ISAC) for NR, Xiaomi, CTC.

- TS 38.300, NR and NG-RAN Overall description; Stage-2 Rel-19, 3GPP.

- TS 23.501, System architecture for the 5G System (5GS); Stage 2 Rel-19, 3GPP.

- TR 23.700-14, Study on Integrated Sensing and Communication; Stage 2 Rel-20, 3GPP.

- R3-256589, draft TR38.765 Study on Integrated Sensing And Communication (ISAC) for NR, CTC, Xiaomi.

- Annex: TP for RAN3 TR38.765

It is proposed to capture the following text proposal in TR38.765:

<<<<<<<<<<<<<<<<<<<< Change Begins >>>>>>>>>>>>>>>>>>>>

References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP RP-251861: " New SID: Study on Integrated Sensing And Communication (ISAC) for NR".

[X]           3GPP TR 23.700-14: " Study on Integrated Sensing and Communication; Stage 2".

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

Network architecture

Editor’s note: This section is to capture the study outcome of network architecture. Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. No inter-gNB coordination will be studied.

3.1	General

Editor’s note: This section is to capture the design principles of the network architecture and the basic functions supported by the architectures.

3.2	Architecture Options

Editor’s note: This section is to capture the study outcome of network architecture options.

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>