3GPP TSG-RAN WG3 Meeting #129bis	R3-256711

Prague, Czech Republic, 13th – 17th October, 2025

Agenda Item:	10.4

Source: 	OPPO

Title:  	Discussion on 6G RAN internal interfaces

Document for:	Discussion and Decision

- Introduction

In Rel-20 6G SID [1], the objectives led by RAN3 are captured as follows.

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2] - Overall RAN architecture aspects [RAN3, RAN2] - RAN-CN functional split, interface, protocol stack and procedures [RAN3] - RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

Besides, an interim milestone is set for RAN3 as shown below.

#### 4.1.1 Interim Milestone Interim results shall be delivered as per the milestones below, in coordination with the RAN Plenary 6G Study [RP-250810]. TSG#112 (June/2026): RAN1 to provide interim assessment on the following areas: - Waveform, modulation, channel coding: scope of enhancements beyond NR baseline ((2) a, c) - Channel bandwidth (min and max), frame structure, numerology ((2) b, d) - Basic sync signal structure and associated periodicity(ies) ((2) h) For objectives where RAN4 may be impacted, RAN1 shall coordinate with RAN4 early to enable the above assessment by June 2026. RAN3 to provide interim study results to allow TSGs to make a decision on: - RAN-CN interface: P2P vs SBI - RAN internal interfaces: CU-DU split, CP-UP split. RAN plenary to make a decision on additional 6G-6G aggregation beyond 6G CA: 6G-6G DC. RAN plenary will task relevant RAN WGs for any specific technical analysis, as needed. NOTE: It is planned to decide on Release-21 timeline in June/2026.

This contribution would like to provide our considerations on RAN internal interfaces, mainly covering CU-DU split.

- Discussion

During the early stage of 5G research, different potential options for CU-DU split were studied and discussed in RAN3, as illustrated in the figure below.

As captured in TR 38.801, the table below shows the characteristics of different CU-DU split options. And the pros and cons of each option have been comprehensively analyzed in RAN3.

|  | Opt. 1 | Opt. 2 | Opt. 3-2 | Opt. 3-1 | Opt. 5 | Opt. 6 | Opt. 7-3 (only for DL) | Opt. 7-2 | Opt. 7-1 |  | Opt. 8 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Baseline available | No | Yes (LTE DC) | No |  |  |  |  |  |  |  | Yes (CPRI) |  |
| Traffic aggregation | No | Yes |  |  |  |  |  |  |  |  |  |  |
| ARQ location | DU |  |  | CU May be more robust under non-ideal transport conditions |  |  |  |  |  |  |  |  |
| Resource pooling in CU | Lowest | in between (higher on the right) |  |  |  |  |  |  |  | Highest |  |  |
|  | RRC only | RRC + L2 (partial) |  |  |  | RRC + L2 | RRC + L2 + PHY (partial) |  |  | RRC + L2 + PHY |  |  |
| Transport NW latency requirement | Loose |  |  |  | NOTE 7 | Tight |  |  |  |  |  |  |
| Transport NW Peak BW requirement | N/A | Lowest | in between (higher on the right) |  |  |  |  |  |  | Highest |  |  |
|  | No UP req. | baseband bits |  |  |  |  |  | Quantized IQ (f) |  | Quant. IQ (t) |  |  |
|  | - | Scales with MIMO layers |  |  |  |  |  |  | Scales with antenna ports |  |  |  |
| Multi-cell/freq. coordination | multiple schedulers (independent per DU) |  |  |  | centralized scheduler (can be common per CU) |  |  |  |  |  |  |  |
| UL Adv. Rx | NOTE 7 |  |  |  |  |  | NA | NOTE 7 | Yes |  |  |  |
| Remarks | NOTE 4 |  |  |  | NOTE 5/6 | NOTE 5 | NOTE 5 | NOTE 5 |  |  |  |  |

In 5G, non-ideal transport conditions between CU and DU need to be considered. The main trade-off for CU-DU split is between centralization gains and hardware/transport costs. Considering the deployment flexibility, higher layer split option is easier to deployed.

Besides, the support of Dual-Connectivity architectures is of course an important factor on the selection of CU-DU split option, which require a centralized PDCP layer. Therefore, the options which does not align with DC architectures is definitely excluded, such as Option 1. And Option 2 is the only choice that aligns with all different DC architectures.

Considering the deployment flexibility and the alignment of DC, higher layer split option (i.e., Option 2 for PDCP/RLC split) is adopted in 5G.

- Higher layer CU-DU split option (i.e., Option 2 for PDCP/RLC split) is adopted in 5G RAN, considering 
1/ the trade-off between centralization gains and hardware/transport costs, and 
2/ the alignment with Dual-Connectivity architecture.

When it comes to 6G, before we start to discuss different CU-DU split options, we may firstly discuss whether to support CU-DU split in 6G RAN or not.

Similar to 5G, the support of DC architecture has impact on CU-DU split in 6G, and the study of 6G CU-DU split options needs to consider the alignment with DC architecture. However, note that the support of NR-6GR DC and/or 6G-6G DC is still pending the progress and conclusion of RAN Plenary study.

- The support of NR-6GR DC and/or 6G-6G DC is still pending the progress and conclusion of RAN Plenary-level study, which has impact on the study of 6G CU-DU split.

As mentioned above, the requirement on transport conditions between CU and DU is always a key issue for CU-DU split options. We understand that in 6G there is also no magic to change the situation, i.e., non-ideal transport conditions between CU and DU should also been considered. If non-ideal transport conditions are still there, it is still hard to support lower layer CU-DU split options in 6G.

- Non-ideal transport conditions between CU and DU has to be considered for 6G CU-DU split.

Furthermore, we observe that, in 5G, although CU-DU interface is fully within RAN node scope, the latency due to non-ideal transport conditions between CU and DU has also impact on UE and the air-interface.

- Although CU-DU interface is fully within RAN node scope, the latency due to non-ideal transport conditions between CU and DU has also impact on UE and the air-interface.

From user plane data transmission perspective, according to the current 5G user plane protocols stacks, the tight cross-layer coordination is still needed between the PDCP layer and the RLC layer.

For example, in 5G Rel-18/19 XR, in order to make the XR traffic transmission properly, as illustrated above, the PDCP layer needs to provide different indications (i.e. SDU discard indication, remaining-time-based RLC retransmission indication and remaining-time-based RLC polling indication) to RLC entity so as to allow the differentiated RLC discard/retransmission/polling behaviours (i.e. SDU discard, remaining-time-based RLC retransmission, RLC-only retransmission, remaining-time-based RLC polling and RLC-only polling).

In case of 5G CU-DU split architecture, PDCP layer is in CU and RLC layer is in DU, thus the discard indication from the CU PDCP to the DU RLC causes extra latency and signalling overheads over F1 interface.

- From user plane data transmission perspective, tight PDCP-RLC interaction between CU and DU is still needed in case of CU-DU split, which may cause extra latency and signalling overheads over F1 interface.

From control plane perspective, taking MAC CE and RRC signalling as an example, when discussing solutions between MAC CE and RRC signalling, we usually consider the latency of MAC CE is much lower than RRC signalling, since non-ideal transport conditions between CU and DU leads to additional latency for RRC layer in CU. Therefore, Layer-1/2 mobility, i.e., LTM, is introduced in 5G on top of Layer-3 mobility. In LTM, DU can directly decide to execute LTM based on L1 measurement reporting of UE, without interaction between DU and CU.

- From control plane perspective, CU-DU split leads to additional latency for RRC layer in CU.

In the last, we understand a standardized CU-DU split solution is for inter-vender CU and DU. Only in case that there is real need on inter-vender CU and DU, a standardized CU-DU split solution is necessary. Otherwise, it can leave to NW implementation.

- CU-DU split solution could be left to NW implementation in case that there is no need on inter-vender CU and DU.

Therefore, before we start to discuss different CU-DU split options in RAN3, we propose to firstly discuss whether to support CU-DU split in 6G RAN or not, taking all observations above into account.

- RAN3 discuss whether to support CU-DU split in 6G RAN or not, before studying different CU-DU split options.

3. Conclusion

Based on the discussion above, we made the following observations and proposals:

- Higher layer CU-DU split option (i.e., Option 2 for PDCP/RLC split) is adopted in 5G RAN, considering 
1/ the trade-off between centralization gains and hardware/transport costs, and 
2/ the alignment with Dual-Connectivity architecture.

- The support of NR-6GR DC and/or 6G-6G DC is still pending the progress and conclusion of RAN Plenary-level study, which has impact on the study of 6G CU-DU split.

- Non-ideal transport conditions between CU and DU has to be considered for 6G CU-DU split.

- Although CU-DU interface is fully within RAN node scope, the latency due to non-ideal transport conditions between CU and DU has also impact on UE and the air-interface.

- From user plane data transmission perspective, tight PDCP-RLC interaction between CU and DU is still needed in case of CU-DU split, which may cause extra latency and signalling overheads over F1 interface.

- From control plane perspective, CU-DU split leads to additional latency for RRC layer in CU.

- CU-DU split solution could be left to NW implementation in case that there is no need on inter-vender CU and DU.

- RAN3 discuss whether to support CU-DU split in 6G RAN or not, before studying different CU-DU split options.

4. Reference

[1] RP-251881, New SID: Study on 6G Radio, NTT DOCOMO