3GPP TSG-RAN WG3 Meeting #129bis	R3-256781

Prague, Czech Republic, Oct. 13th-17th

Source:				NEC

Agenda item: 	9.2.1 R19 SON/MDT

Title: 	Correction for MRO of LTM failure without RLF report

Document for:				Discussion and decision

- Introduction

In this contribution, we discuss a correction for LTM MRO failure without RLF report, to allow the the source gNB-DU to have the exact identity of the source cell and target cell of the failed LTM cell switch.

- Discussion

In Rel-19 MRO for LTM, in case of absent of RLF-report of a failed LTM cell switch, the gNB-CU can transmit to the source DU the LTM failure information, which includes CGI of the re-establishment or recovery cell, the C-RNTI of the UE in the source cell, and the failure type of the LTM cell switch.

However, the identity of the source cell and target cell of the LTM cell switch procedure are not provided to the source DU. In our understanding, if this information is not included, it requires the CU to provide this information immediately after the LTM failure before a next LTM triggered by the same DU for the same UE, because otherwise the information may mislead the gNB-DU. This is not easy to be achieved, because it is hard for the CU to predict when the following LTM cell switch would be triggered, and it may also depend on when the CU receives the Failure Indication message from another gNB, if the UE re-establishes the connection in another gNB. And we also think it brings difficulty on the network device implementation. So avoid the case that the source gNB-DU associates the information of LTM failure without RLF report with a different LTM cell swith procedure, we propose to add the source cell identity and target cell identity as optional IEs in the Reporting without RLF report IE within the CU-DU CELL SWITCH NOTIFICATION message. And this is also to align with the Xn interface for legacy handover, i.e., there is source cell ID and target cell ID in the Handover Report message, and failed cell ID in the Failure Indication for the case of failure without RLF-report.

Additionally, we also notice that the F1AP procedures description for the Reporting without RLF report IE is missing, and propose to add it.

Proposal 1. Add source cell identity and target cell identity as optional information in the Reporting without RLF report IE within the CU-DU CELL SWITCH NOTIFICATION message.

Proposal 2. Add related F1AP procedure description for the Reporting without RLF report IE.

Proposal 3. Agree to the CR for TS 38.473 in Annex that capture the above proposals.

# 3. Conclusion

In this contribution we made the following proposals:

Proposal 1. Add source cell identity and target cell identity as optional information in the Reporting without RLF report IE within the CU-DU CELL SWITCH NOTIFICATION message.

Proposal 2. Add related F1AP procedure description for the Reporting without RLF report IE.

Proposal 3. Agree to the CR for TS 38.473 in Annex that capture the above proposals.

Annex

8.11.1	Access and Mobility Indication

8.11.1.1	General

This procedure is initiated by gNB-CU to send the Access and Mobility related Information to gNB-DU.

The procedure uses non-UE-associated signalling.

8.11.1.2	Successful Operation

Figure 8.11.1.2-1: Access and Mobility Indication procedure. Successful operation

The Access and Mobility Indication procedure is initiated by ACCESS AND MOBILITY INDICATION message sent from gNB-CU to gNB-DU.

If the ACCESS AND MOBILITY INDICATION message contains the RA Report List IE the gNB-DU shall take it into account for optimisation of RACH access procedures.

If the ACCESS AND MOBILITY INDICATION message contains the RLF Report Information List IE the gNB-DU shall take it into account for optimisation of mobility parameters.

If the ACCESS AND MOBILITY INDICATION message contains the Successful HO Report Information List IE the gNB-DU may take it into account for optimisation of mobility parameters.

If the ACCESS AND MOBILITY INDICATION message contains the Successful PSCell Change Report Information List IE, the gNB-DU may take it into account for optimisation of PSCell change/addition related parameters.

If the ACCESS AND MOBILITY INDICATION message contains the BFR SSB Index IE within the MRO for LTM Information IE, the gNB-DU shall, if supported, consider that a Beam Failure Recovery after successful LTM Cell Switch has occurred in the target DU for the UE identified by the gNB-DU UE F1AP ID IE, and may take it into account for optimisation of target beam selection.

If the ACCESS AND MOBILITY INDICATION message contains the Target SSB Index after Cell Switch Failure IE within the MRO for LTM Information IE, the gNB-DU shall, if supported, consider that a successful recovery or re-establishment occurred in this SSB after an LTM Cell Switch Failure for the UE identified by the gNB-DU UE F1AP ID IE, and may take it into account for optimisation of target beam selection or optimisation of Timing Advance related parameters for the UE identified by the gNB-DU UE F1AP ID IE.

If the ACCESS AND MOBILITY INDICATION message contains the TA Information IE within the MRO for LTM Information IE, the gNB-DU may take it into account for optimisation of Timing Advance related parameters or optimisation of target beam selection for the UE identified by the gNB-DU UE F1AP ID IE.

If the ACCESS AND MOBILITY INDICATION message contains the Reporting without RLF report IE, the gNB-DU may take it into account for optimisation of LTM cell switch related parameters

9.2.10.1	ACCESS AND MOBILITY INDICATION

This message is sent by gNB-CU to gNB-DU to provide access and mobility information to the gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| RA Report List |  | 0..1 |  |  | YES | ignore |
| >RA Report Item |  | 1 .. <maxnoofRAReports> |  |  | - |  |
| >>RA Report Container | M |  | OCTET STRING | Includes the RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| RLF Report Information List |  | 0..1 |  |  | YES | ignore |
| >RLF Report Information Item |  | 1 .. <maxnoofRLFReports> |  |  | - |  |
| >>NR UE RLF Report Container | M |  | OCTET STRING | Includes the nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| >>C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the source gNB-DU. This IE is included in case the gNB-DU responsible for the LTM failure is not the gNB-DU serving the UE at the time of LTM failure. | YES | ignore |
| >>RLF Report Failure Type | O |  | ENUMERATED (too late LTM, too early LTM, LTM to wrong cell,...) |  | YES | ignore |
| Successful HO Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report Information Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  | - |  |
| >>Successful HO Report Container | M |  | OCTET STRING | Includes the SuccessHO-Report IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| Successful PSCell Change Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful PSCell Change Report information Item |  | 1..<maxnoofSuccessfulPSCellChangeReports> |  |  | - |  |
| >>Successful PSCell Change Report Container | M |  | OCTET STRING | Includes the SuccessPSCell-Report IE as defined in TS 38.331 [8]. | - |  |
| Reporting without RLF report |  | 0..1 |  |  |  |  |
| >NR CGI | M |  | 9.3.1.12 | NR CGI for the re-establishment or recovery cell. | - |  |
| >C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the source gNB-DU | YES | ignore |
| >LTM Failure Type | O |  | ENUMERATED (too late LTM, too early LTM, LTM to wrong cell,...) |  | YES | ignore |
| > Source cell CGI | O |  | 9.3.1.12 | NR CGI of source cell of the LTM cell switch procedure. | YES | ignore |
| > Target cell CGI | O |  | 9.3.1.12 | NR CGI of target cell for LTM cell switch procedure | YES | ignore |
| MRO for LTM Information |  | 0..1 |  |  | YES | ignore |
| >gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | - |  |
| >BFR SSB Index | O |  | INTEGER (0..63) | SSB Index of the recovery beam used at successful Beam Failure Recovery. | - |  |
| >Target SSB Index after Cell Switch Failure | O |  | INTEGER (0..63) | SSB Index of the re-established or recovery beam after LTM Cell Switch Failure. | - |  |
| >TA Information | O |  | INTEGER (0..4095) | Indicates the TA value, as defined in TS 38.213 [31], used at successful Random Access during LTM recovery or re-establishment after a Cell Switch failure in same cell. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRAReports | Maximum no. of RA Reports, the maximum value is 64. |
| maxnoofRLFReports | Maximum no. of RLF Reports, the maximum value is 64. |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports, the maximum value is 64. |
| maxnoofSuccessfulPSCellChangeReports | Maximum no. of Successful PSCell Change Reports. Value is 64. |