3GPP TSG-RAN WG3 #129bis	R3-256784

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:		13.2

Title:		Discussion on Network architecture for ISAC

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In the ISAC SID [1], the following objects should be studied in RAN that:

Evaluate the performance of gNB-based mono-static sensing (i.e., single TRP with co-located sensing transmitter and receiver) for UAV use case [RAN1] - Identify and study metrics, measurements, and relevant measurement quantization for UAV use case - As baseline, existing DL NR waveform and DL NR reference signals are to be used for evaluations. - For other waveform and reference signals, companies are to share relevant information - No UE impacts - Deployment scenario and assumptions for channel model calibration for UAV sensing targets in the Rel-19 ISAC channel model SI [FS_Sensing_NR] are used as starting point for evaluation assumptions. - FR1 frequency range is prioritized. Study the procedures, signaling between RAN and CN to support ISAC [RAN3] Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

This contribution will discuss the network architecture regarding the ISAC.

# 2. Discussion

The ISAC architecture design is expected to only support gNB-based sensing operations for UAV detection and tracking use case that:

Figure 1: Demonstration structure of gNB mono-static sensing mode

2.1 RAN and Sensing Function candidate architecture

The ISAC TR 23.700-14 [2] from SA2 provides various solutions regarding network architecture, sensing service configuration, and sensing data collection. The following architectural diagrams represent the most frequently architectures in TR 23.700-14.

| Candidate Architecutures: | Observations: |
| --- | --- |
| Architecture 1: | The direct connection (Nx) between RAN and Sensing Function (e.g., Solution#21, #22). - The new interface (Nx) is needed between the RAN and the Sensing Function (SF). - The new interface will be established when the SF discover and select the RAN. - The control signaling and sensing data will be transported via the new interface directly between RAN and SF. |
| Architecture 2: | The direct connection (Nx) between RAN and Sensing Control function (SCF), and direct connection (Ny) between RAN and Sensing Processing Function (SPF) (e.g., Solution#23, #25). - The new interface (Nx and Ny) are needed between the RAN and the SCF and SPF, respectively. - The new interface (Nx and Ny) will be established when the SCF discover and select the RAN. - The Nx is used to transmit the control signaling while the Ny is is used to transmit the sensing data. |
| Architecture 3: | The indirect connection between RAN and SF via AMF (e.g., Solution #7, #24) where the SF may be further split into SCF and SPF. - The N2 interface is re-used between the RAN and AMF to carry the sensing information. - No RAN-SF interface establishment needed. - The control signaling and sensing data will be transported explicitly over the N2 interface. |
| Architecture 4: | The indirect connection between RAN and SF via AMF, but the there is an RAN to SF interface (NRPPa-like). (e.g., Solution#1) - A new interface is needed between the RAN and the Sensing Function (SF). - The new interface will be established when the SF discover and select the RAN. - The control signaling and sensing data will be transported via the new interface directly between RAN and SF. And the signaling or sensing data may be transparent to the AMF. |

From a RAN3 perspective, the different architectures mentioned above will have different impacts on interface management, control signaling exchange, and sensing data reporting.

Proposal 1: RAN3 should evaluate and study the following aspects for both direct and indirect connections between RAN and SF:

Interface management

Control signaling exchange

Sensing data reporting

Proposal 2: For the both direct and indirect connections between RAN and SF, the interface between RAN and SF has the following options:

Define a new interface between RAN and SF for direct connection.

Re-use the N2 interface between RAN and SF for indirect connection via AMF.

Proposal 3: The indirectly connection and transport the control Signaling and sensing data explicitly over the N2 interface (Architecture 3) should be considered as a baseline solution since it has minimum RAN-CN interface.

# 3. Conclusion

Proposal 1: RAN3 should evaluate and study the following aspects for both direct and indirect connections between RAN and SF:

Interface management

Control signaling exchange

Sensing data reporting

Proposal 2: For the both direct and indirect connections between RAN and SF, the interface between RAN and SF has the following options:

Define a new interface between RAN and SF for direct connection.

Re-use the N2 interface between RAN and SF for indirect connection via AMF.

Proposal 3: The indirectly connection and transport the control Signaling and sensing data explicitly over the N2 interface (Architecture 3) should be considered as a baseline solution since it has minimum RAN-CN interface.

# References

[1] RP-252819, Study on Integrated Sensing And Communication (ISAC) for NR

[2] TR 23.700, Study on Integrated Sensing and Communication