3GPP TSG-RAN WG3 Meeting #129bis	R3-256827

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:	13.3

Source: 	Qualcomm Incorporated

Title: 	Signalling and Procedures for NR Sensing Support

Document for:		Discussion and Decision

# 1.	Introduction

At RAN#108, a new (RAN1-led) Study Item has been approved to study "Integrated Sensing And Communication (ISAC) for NR", which has been slightly revised at RAN#109 [1]. The objectives for this study are shown below.

This study item aims to study the following aspects for Integrated Sensing and Communication (ISAC): Evaluate the performance of gNB-based mono-static sensing (i.e., single TRP with co-located sensing transmitter and receiver) for UAV use case [RAN1] - Identify and study metrics, measurements, and relevant measurement quantization for UAV use case - As baseline, existing DL NR waveform and DL NR reference signals are to be used for evaluations. - For other waveform and reference signals, companies are to share relevant information - No UE impacts - Deployment scenario and assumptions for channel model calibration for UAV sensing targets in the Rel-19 ISAC channel model SI [FS_Sensing_NR] are used as starting point for evaluation assumptions. - FR1 frequency range is prioritized. Study the procedures, signalling between RAN and CN to support ISAC [RAN3] Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

For the RAN3 objectives, the study is expected to be completed at RAN#112 (June 2026; which gives five RAN3 meetings until RAN3#132). A parallel study is ongoing in SA2 [2], which is expected to be completed at SA#110 (December 2025). The RAN1 aspects are expected to be completed at RAN1#124 (February 2026 (RAN#111)).

Some general aspects on the architecture and protocols to support sensing are discussed in our companion contribution in [3]. In this contribution, we discuss the general procedures to support sensing.

# 2.	Procedures and signalling between SF and gNB

New procedures between the Sensing Function (SF, see [3]) and gNB are required to support (at least) one or more of the following functions.

-	Request Sensing TRP information by an SF from the gNB, such as TP and/or RP functionality, geographical coordinates of the TPs and RPs, and possibly other configuration information (e.g., beam antenna information, etc., dependent on RAN1).

-	Request DL "Sensing Signal Information" by an SF from a TP which may be required to perform the sensing measurements at an RP (e.g., Sensing Signal Transmission Characteristics, dependent on RAN1), e.g., if TPs and RPs are not co-located (bi-static).

-	Request Sensing Signal Measurements/Results by an SF from an RP, possibly including the Sensing Signal Transmission Characteristics (e.g., frequency, bandwidth, etc.).

By comparing the above with positioning [4],[5], one can observe that the above would resemble the following NRPPa procedures [5]:

-	TRP Information Exchange

-	Positioning Information Exchange

-	Measurement Information Transfer

Accordingly, a general procedure for the sensing operation could be as shown in Figure 1.

Figure 1: Sensing Procedure between SF and gNB.

(1)	The Sensing Function (SF) receives a Sensing Request from some entity in the core network. The details of this Sensing Request may be defined by SA2. However, it is assumed that this Sensing Request includes (at least) some information on the sensing area (e.g., a geographical area where the sensing should be performed to detect the object, together with QoS information such as desired location/velocity accuracy).

(2)	The SF may perform a TRP Information Exchange procedure to obtain required information from gNBs/TRPs, such as geographical coordinates of TRPs, sensing functionality (e.g., TP and/or RP functionality, etc.). The SF may send a TRP INFORMATION REQUEST message to the gNB to request the desired information, which the gNB may then provide in a TRP INFORMATION RESPONSE message. If the gNB is not able to provide any information, it may return an TRP INFORMATION FAILURE message indicating the cause of the failure.

(3)	The SF may select TRPs in the sensing area which should perform the sensing operation.

NOTE 1:	According to the SID, only single-TRP sensing is considered with no coordination between gNBs. However, the sensing operation may still need to be performed by multiple gNBs/TRPs in the sensing area, since the location of the desired object may not be known a priori, and the desired object (UAV in our case) is generally moving.

NOTE 2:	According to the SID, applicability to bi-static sensing may be considered. In this case, the SF may select the TP(s) and RP(s) pairs performing the sensing operation.

(4)	The SF may request a Sensing Signal Configuration from the sensing TP. The request may include a desired sensing signal characteristic, such as desired bandwidth of the sensing signal. The SF may send a SENSING INFORMATION REQUEST message to the gNB/TP to request a sensing signal configuration, which the gNB/TP provides in a SENSING INFORMATION RESPONSE message to the SF. If the gNB is not able to configure a DL sensing signal at the requested TP, it may return a SENSING INFORMATION FAILURE message indicating the cause of the failure.

(5)	The SF sends a SENSING DATA REQUEST message to the selected RP to request sensing measurement information. The message includes any information required for the RP to perform the measurements, e.g., frequency, bandwidth, etc. of the sensing signal (as obtained at Step 4).

(6)	If the report characteristics in step 5 is set to "on demand", the RP obtains the requested sensing measurement information and returns them in a SENSING DATA RESPONSE message to the SF. The SENSING DATA RESPONSE message includes the obtained sensing measurement information.

If the report characteristics in step 5 is set to "periodic", the RP replies with a SENSING DATA RESPONSE message without including any sensing measurements in the message. The RP then periodically initiates the SENSING DATA REPORT procedure in step 7, at the requested reporting periodicity.

If the gNB/RP is not able to accept the SENSING DATA REQUEST message in step 5, the gNB/RP returns a failure message indicating the cause of the failure.

(7)	The RP periodically provides the sensing measurement information to the SF if that was requested at step 5.

(8)	At any time after step 6, the SF may send a SENSING DATA UPDATE message to the RP providing updated information required for the RP to perform the sensing measurements. Upon receiving the message, the RP overwrites the previously received measurement configuration information.

(9)	If the previously requested sensing measurement information can no longer be reported, the RP notifies the SF by sending a SENSING DATA FAILURE message.

(10)	When the SF wants to abort an ongoing sensing measurement it sends a SENSING DATA ABORT message to the gNB.

Observation 1:	A SF needs to have control and visibility of individual TRPs in (at least some) sensing procedures.

Observation 2:	A sensing session could be performed with the following elementary procedures:
-	TRP Information Exchange
-	Sensing Signal Information Exchange
-	Sensing Data Information Transfer.

Observation 3:	The elementary procedures according to Observation 2 would be analogous to the corresponding NRPPa procedures:
-	TRP Information Exchange
-	Positioning Information Exchange
-	Measurement Information Transfer.

Proposal 1:	A "Sensing Protocol" should support at least the following elementary procedures:
-	TRP Information Exchange (to exchange TRP related information),
-	Sensing Signal Information Exchange (to exchange sensing signal characteristics),
-	Sensing Data Information Transfer (to exchange the sensing measurements/results).

Proposal 2:	The TRP(s) participating in the sensing operation should be selected by the SF.

Given Observation 3, RAN3 should study whether NRPPa could be extended to support the sensing operations (instead of defining a new "Sensing Protocol").

Proposal 3:	RAN3 study the possibility of extending NRPPa to support sensing.

# 3.	Signalling between gNB-CU and gNB-DU

For the "Sensing Protocol", the messages and functionality should not depend on the specific architecture deployed (e.g. monolithic- or split gNB). The NRPPa elementary procedures according to Observation 3 [5] are also mirrored in F1AP [6]. This duplication of messages seems unnecessary and increases the specification maintenance and implementation complexity. Based on the procedure in section 2 above, the "Sensing Protocol" messages specifically address TRP(s) (TP and/or RP). Corresponding response messages will also be structured per TRP (e.g., sensing results of a specific RP). In case of a split-gNB, the gNB-CU will essentially route these requests to the gNB-DU hosting the TRP(s). Therefore, the structure of the F1AP message would be very similar or the same to that in the "Sensing Protocol".

Observation 4:	A number of "Sensing Protocol" messages may include (a list of) TRP(s) with associated requests or responses, and the resulting F1 messages will have the same structure.

In order to avoid duplicated sensing message definition in the "Sensing Protocol" and in F1AP (which increases specification maintenance effort and could increase implementation), it would be desirable to collect all sensing related messages and procedures in a single protocol ("Sensing Protocol"). This would be straightforward if "Sensing Protocol" messages can be relayed through a gNB-CU without change between an SF and a gNB-DU/TRP.

For example, the Figure 2 below shows a typical NRPPa message structure which includes some header information and TRP-level information. The messages for the "Sensing Protocol" (whether new protocol or extension of NRPPa) are expected to have the same structure.

Figure 2: Generic message structure for TRP information.

In case of aggregated architecture, the gNB processes the information in "Sensing Messages" and controls TRPs based on implementation. In the case of the split architecture, the gNB-CU would only need to process some header information and forward the TRP-level Information to specific TRPs in F1AP messages.

The TRP-level information would not need to be comprehended by the gNB-CU. The TRP-level information could be included in OCTET STRINGs whose internal encoding would be defined in the "Sensing Protocol" only (e.g., NRPPa if re-used for sensing). This would enable simple copy/paste without interpretation, when transferring TRP-level information from a "Sensing Protocol" message to an F1AP message or the reverse. However, interpretation could still take place in the gNB-CU, if required (just as RRC messages conveyed in XnAP can be optionally interpreted in some cases).

Observation 5:	A generic F1-AP UL/DL Transport Message could be defined, which carries an OCTET STRING container. The message definition required for gNB-CU and gNB-DU/TRP signalling can be defined in the "Sensing Protocol".

This approach would not require mirroring every single "Sensing Protocol" change in F1AP, which has proven to be a rather cumbersome process in positioning and may also lead to errors (either in the specs or implementation).

Note, if existing NRPPa procedures — such as TRP Information Exchange — are extended to support sensing (as outlined in Observations 2 and 3), the current approach of mirroring NRPPa changes to F1AP will still be necessary. However, any newly introduced sensing-specific messages — whether integrated into NRPPa or defined within a dedicated "Sensing Protocol" — can adopt the simpler design principles described above (e.g., Sensing Data Information Transfer, which may have more stringent "real-time" requirements).

Proposal 4:	Study a generic F1-AP UL/DL Transport Message for sensing messages, which carry an OCTET STRING container for the TRP-level message information whose internal encoding would be defined in the "Sensing Protocol" only.

# 4.	Summary

In this contribution, we discussed the general procedures to support sensing. The following Observations and Proposals were made.

Observation 1:	A SF needs to have control and visibility of individual TRPs in (at least some) sensing procedures.

Observation 2:	A sensing session could be performed with the following elementary procedures:
-	TRP Information Exchange
-	Sensing Signal Information Exchange
-	Sensing Data Information Transfer.

Observation 3:	The elementary procedures according to Observation 2 would be analogous to the corresponding NRPPa procedures:
-	TRP Information Exchange
-	Positioning Information Exchange
-	Measurement Information Transfer.

Observation 4:	A number of "Sensing Protocol" messages may include (a list of) TRP(s) with associated requests or responses, and the resulting F1 messages will have the same structure.

Observation 5:	A generic F1-AP UL/DL Transport Message could be defined, which carries an OCTET STRING container. The message definition required for gNB-CU and gNB-DU/TRP signalling can be defined in the "Sensing Protocol".

Proposal 1:	A "Sensing Protocol" should support at least the following elementary procedures:
-	TRP Information Exchange (to exchange TRP related information),
-	Sensing Signal Information Exchange (to exchange sensing signal characteristics),
-	Sensing Data Information Transfer (to exchange the sensing measurements/results).

Proposal 2:	The TRP(s) participating in the sensing operation should be selected by the SF.

Proposal 3:	RAN3 study the possibility of extending NRPPa to support sensing.

Proposal 4:	Study a generic F1-AP UL/DL Transport Message for sensing messages, which carry an OCTET STRING container for the TRP-level message information whose internal encoding would be defined in the "Sensing Protocol" only.

# References

[1]	RP-252819, "Revised SID: Study on Integrated Sensing And Communication (ISAC) for NR".

[2]	SP-250401, "New SID on Study on Stage 2 for Integrated Sensing and Communication".

[3]	R3-256826, "Network Architecture and Protocol Aspects for NR Sensing Support", Qualcomm Incorporated.

[4]	3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[5]	3GPP TS 38.455: "NR Positioning Protocol A (NRPPa)".

[6]	3GPP TS 38.473: "F1 application protocol (F1AP)".