3GPP TSG-RAN WG3 Meeting #129-bis                      R3-256874

Prague, Czech Republic, 13th ~17th Oct, 2025

Agenda Item:	9.2.5

Source:	CATT

Title:	Correction on LTM candidate PSCell cancel in DC

Document for:	Discussion

# 1	Introduction

LTM candidate PSCell partial cancel is supported in inter-SCG LTM, this contribution is for the IE designation for PSCell cancel.

2	Discussion

The candidate PSCells to be cancelled is included in the SN Modification Required message.

The MN may inform the source SN about the cancellation of the prepared candidate PSCells at a candidate SN using SN modification request message.

the SN Modification Required message includes an indicator indicating the candidate PSCells to be cancelled.

To support source SN or candidate SN initial cancellation, we introduced LTM Candidate PSCell To be Cancelled List IE in S-NODE MODIFICATION REQUIRED message. The list is reference to 9.2.3.242 LTM Candidate PSCell Request List, which is also used for MN request candidate SN providing candidate PSCell configuration. This structure makes specification a bit confusing and easy to be interpreted as cancel all the requested cells.

And in the S-NODE MODIFICATION CONFIRM message, we introduce LTM Candidate PSCell Information Update Required IE to reflect the cancel results. The list is reference to 9.2.3.243 LTM Candidate PSCell Prepared List IE, which also used for candidate SN providing candidate PSCell configuration. This structure make it hard to decide the IE presence inside LTM Candidate Prepared PSCell List IE. For example, if we only considered S-NODE MODIFICATION CONFIRM message, all the IEs inside the LTM Candidate PSCell Prepared List IE shall be optional except PSCell ID IE. Such designation is not align with our intention.

The same issues are also exist in S-NODE MODIFICATION REQUEST and S-NODE MODIFICATION REQUEST ACKNOELEDGE message. Based on above, we suggest to introducing two new lists for candidate cell cancel, and only need to contain necessary information in it.

9.1.2.8	S-NODE MODIFICATION REQUIRED

| LTM Candidate PSCell Information Update Required | O |  |  |  | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >LTM Candidate PSCell To be Cancelled List | O |  |  |  | – |  |
| >>LTM Candidate PSCell to be cancelled Item |  | 1 .. < maxnoofLTMCells> |  |  |  |  |
| >>>PSCell ID | M |  | NR CGI 9.2.2.7 |  |  |  |

9.1.2.9	S-NODE MODIFICATION CONFIRM

| LTM Candidate PSCell Information Cancel Confirm | O |  |  |  | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >LTM Candidate PSCell Canceled List | 0…1 |  |  |  | – |  |
| >>LTM Candidate PSCell Cancelled Item |  | 1 .. < maxnoofLTMCells> |  |  |  |  |
| >>>PSCell ID | M |  | NR CGI 9.2.2.7 |  |  |  |
| >CSI Resource Configuration | O |  | 9.2.3.223 |  | – |  |
| >LTM Configuration ID Mapping List | O |  | 9.2.3.221 |  | – |  |

Proposal 1: Suggest to using a new list for LTM Candidate PSCell cancel in S-NODE MODIFICATION REQUIRED and S-NODE MODIFICATION REQUEST.

Proposal 2: Suggest to using a new list for LTM Candidate PSCell cancel in S-NODE MODIFICATION CONFIRM message.

3	Conclusion

Proposal 1: Suggest to using a new list for LTM Candidate PSCell cancel in S-NODE MODIFICATION REQUIRED and S-NODE MODIFICATION REQUEST.

Proposal 2: Suggest to using a new list for LTM Candidate PSCell cancel in S-NODE MODIFICATION CONFIRM message.