3GPP TSG RAN WG3 Meeting #129-bis	R3-256935

Prague, Czech Republic, 13 – 17 October, 2025

Title: 	Support for Continuous MDT in RAN3 specifications

Source: 	Ericsson, Deutsche Telekom, FiberCop, Jio Platforms, InterDigital, BT, Orange, Verizon

Agenda item:	9.2.2

Document Type:	Discussion and agreement

# 1. Introduction

During RAN3#128 meeting progress has been achieved and the following agreements were taken:

Information from OAM to participating nodes in the NG-RAN allowing for the identification of a continuous MDT process are provided in the form of specific MDT TR(s).

In the most obvious solution, this implies that specific TRs are allocated for Continuous MDT and the RAN knows that these TRs are specific for the continuous MDT process.

Solution for continuous MDT shall not have an impact on the 5GC in R19 from RAN3 perspective.

This is an important statement that removes solutions involving the AMF.

Agree to use TR(s) and TRSR(s) to resolve the MDT measurement correlation problem.

This agreement can derive in different solutions, but one of them covers the possibility to reuse the TRSR across gNBs and RRC states.

Furthermore, an LS to SA5 was agreed in R3-253958 [1], capturing the agreements above and answering some questions SA5 asked RAN3 specifically.

An LS response from SA5 has been received [2] and SA5 is

“SA5 has studied the request and agreed that the principles present by the RAN3 incoming LS is feasible. Therefore, SA5 has agreed a solution for Continuous Management-Based MDT in the context of non-split RAN architecture. For split RAN architecture, the same solution is supported at the gNB-CU-CP only.”

In particular SA5 agreed that the specification of Continuous Management-Based MDT is based on the following principles [3]:

“

- Reuse of the existing Management-Based MDT framework, with minimal impact on the current architecture and procedures.

- Introduction of an OAM-triggered activation mechanism toward participating NG-RAN nodes, enabling identification of a continuous MDT job through specific Trace Reference(s).

- No impact to 5GC functionality.

- No impact to the UE, ensuring that UE behaviour remains unchanged.

- Use of Trace Reference (TR) and Trace Recording Session Reference (TRSR) to support correlation of MDT measurements collected across nodes, including UE transitions between RRC states and UE mobility.”

The following is specified in TS32.422 [3]:

Continuous management-based MDT: a management-based MDT functionality that enables continuous collection of MDT data, including both Immediate MDT and Logged MDT, within a specified area. NOTE: This MDT data collection captures UE transitions between RRC states (RRC_IDLE, RRC_INACTIVE, RRC_CONNECTED), as well as UE mobility between participating NG-RAN nodes. The MDT data collection on selected UEs is stopped when both Immediate MDT and Logged MDT are deactivated. […] For non-split RAN architecture, the C-MDT data collection procedure shall be same as specified in subclause 4.1.1.9.2 with following additions. 1) The management system shall either send two Trace Session activation requests (one for Immediate MDT, another for Logged MDT) with same TR to the gNB, or send one Trace Session activation request with Job Type (IMMEDIATE_MDT_AND_ LOGGED_MDT) to the gNB. In additional of the trace control and configuration parameters specified in subclause 4.1.1.9.2 step 1, the Trace Session activation request(s) shall contain a TRSR Prefix Configuration parameter, which reserves a TRSR range(s) for the gNB during TRSR assignment. This TRSR Prefix Configuration parameter is used to identify this is a C-MDT activation request. Furthermore, the management system shall ensure that Trace Session activation requests with the same TR are sent to all participating NR-RAN nodes involved in the same C-MDT job. 2) Upon receiving the Trace Session activation request(s), the gNB shall identify the Trace Session activation request(s) as a C-MDT job, start a C-MDT session, and store the associated configuration parameters. 3) The gNB and UE shall follow the steps specified in subclause 4.1.1.9.2, step 3 to 8, with following additions: - The gNB shall configure the selected UE with an Immediate MDT session and a Logged MDT session using the configurations from the Trace Session activation request(s). A unique TRSR is assigned to the UE for Immediate MDT and Logged MDT. The TRSR is allocated based on the received TRSR Prefix Configuration parameter. - Upon transitioning into RRC_CONNECTED, the UE may report Logged MDT measurements, including TR and TRSR, to the gNB, as specified in subclause 4.1.1.9.2 step 7 and 8. If the reported TR is same as the Trace Reference received in the Trace Session Activation request(s), the gNB shall identify the UE as part of the same C-MDT job. If the area-based selection and user consent conditions are satisfied, the gNB shall reconfigure the UE with an Immediate MDT and a Logged MDT. - In any case, if a new TRSR is assigned, the correlation between the old TRSR and the new TRSR shall be reported by gNB to the TCE.

As it can be seen in the definition of Continuous management-based MDT, “This MDT data collection captures UE transitions between RRC states (RRC_IDLE, RRC_INACTIVE, RRC_CONNECTED), as well as UE mobility between participating NG-RAN nodes”.

In the CR agreed by SA5 for TS32.422, mechanisms to ensure reselection of the UE after transitioning to RRC_CONNECTED are described. However, no description is present on how the UE is reselected after mobility to a different gNB. This is obvious as such mobility solutions rely on RAN3 discussions and on changes to RAN3 specifications.

Observation 1: the solution for Continuous management-based MDT agreed by SA5 does not include mechanisms that allow for UE reselection after mobility to a different gNB because this is out of SA5 scope and in RAN3 scope.

In order to derive how to enable selection of a UE configured with Continuous management-based MDT at the target gNB, the solution from SA5 has to be carefully analysed. In that solution, the following is stated:

- Upon transitioning into RRC_CONNECTED, the UE may report Logged MDT measurements, including TR and TRSR, to the gNB, as specified in subclause 4.1.1.9.2 step 7 and 8. If the reported TR is same as the Trace Reference received in the Trace Session Activation request(s), the gNB shall identify the UE as part of the same C-MDT job. If the area-based selection and user consent conditions are satisfied, the gNB shall reconfigure the UE with an Immediate MDT and a Logged MDT. - In any case, if a new TRSR is assigned, the correlation between the old TRSR and the new TRSR shall be reported by gNB to the TCE.

From the above, two aspects can be deduced. One is that for a new gNB serving the UE, the way to recognise that the UE was previously configured with a Continuous management-based MDT job is to receive and check the TR assigned to the UE by its previously serving gNB.

The second aspect is that, when the UE is served by a new gNB, its TRSR may be changed. In this case, the new serving gNB shall notify such change to the TCE by signalling as part of the Trace Record Header the old TRSR and the new TRSR.

This is shown in the CR agreed by SA5 for TS32.423, which reports the following:

### 5.2.2 Trace Record Header The trace record header contains the common fields as specified in the Table 5.2.2-1, in addition it may also contain vendor specific extensions. Table 5.2.2.1 : Common fields in the trace record header Trace Record Header field name; Description. timeStamp (M); Time stamp (in milliseconds since Epoch) of when the streaming trace record is produced internally in the Producer encoded as (64 bit integer). nfInstanceId (M); Unique id of the Producer NF instance that produced this trace record represented by a (String). nfType (M); Type of the Producer NF that produced this trace record represented by a (String). traceReference (M); Trace Reference (see clause 5.6 of 3GPP TS 32.422 [3]) (represented by a 6 bytes octet string) See Note 6.. traceRecordingSessionReference (M); Trace Recording Session Reference (see clause 5.7 of 3GPP TS 32.422 [3]) represented by a (2 byte octet string. See Note 1.). traceRecordTypeId (M); Identifier of the trace record type (see clause 5.2.4 for details) represented by an ENUM with the following values: NORMAL, TRACE_SESSION_START, TRACE_SESSION_STOP, TRACE_RECORDING_SESSION_START, TRACE_RECORDING_SESSION_STOP, TRACE_STREAM_HEARTBEAT, TRACE_RECORDING_SESSION_NOT_STARTED, TRACE_RECORDING_SESSION_DROPPED_EVENTS, TRACE_FILE_OPEN, TRACE_FILE_CLOSE, TRACE_FILE_ABNORMAL_CLOSED, TRACE_RECORDING_SESSION_THROTTLED_START, TRACE_RECORDING_SESSION_THROTTLED_STOP. (See Note 2).. ranUeId (O); RAN defined id to represent a UE (8 byte octet string. See Note 3.). payloadSchemaURI (O); URI identifying the schema to be used in order to decode the payload represented by a (String. See Note 4.). globalGnbId (CM); Global gNB ID, as defined in subclause 9.3.1.6 of 3GPP TS 38.413 [23]. Applied for trace reported by gNB-CU-CP, gNB-CU-UP, gNB-DU.. vendorExtension (O); Vendor-specific extension(s) (See Note 5.). associatedTraceRecordingSessionReference (O); Associated Trace Recording Session Reference used in continuous MDT represented by a 2 byte octet string.. CHOICE 1.1: traceRecordName (O); A unique name string of a Trace Record. This attribute is not required for Administration events.. CHOICE 1.2: traceRecordId (O); A unique integer ID of a Trace Record.. NOTE 1: The traceRecordingSessionReference must be present for the Streaming Trace Records with non-zero size payload where the payload carries data captured for a Trace Recording Session and in administrative messages related to a Trace Recording Session (e.g. "Trace Recording Session Start" or "Trace Recording Session Stop"). NOTE 2: The traceRecordTypeId with value "NORMAL" is used for Streaming Trace Records that do not carry an administrative message. NOTE 3: The ranUeId field is present in the trace record header when the identifier is supported by RAN. If RAN UE Id (see 3GPP TS 37.483 [46] and 3GPP TS 38.473 [26]) has been captured in the traced signaling messages that value is used. NOTE 4: The payloadSchemaURI is not required for Streaming Trace Records with payload of zero-size, or payload using common payload format (e.g. used to convey Streaming Trace administrative messages). NOTE 5: The vendorExtension is typically a generic list of key-value pairs. NOTE 6: The encoding of the Trace Reference is a 6-byte Octet String in BCD format. The first 3-byte Octet String is the PLMN ID which consists of MCC and MNC. The next 3-byte Octet String is the Trace ID. The PLMN ID is encoded as specified in clause 9.3.3.5 of 3GPP TS 38.413 [23]..

Therefore, during a connected mode mobility procedure, in order to make a target gNB to be able to reconfigure a UE for Continuous management-based MDT, it is necessary that:

- The source gNB signals to the target gNB the TR assigned to the UE at the source gNB and

- The source gNB signals to the target gNB the TRSR assigned to the UE at the source gNB

The target gNBis able to deduce that the UE has to be configured with continuous MDT by checking the TR received from the source gNB. The target gNB will use the TRSR received from the source gNB to assign it to the UE and ensure continuity of the Continuous management-based MDT process.

Proposal 1: To support the solution on Continuous management-based MDT agreed by SA5, it is proposed that the TR and TRSR assigned by a source gNB to a UE configured for Continuous management-based MDT is included in the Xn: Handover Request and NG: Handover Required and NG: Handover Request messages.

# 2. Conclusion

In this paper changes need in the RAN3 specifications to support the Rel19 solution on Continuous management-based MDT agreed by SA5 are presented.

The following proposals and observations are derived:

Observation 1: the solution for Continuous management-based MDT agreed by SA5 does not include mechanisms that allow for UE reselection after mobility to a different gNB because this is out of SA5 scope and in RAN3 scope.

Proposal 1: To support the solution on Continuous management-based MDT agreed by SA5, it is proposed that the TR and TRSR assigned by a source gNB to a UE configured for Continuous management-based MDT is included in the Xn: Handover Request and NG: Handover Required and NG: Handover Request messages.

CRs mirroring the above proposals are available in R3-256937 and R3-256938, while a CR reflecting the solution agreed by SA5 to TS32.720 is available in R3-256936.

A Reply LS to SA5 is also available in R3-256944.

2. References

- R3-253958, Reply LS on Continuous MDT, RAN3

- S5-254110, Reply to LS on Continuous MDT

- S5-254111, Rel-19 CR TS 32.422 Continuous MDT