3GPP TSG-RAN WG3 Meeting # 129bis	        R3-256936

# Prague, Czech Republic, 13 – 17 October, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.320 | CR | draft | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Continuous MDT support |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Deutsche Telekom, FiberCop, Jio Platforms, InterDigital, BT, Orange, Verizon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_NGRAN_enh-Core, TEI18 |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR provides the stage 2 specification text for the Continuous Management-Based MDT procedure in line with the continuous MDT operation specified in SA5. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Defining the stage 2 text for Continuous MDT procedure Impact assessment towards the previous version of the specification (same release): This CR has isolated impact towards the previous version of the specification (same release). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unable to support Continuous MDT procedure as agreed in SA5 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 5.1.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.423 CR1559 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS 38.413 CR1352 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< Start of Set of Changes >>>>>>>>>>>>>>>>>>>>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

ACK	Acknowledgement

AICH	Acquisition Indicator CHannel

BLER	Block Error Rate

BSSID	Basic Service Set Identifier

C-MDT	Continuous management-based MDT

CA	Carrier Aggregation

CDMA	Code Division Multiple Access

CHO	Conditional Handover

CN	Core Network

CPICH	Common Pilot CHannel

DAPS	Dual Active Protocol Stack

DCH	Dedicated CHannel

DL	Downlink

DRX	Discontinuous Reception

ECGI	E-UTRAN Cell Global Identifier

E-CID	Enhanced Cell-ID (positioning method)

E-DCH	Enhanced Uplink DCH

EDGE	Enhanced Data rates for GSM Evolution

E-RUCCH	E-DCH Random Access Uplink Control CHannel

eNB	Evolved NodeB

EPLMN	Equivalent PLMN

E-UTRA	Evolved UTRA

E-UTRAN	Evolved UTRAN

FACH	Forward Access CHannel

FDD	Frequency Division Duplex

FIFO	First Input First Output

FPACH	Fast Physical Access CHannel

GERAN	GSM EDGE Radio Access Network

gNB	Next Generation Node B

GNSS	Global Navigation Satellite System

HESSID	Homogenous Extended Service Set Identifier

HOF	Handover Failure

IMEI-SV	International Mobile Equipment Identity Software Version

IMSI	International Mobile Subscriber Identity

IP	Internet Protocol

ISCP	Interference on Signal Code Power

LA	Location Area

LTE	Long Term Evolution

MAC	Medium Access Control

MBMS	Multimedia Broadcast Multicast Service

MBSFN	MBMS Single Frequency Network

MDT	Minimization of Drive-Tests

NG-RAN	Next Generation RAN

NPN	Non-Public Network

NR	New Radio

OAM	Operation and Maintenance

P-CCPCH	Primary Physical Common Control CHannel

PCH	Paging CHannel

PCI	Physical Cell Id

PDCP	Packet Data Convergence Protocol

PH	Power Headroom

PLMN	Public Land Mobile Network

PNI-NPN	Public Network Integrated Non-Public Network

PS	Packet Switched

QCI	QoS Class Identifier

QoS	Quality of Service

RA	Routing Area

RAB	Radio Access Bearer

RAT	Radio Access Technology

RB	Radio Bearer

RF	Radio Frequency

RLC	Radio Link Control

RLF	Radio Link Failure

RNC	Radio Network Controller

RPLMN	Registered PLMN

RRC	Radio Resource Control

RRM	Radio Resource Management

RSCP	Received Signal Code Power

RSRP	Reference Signal Received Power

RSRQ	Reference Signal Received Quality

RSSI	Received Signal Strength Indicator

RTT	Round Trip Time

RTWP	Received Total Wideband Power

SCell	Secondary Cell

SIR	Signal to Interference Ratio

SINR	Signal to Noise plus Interference Ratio

SNPN	Stand-alone Non-Public Network

SNR	Signal to Noise Ratio

SON	Self Organizing/Optimizing Network

SRB	Signalling Radio Bearer

SRNC	Serving RNC

SSB	Synchronization Signal Block

SSID	Service Set Identifier

TA	Tracking Area

TCE	Trace Collection Entity

TDD	Time Division Duplex

UE	User Equipment

UL	Uplink

UMTS	Universal Mobile Telecommunication System

UPH	Uplink PH

URA	UTRAN Registration Area

UTRA	Universal Terrestrial Radio Access

UTRAN	Universal Terrestrial Radio Access Network

<<<<<<<<<<<<<<<<<<<< End of Set of Changes >>>>>>>>>>>>>>>>>>>>

-- TEXT OMITTED –

<<<<<<<<<<<<<<<<<<<< Start of set of Changes >>>>>>>>>>>>>>>>>>>>

### 5.1.x	Continuous Management Based MDT procedure

The Continuous Management Based MDT operation is described in [6]. A RAN centric description is provided by the following figure and steps:

Figure 5.1.x-1: Example of signalling support for Continuous Management Based MDT

- RAN receives a configuration for an Immediate MDT and a Logged MDT, which are associated to a range of TRs that identify Continuous Management Based MDT configurations. TR1 is assigned to the Management Based Immediate MDT and to the Management Based Logged MDT.

- The RAN may configure the UE with Immediate MDT and Logged MDT. The Immediate MDT configuration corresponds to TR1, TRSR A and the logged MDT configuration corresponds to TR1 and TRSR A. The TRSR assigned to the UE during the course of the continuous MDT session may remain the same or it may be updated.

- The UE reports Immediate MDT measurements collected while in RRC_CONNECTED and according to the Immediate MDT configuration received.

- NG-RAN1 reports the measurements corresponding to the Management Based Immediate MDT configuration for TR1 and TRSR A to the TCE.

- The UE moves to RRC_IDLE/INACTIVE. The UE will collect Logged MDT measurements and store them as per received Logged MDT configuration. Logs include TR1 and TRSR A assigned by the source gNB.

- When the UE moves from RRC_IDLE/INACTIVE to RRC_CONNECTED, it reports Logged MDT logs, including TR1 and TRSR A. NG-RAN 2 realizes that the UE shall be configured with continuous MDT by means of reading TR1.

- NG-RAN 2 reports to the TCE the Trace Records including the Logged MDT measurements. These include TR1 and TRSR 1. By checking the TRSR the TCE can correlate the previous Immediate MDT measurements (for TR1 and TRSR A) with the Logged MDT measurements (associated to TR1 and TRSR A).

- Step 2, 3 and 4 are repeated at NG-RAN 2. The UE is configured with Management Based Immediate MDT with TR1 and TRSR A and Immediate MDT measurements are signalled to the TCE.  NG-RAN2 may reassign a new TRSR, in which case NG-RAN2 indicates both new and old TRSRs to the TCE.

- During connected mode mobility NG-RAN 2 includes in the Handover Preparation signalling towards NG-RAN 1 TR1 and the TRSR in use for the UE. NG-RAN 1 is able to deduce that the UE has to be configured with continuous MDT (by checking the TR) and that the same TRSR received from NG-RAN 2 shall be reported to the TCE to enable correlation of Continuous Management Based MDT measurements.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>