3GPP TSG-RAN WG3 #129-bis	R3-257074

Prague, Czech Republic, 13-17 October 2025

Title: 	(TP to TR 38.745) Considerations for Handover Enhancements in Release-20

Source: 	Ericsson

Agenda item:	12.2.3

Document Type:	Discussion and Decision

# 1.	Introduction

The objectives of Study on AIML for NG-RAN Phase 3 (RP-252867) which explores new AI/ML-based use cases and identification of enhancements to support AI/ML functionality is specified as follows:

- Study the AI/ML-based mobility use case based on the principles of AI/ML for NG-RAN as captured in TS 38.300 and TS 38.401 with existing NG-RAN interfaces and architecture, including the following scenarios:

  - multi-hop UE trajectory across gNBs;

  - intra-CU LTM;

  - handover enhancements, e.g. inter-CU LTM.

NOTE: No UE impacts.

This contribution discusses aspects corresponding to the handover enhancements area.

# 2.	Discussion

### 2.1	Background

2.1.1	Measured UE trajectory and UE performance

RAN3 as part of Release 18 normative work introduced the functionality of measured UE trajectory and UE performance, where a node (receiving a UE via an incoming handover) measures and reports back to the source of the handover, the trajectory or performance metrics of the UE measured at the current node.

UE trajectory in this case is defined as a list of cells in the target node, i.e., intra-gNB cells, that the UE traversed through and the time of stay in each of these cells. In Rel-18, measured UE trajectory was limited to a single node, i.e., UE trajectory collection ceases when/if the UE leaves the current gNB for a different node. In addition to the above criterion, there exists other conditions which define the termination of measured UE trajectory collection, e.g., a maximum time for which UE trajectory shall be collected, and a maximum number of visited cells in the serving gNB. When any of the above criteria are met, the UE trajectory collection is terminated, and the collected information is signalled to the source node.

UE performance is defined as the following metrics in Rel-18: Average UE throughput in UL and DL, Average packet delay in UL and DL, and average packet loss in the DL. There also exist conditions which define the termination of UE performance collection at a gNB, i.e., a maximum time since the UE was handed over to the current gNB, UE moves to RRC_INACTIVE or RRC_IDLE state, and the UE being handed over to another cell. The serving gNB performs UE performance collection as long as one of the specified exit conditions are met, and the collected measurements are signalled to the source gNB after the collection is terminated.

The exchange of the above measurements and the associated required configurations are described in TS 38.423 as part of the Data Collection Reporting Initiation and Data Collection Reporting procedures, where a neighbouring gNB may configure the reporting of chosen metrics from another node. In the context of UE trajectory and UE performance metrics, the presence of a DataCollectionID (an ID corresponding to a configured Data Collection Reporting procedure between the two nodes) in the Handover Request message triggers the target node to collect and report the metrics according to parameters configured in the corresponding Data Collection Reporting procedure.

2.1.2	Inter-CU L1/L2 Triggered Mobility (LTM)

Inter-CU LTM was standardised during Release 19. With LTM, the source node may use Layer 3 (L3) measurement reports from the UE to determine candidate target cells for LTM. In the LTM preparation phase, depending on the received measurements, the gNB may configure one or more cells as LTM candidate cells at the UE via an RRC Reconfiguration message. Prior to configuring the LTM candidate cells in the UE, the serving node initiates LTM preparation towards the candidate network nodes, where the candidate neighbours (either controlled by the same gNB-CU-CP or another gNB-CU-CP) learn about the UE-associated configuration for the LTM procedure either via the UE CONTEXT SETUP REQUEST or UE CONTEXT MODIFICATION REQUEST message in case of intra-gNB-CU LTM or via the HANDOVER REQUEST or the LTM CONFIGURATION UPDATE message in case of inter-gNB-CU LTM. For candidates where the LTM preparation phase is successful, the source node configures the UE for the specific candidate LTM cells. The UE performs early UL and DL pre-synchronization, with the goal of these steps being to obtain TA information in the uplink and DL sync in the downlink before the mobility action is triggered.

After completion of the LTM preparation phase, the UE measures and reports L1 and/or L3 measurements of the candidate LTM cells to the serving gNB, which decides to trigger LTM Cell Switch by signalling an LTM Cell Switch command with a pointer to one of the configured candidate cells. The UE then detaches from the source cell and connects to the candidate/target LTM cell and resumes transmission/reception of UL/DL data at the new serving cell.

After the initial LTM mobility event, the UE and the involved network nodes continue to keep the original set of LTM candidate cells from the initial serving gNB. The current serving gNB may update parts of the configuration, for example the TA information depending on reports from the UE. The current serving gNB may signal a subsequent LTM cell switch to the UE by sending the LTM Cell Switch command with a pointer to a different LTM candidate. The UE may continue its current session, potentially among the different candidate LTM cells configured by the initial serving gNB as long as the measurement reports from the UE results in the UE moving to one of the LTM candidate cells. The benefit of the above is that the signalling/configuration heavy parts of the procedure are performed once, and the associated configuration is maintained through different subsequent LTM mobility events, as long as the UE’s measurement reports indicate good coverage on the set of LTM candidate cells.

### 2.2	Considerations for Handover Enhancements in Release 20

2.2.1	LTM-related enhancements

In this section, we present our views on enhancements to be studied by RAN3 in context of the goal on handover enhancements in the Rel-20 SI.

The enhancements introduced as part of Rel-18 by RAN3 in the AI/ML for NG-RAN WI focussed on data collection enhancements and signalling support over existing network interfaces for three main use cases, i.e., network energy saving, load balancing, and mobility optimization. RAN3 introduced procedures to configure and report data pertaining to the above use cases with the vision that the introduced procedures may be further extended in the future for newly identified AI/ML-driven use cases. In fact, the Rel-19 WI introduced further extensions to the procedures. As AI/ML techniques develop, in addition to increased adoption rates, it is foreseen that AI/ML will assist the operators to improve network management and user experience in a proactive way. However, a central aspect, in relation to the above, is that the available procedures and mechanisms to collect data for AI/ML must be updated to collect relevant information in the context of new enhancements that may have been added to the standard.

LTM is one such enhancement which was also introduced during Rel-18 and Rel-19. In the below, we show that there exist differences between LTM and legacy mobility procedures such that it requires extensions or amendments to existing procedures for collection and reporting of data in the AI/ML context.

In the LTM execution phase (excluding the conditional LTM case), the serving gNB-CU initiates the mobility event by signalling the Cell Switch Command to the UE and subsequently a Cell Switch Notification (or a DU-CU Cell Switch Notification in the case of intra-gNB-CU LTM) to the target gNB-CU-CP. After a UE successfully connects to the target cell, the target gNB-DU signals an Access Success message to the gNB-CU-CP (in case of an intra-gNB-CU LTM) or a Handover Success message (in case of an inter-gNB-CU LTM) to the gNB-CU-CP. After completion of the LTM execution, the UE is connected to and is controlled by the new serving gNB-CU.

As described previously, the Data Collection Reporting Initiation and Data Collection Reporting are two procedures defined for the configuration and reporting of AI/ML related information between neighbouring nodes. If we now consider using the above procedures in order to obtain feedback for a mobility event, e.g., at handover, the source of the handover may ask the target to collect UE performance data or measure UE’s trajectory by inserting a DataCollectionID in the HANDOVER REQUEST message. The DataCollectionID corresponds to a procedure that has been previously configured between the source and the target and includes further configuration and reporting instructions for the collected metrics.

In context of the procedures defined for inter-gNB-CU LTM, the gNB/gNB-CU that initiates the LTM preparation procedure for a UE uses the HANDOVER REQUEST message to configure all candidate LTM targets, while any further changes to the LTM configuration for the UE is performed using the LTM CONFIGURATION UPDATE message. The above potentially implies that the node initiating the LTM configuration may potentially request for feedback from all of the candidate LTM targets. To understand this further, we present below, the interactions between the HANDOVER REQUEST message and the Data Collection procedure is specified as the following in TS 38.423 in clause 8.2.1.2:

If the Data Collection ID IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, report to the source NG-RAN node after successful handover, via the Data Collection Reporting procedure, the requested information configured via the previous Data Collection Reporting Initiation procedure corresponding to the NG-RAN node1 Measurement ID IE, allocated by the source NG-RAN node, and the NG-RAN node2 Measurement ID IE, allocated by the target NG-RAN node.

In the above excerpt, the relation between the nodes involved in the mobility event is defined in terms of the source NG-RAN node and the target NG-RAN node, however, in LTM it may be argued whether the node initiating the LTM preparation should be interpreted as the source node for all subsequent LTM mobility events. The above is surely not the case.

Proposal 1: RAN3 to discuss whether the Data Collection Reporting Initiation and Data Collection Reporting procedure can be initiated by the current serving node for LTM, or candidate node for LTM

Another aspect that needs consideration is that the LTM procedures use a different message, Cell Switch Command, to trigger imminent mobility to a certain candidate LTM node. Although the HANDOVER REQUEST message is used as part of LTM preparation, it does not imminently trigger a mobility event. Therefore possibilities/mechanisms for nodes involved in LTM procedures to collect data using the Data Collection Reporting Initiation and Data Collection Reporting procedures need to be clarified.

Proposal 2: RAN3 to discuss on the applicability of the Data Collection Reporting Initiation and Data Collection Reporting procedures in context of LTM.

Furthermore, the conditions for termination of collection of UE performance and measured UE trajectory as defined in TS 38.423 specifies that handover to a different cell or handover to a cell belonging to a different NG-RAN shall be treated as exit conditions respectively. The interpretation of the above in context of LTM mobility is unclear.

Proposal 3: RAN3 to discuss LTM-specific exit conditions for terminating the collection of UE performance and UE trajectory measurements.

The configuration pertaining to LTM, which was originally populated by the initiating gNB both at the UE (via RRC Reconfiguration messages) and at the different candidate LTM targets (either via HANDOVER REQUEST and later by LTM CONFIGURATION UPDATE in terms of inter-gNB-CU LTM, or via the UE CONTEXT SETUP REQUEST and UE CONTEXT MODIFICATION REQUEST in the case of intra-gNB-CU LTM), is maintained by the serving gNB-CU, where it may update the initial configuration with new information, for e.g., updating Timing Advance related information for the different candidate nodes. However, it is important to note that the initial set of LTM candidates determined by the initiating gNB-CU is kept across any subsequent LTM handovers.

The serving gNB-CU may however determine that a UE should be handed over to a cell/node that is currently not part of its LTM configuration, when no viable cells in the existing LTM configuration can be identified or the newly identified neighbour cell that is not part of the LTM configuration offers much better radio coverage. Under such scenarios, the serving gNB-CU may trigger a legacy L3 handover event or another form of mobility to a candidate neighbour node/cell. In the above context, it may be useful for the initiating gNB to know about the conditions that led to the UE being handed over to a non-LTM candidate cell.

As described in Proposal 1, the existing definition of source and target nodes in the Data Collection Reporting Initiation and Data Collection Reporting procedures are unclear when considering LTM as a use-case. In addition to the above, specific to the question of the node configuring the LTM candidate cells receiving feedback, it is important to enable such a mechanism such that better candidate LTM targets can be configured in the future.

In terms of information that may be useful for the gNB initiating LTM preparation, the following may be considered as a starting point:

- Information regarding conditions due to which the UE underwent a non-LTM mobility at the last node in the sequence of LTMs, for e.g., mobility due to coverage reasons, mobility due to predicted trajectory, mobility due to performance reasons

- LTM specific Multi-hop UE trajectory

- Measurements corresponding to LTM candidate cells when the non-LTM mobility decision was made

Proposal 4: RAN3 to discuss the issue of providing feedback information to the gNB initiating the LTM preparation.

2.2.2	Other enhancements

As previously described, the Data Collection Reporting Initiation and the Data Collection Reporting XnAP procedures also allow a gNB (e.g., gNB1) to request UE feedback from a neighbouring gNB (e.g., gNB2) in relation to handed-over UEs. The exit conditions under which gNB2 terminates the collection of UE feedback and reports it back to gNB1 are described in clause 8.4.13.2 in TS 38.423. In the case of measured UE performance, for single connectivity, the exit conditions are expiration of a timer, the UE moving to RRC_IDLE or RRC_INACTIVE state, or the UE moving to another cell.

When receiving the UE feedback, gNB1 is not aware of the particular exit condition nor the amount of time during which gNB2 collected the data. So, if we assume that gNB1 is interested in collecting UE performance during a 5-second interval, the UE can move to RRC_INACTIVE, or to RRC_IDLE, or can be handed over to another cell at any time between the moment the collection starts and until expiration of the timer. It is quite evident that the usefulness of the measured UE performance is not the same if the exit condition (UE transitions to RRC_INACTIVE or to RRC_IDLE, or UE is handed over to another cell) manifests itself after 200ms or after 4800ms. In other words, the amount of time during which the UE performance was collected is an important piece of information that makes the UE Performance feedback more or less relevant/accurate for the requesting node. However, the requesting node (gNB1) is totally unaware of the time duration for which the UE Performance was collected and, because of that, it cannot determine if some reported UE Performance metrics are valid and accurate or if they should be filtered out and not considered.

Observation 1: The amount of time during which the UE performance is collected is a useful indication of the goodness of the collected UE performance.

In the present framework, the node receiving the measured UE performance does not know the conditions under which the UE performance was collected. Following the previous example, if the collection of measured UE performance for a particular UE was terminated after 200ms, it is most likely due to the UE being handed over because the original target cell was not the best cell or due to the UE moving to RRC_IDLE after an RLF. On the other hand, if the collection was terminated after 4800ms, the UE may have changed cells due to normal UE mobility or the UE may have moved to RRC_IDLE after finishing a connection.

Therefore, we propose that RAN3 considers, in the scope of this AI, enhancements to the reporting of UE feedback that provide a better understanding of the data.

Proposal 5: RAN3 to discuss enhancements to the reporting of UE feedback.

# 3.	Conclusion

In this contribution, the following observations and proposals are made:

Proposal 1: RAN3 to discuss whether the Data Collection Reporting Initiation and Data Collection Reporting procedure can be initiated by the current serving node for LTM, or candidate node for LTM

Proposal 2: RAN3 to discuss on the applicability of the Data Collection Reporting Initiation and Data Collection Reporting procedures in context of LTM.

Proposal 3: RAN3 to discuss LTM-specific exit conditions for terminating the collection of UE performance and UE trajectory measurements.

Proposal 4: RAN3 to discuss the issue of providing feedback information to the gNB initiating the LTM preparation.

Observation 1: The amount of time during which the UE performance is collected is a useful indication of the goodness of the collected UE performance.

Proposal 5: RAN3 to discuss enhancements to the reporting of UE feedback.

# 4.	Appendix

In this section we provide a TP to TR 38.745.

<<<<< First change >>>>>>

# 4	Use cases and Solutions

<<<<< Omitting unmodified content >>>>>>

## 4.3	Other handover enhancements

Editor’s Note: Identify other handover enhancements via AI/ML, e.g., inter-CU LTM

Good handover performance is critical to mobile networks. Mobility and mobility-related enhancements have shaped the standard in so many ways. As new mobility features are introduced, e.g., LTM and variations of it (intra-CU, inter-CU, and conditional LTM), it is important to ensure that other existing frameworks which serve SON/MRO/AI/ML based mobility related use cases remain viable in light of the new functional additions to the standard.

As part of the discussions in the SI in Release 20, it has emerged that there are some limitations in obtaining data for AI/ML-based use cases by using the Data Collection Reporting Initiation and the Data Collection Reporting procedure in context of inter-CU LTM enhancements introduced as part of Release 19. The WI following the Release 20 SI shall consider investigating this further and specify normatively the required updates.

The Data Collection framework introduced in Rel-18 already allows NG-RAN nodes to collect UE feedback information after successful handover in order to optimize future mobility decisions. This framework may be further enhanced by providing richer UE feedback information, e.g., cause of data collection termination, amount of time during which the UE feedback was collected, which may enable improved mobility decisions.

<<<<< End of changes >>>>>>