3GPP TSG-RAN WG3 Meeting #129bis	R3-257093

Prague, Czech Republic, 13 – 17 October, 2025

Agenda item:	8.1

Source:	Nokia

Title:	UE Performance Metrics in Rel-18

Document for:	Decision

# 1	Introduction

In this meeting, RAN3 received a reply LS from SA5 related to per-UE UE Performance metrics in [1]. This LS was in response to an LS that RAN3 sent to SA5 in [2] to ask SA5 questions related to average packet delay and average packet loss measurements.

In particular, RAN3 had asked SA5 whether the UL and DL Average Packet Delay per-UE measurements in TS 28.558 clause 6.3.1.1 are aggregated per-DRB per-UE and are derived from per-DRB measurements across all configured DRBs for the concerned UE. RAN3 further had asked SA5 to specify those measurements if per UE aggregation was not supported but was, nevertheless, feasible. When it comes to average packet loss measurements, RAN3 had asked SA5 whether it was feasible to specify, both for split and non-split architecture, Packet Loss Rate metrics with a per-DRB per-UE granularity in the UL and DL at PDCP layer, which could be aggregated to reflect per-UE Packet Loss.

In their LS to RAN3 in [1] SA5 confirmed RAN3’s assumption that the measurements defined in TS 28.558 clause 6.3.1.1 related to Average Packet Delay are aggregated per-DRB per-UE to reflect corresponding per-UE metrics. When it comes to Average Packet Loss measurements, SA5 confirmed that it is feasible to specify the packet loss/drop rate measurements with a per-DRB per-UE granularity and that they have specified the corresponding packet loss and packet drop rate measurements in TS 28.558. In the CRs attached to the LS, a packet drop rate measurement is introduced in the DL (DL PDCP SDU Drop Rate in gNB-CU-UP) and a packet loss rate measurement is introduced in the UL (UL PDCP SDU Loss Rate).

In this paper, we provide our views on the reply LS from SA5.

# 2	Discussion

The UE Performance introduced by RAN3 to be exchanged over Xn is a Packet Loss Rate UE Performance metric. However, the metric introduced in the DL from SA5 is a Packet Drop Rate metric. As can be seen from SA5 specification TS 28.552, the packet drop rate and packet loss rate measurements are defined as follows:

From the above definitions, it is easy to see that the packet drop rate corresponds only to packets that were never sent to the target and therefore were dropped. Those packets are considered in the calculation of the packet loss rate, which additionally takes into account packets that were dropped. Therefore, packet loss rate seems to better suit the impacts in the UE Performance after a Handover event since it is more inclusive.

Observation 1: Packet Loss Rate is more suitable to reflect the impact of a Handover event to the UE Performance as opposed to Packet Drop Rate.

In addition, even though RAN3 requested SA5 to provide those metrics in a per UE granularity the reply from SA5 related to the Average Packet Loss measurements indicated the following: “SA5 confirms that it is feasible to specify the packet loss/drop measurements with a per-DRB per-UE granularity”, which indicates that those metrics are not aggregated per UE.

Observation 2: In their reply LS, SA5 confirmed that it is feasible to specify the packet loss/drop measurements with a per-DRB per-UE granularity without explicit indication that these measurements are aggregated per UE.

In our view, this is an aspect that needs further clarification with SA5 since the metrics exchanged over Xn are Performance metrics per UE. Therefore, we propose to send an LS to SA5 to ask them to clarify whether packet loss/drop rate measurements are per DRB per UE granularity or aggregated per UE.

Proposal 1: Send an LS to SA5 to ask them to clarify whether packet loss/drop rate measurements are per DRB per UE granularity or aggregated per UE.

Another aspect relates to the actual method of aggregation of per-DRB per-UE measurements into actual per UE measurements. During Rel-19, RAN3 discussed the aggregation method, and whether the per-UE UE Performance metrics are obtained as an average or weighted average of the per-DRB measurements. Finally, RAN3 decided that it would be better in terms of interoperability to let SA5 decide the aggregation method and RAN3 to refer to SA5 specifications. This decision was thought to be more appropriate also since SA5 is the right group to define metrics. However, after checking the CRs provided by SA5 for Rel-18 and Rel-19 related to DL PDCP SDU Drop Rate in gNB-CU-UP and UL PDCP SDU Loss Rate we observe that the aggregation method is not specified. Therefore, the introduced metrics in TS 28.558 do not support an interoperable solution. Even though we think that DL Average Packet Drop Rate is not useful for UE Performance evaluation, it would still be needed to specify the aggregation method to obtain this per-UE measurement for interoperability purposes.

Proposal 2: In the LS to SA5 further ask SA5 to specify the aggregation method used to obtain UE Performance metrics per-UE granularity (UL/DL Average Packet Delay, UL Average Packet Loss, DL Average Packet Drop) from the per-DRB per-UE measurements.

Another aspect relates to the Packet Loss Rate DL metric. Currently, in our specifications the semantics next to Packet Loss Rate DL IE indicate that it is defined according to a packet drop rate however there is a common understanding that “ The UL/DL Packet Loss IEs will be revised with a reference to an SA5 definition.”. In our view, we see two possible ways forward:

- Ask SA5 to introduce a PDCP level Packet Loss Rate DL measurement for UE Performance feedback, with a per UE measurement granularity.

- Remove Packet Drop Rate DL measurement and only consider Packet Loss Rate UL measurement.

Even though we would prefer to ask SA5 to introduce a PDCP level Packet Loss Rate DL, given that we are in correction phase of Rel-19 we think that the best way forward in order to avoid delays in the finalization of the Rel-19 work is to remove packet drop rate DL measurements from Rel-18 and Rel-19 RAN3 specifications.

Proposal 3: Remove Packet Loss Rate DL measurement, which corresponds to a DL PDCP SDU Drop Rate, from Rel-18 (and Rel-19) specifications.

We provide the corresponding Rel-18 CR in R3-257094.

# 3	Conclusion

In this paper we make the following observations and proposals:

Observation 1: Packet Loss Rate is more suitable to reflect the impact of a Handover event to the UE Performance as opposed to Packet Drop Rate.

Observation 2: In their reply LS, SA5 confirmed that it is feasible to specify the packet loss/drop measurements with a per-DRB per-UE granularity without explicit indication that these measurements are aggregated per UE.

Proposal 1: Send an LS to SA5 to ask them to clarify whether packet loss/drop measurements are per DRB per UE granularity or aggregated per UE.

Proposal 2: In the LS to SA5 further ask SA5 to specify the aggregation method used to obtain UE Performance metrics per-UE granularity (UL/DL Average Packet Delay, UL Average Packet Loss, DL Average Packet Drop) from the per-DRB per-UE measurements.

Proposal 3: Remove Packet Loss Rate DL measurement, which corresponds to a DL PDCP SDU Drop Rate, from Rel-18 (and Rel-19) specifications.

# References

[1]	R3-256518 – S5-253854, Reply LS on per-UE UE performance metrics, SA5

[2]	R3-253816 – S5-253295, Reply LS on per-UE UE performance metrics, RAN3