3GPP TSG-RAN WG3 Meeting #129-bis	R3-257127

Prague, Czech Republic, 13th ~17th Oct, 2025

Title:	CSI-RS Resource Set and CSI IM Resource Transfer for inter-CU LTM

Agenda Item:	9.2.5

Source:	Huawei, Jio Platforms, CATT, China Telecom, Lenovo, Nokia

Document for:	Discussion

- Introduction

The Rel-19 Mobility Enhancement WI was completed at last meeting. There are two IEs which are functionally essential for inter-CU LTM not discussed due to lack of time and are marked as “need to be refined” in ASN.1.

NZP-CSI-RS-ResourceSetConfiguration ::= SEQUENCE { -- This IE may need to be refined. cSI-RSResourceSetToAddModList OCTET STRING OPTIONAL, cSI-RSResourceSetToReleaseList OCTET STRING OPTIONAL, iE-Extensions ProtocolExtensionContainer { { NZP-CSI-RS-ResourceSetConfiguration-ExtIEs} } OPTIONAL, ... } NZP-CSI-RS-ResourceSetConfiguration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= { ... } CSI-IM-ResourceConfiguration ::= SEQUENCE { -- This IE may need to be refined. cSI-IMResourceToAddModList OCTET STRING OPTIONAL, cSI-IMResourceToReleaseList OCTET STRING OPTIONAL, cSI-IMResourceSetToAddModList OCTET STRING OPTIONAL, cSI-IMResourceSetToReleaseList OCTET STRING OPTIONAL, iE-Extensions ProtocolExtensionContainer { { CSI-IM-ResourceConfiguration-ExtIEs} } OPTIONAL, ... } CSI-IM-ResourceConfiguration-ExtIEs XNAP-PROTOCOL-EXTENSION ::= { ... }

This contribution provides further justifications on the transfer of the CSI-RS Resource Set Configuration and the CSI-IM Resource Configuration.

2	Discussion

From Rel-18 intra-CU LTM to Rel-19 inter-CU LTM, the LTM related low layer configurations are transferred over F1AP and Xn AP during LTM preparation as explicit IEs, to enable the (source) gNB-CU to generate the RRC LTM-Candidate IE. Those low layer configurations includes ltm-CandidateConfig-r18, ltm-EarlyUL-SyncConfig, and ltm-NZP-CSI-RS-ResourceToAddModList-r19, etc.

According to the latest RRC specification as quoted below, there are three IEs in the RRC LTM-Candidate IE are still missing in F1AP and XnAP:

- ltm-NZP-CSI-RS-ResourceSet

- ltm-CSI-IM-Resource

- ltm-CSI-IM-ResourceSet

#### – LTM-Candidate The IE LTM-Candidate concerns a LTM candidate configuration to add or modify. LTM-Candidate information element -- ASN1START -- TAG-LTM-CANDIDATE-START LTM-Candidate-r18 ::= SEQUENCE { ltm-CandidateId-r18 LTM-CandidateId-r18, ltm-CandidatePCI-r18 PhysCellId OPTIONAL, -- Need M ltm-SSB-Config-r18 LTM-SSB-Config-r18 OPTIONAL, -- Need M ltm-CandidateConfig-r18 OCTET STRING (CONTAINING RRCReconfiguration) OPTIONAL, -- Need M ltm-ConfigComplete-r18 ENUMERATED {true} OPTIONAL, -- Need R ltm-EarlyUL-SyncConfig-r18 OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18) OPTIONAL, -- Need R ltm-EarlyUL-SyncConfigSUL-r18 OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18) OPTIONAL, -- Need R ltm-TCI-Info-r18 LTM-TCI-Info-r18 OPTIONAL, -- Need M ltm-NoResetID-r18 INTEGER (1..maxNrofLTM-Configs-plus1-r18) OPTIONAL, -- Need M ltm-UE-MeasuredTA-ID-r18 INTEGER (1..maxNrofLTM-Configs-plus1-r18) OPTIONAL, -- Need M ..., [[ ltm-NoSecurityChangeID-r19 LTM-NoSecurityChangeId-r19 OPTIONAL, -- Need M ltm-ExecutionCondition-r19 SetupRelease {LTM-ExecutionConditionList-r19} OPTIONAL, -- Need M ltm-NZP-CSI-RS-ResourceToAddModList-r19 SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource OPTIONAL, -- Need N ltm-NZP-CSI-RS-ResourceToReleaseList-r19 SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-ResourceId OPTIONAL, -- Need N ltm-NZP-CSI-RS-ResourceSetToAddModList-r19 SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet OPTIONAL, -- Need N ltm-NZP-CSI-RS-ResourceSetToReleaseList-r19 SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSetId OPTIONAL, -- Need N ltm-CSI-ReportConfig-r19 LTM-CSI-ReportConfig-r18 OPTIONAL, -- Need N ltm-CSI-IM-ResourceToAddModList-r19 SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-Resource OPTIONAL, -- Need N ltm-CSI-IM-ResourceToReleaseList-r19 SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-ResourceId OPTIONAL, -- Need N ltm-CSI-IM-ResourceSetToAddModList-r19 SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets)) OF CSI-IM-ResourceSet OPTIONAL, -- Need N ltm-CSI-IM-ResourceSetToReleaseList-r19 SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets)) OF CSI-IM-ResourceSetId OPTIONAL -- Need N ]] }

It is noted that the ltm-NZP-CSI-RS-ResourceSet in the LTM-Candidate which is different from the ltm-NZP-CSI-RS-ResourceSet-r19 in the LTM-CSI-ResourceConfig IE. The ltm-NZP-CSI-RS-ResourceSet in the LTM-Candidate is generated by the candidate cell and further defines some parameters for a set of CSI-RS resources. Details are quoted below. While the ltm-NZP-CSI-RS-ResourceSet-r19 in the LTM-CSI-ResourceConfig IE is generated by the source gNB to associate the CSI-RS resource with the candidate Id.

#### – NZP-CSI-RS-ResourceSet The IE NZP-CSI-RS-ResourceSet is a set of Non-Zero-Power (NZP) CSI-RS resources (their IDs) and set-specific parameters. NZP-CSI-RS-ResourceSet information element -- ASN1START -- TAG-NZP-CSI-RS-RESOURCESET-START NZP-CSI-RS-ResourceSet ::= SEQUENCE { nzp-CSI-ResourceSetId NZP-CSI-RS-ResourceSetId, nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId, repetition ENUMERATED { on, off } OPTIONAL, -- Need S aperiodicTriggeringOffset INTEGER(0..6) OPTIONAL, -- Need S trs-Info ENUMERATED {true} OPTIONAL, -- Need R ..., [[ aperiodicTriggeringOffset-r16 INTEGER(0..31) OPTIONAL -- Need S ]], [[ pdc-Info-r17 ENUMERATED {true} OPTIONAL, -- Need R cmrGroupingAndPairing-r17 CMRGroupingAndPairing-r17 OPTIONAL, -- Need R aperiodicTriggeringOffset-r17 INTEGER (0..124) OPTIONAL, -- Need S aperiodicTriggeringOffsetL2-r17 INTEGER(0..31) OPTIONAL -- Need R ]], [[ resourceType-r18 ENUMERATED {periodic} OPTIONAL -- Cond LTM ]] } CMRGroupingAndPairing-r17 ::= SEQUENCE { nrofResourcesGroup1-r17 INTEGER (1..7), pair1OfNZP-CSI-RS-r17 NZP-CSI-RS-Pairing-r17 OPTIONAL, -- Need R pair2OfNZP-CSI-RS-r17 NZP-CSI-RS-Pairing-r17 OPTIONAL -- Need R } NZP-CSI-RS-Pairing-r17 ::= SEQUENCE { nzp-CSI-RS-ResourceId1-r17 INTEGER (1..7), nzp-CSI-RS-ResourceId2-r17 INTEGER (1..7) } -- TAG-NZP-CSI-RS-RESOURCESET-STOP -- ASN1STOP

The usage justification of those three IEs, ltm-NZP-CSI-RS-ResourceSet ltm-CSI-IM-Resource, and the ltm-CSI-IM-ResourceSet are provided in the LS from RAN1 in [1].

Furthermore, according to the RAN1 design in [1], the CSI-IM-Resource can be additionally configured for CSI-IM measurement when the CSI-RS resource for CSI acquisition is configured. There seems no need to have a separate request indicator from the source gNB for the CSI-IM resource. It is up to candidate gNB implementation whether to additionally provide the CSI-IM resource or not when the CSI-RS resource for CSI acquisition is requested and provided and taking the UE capability into account.

On the other hand, another option is to allow the source gNB to require the CSI-IM resources from the candidate gNB by an explicit indicator which is similar as the request for CSI-RS resources for other purpose. This is also workable because RAN1 has defined a UE capability where the UE can indicate support for CSI acquisition using only CSI-RS without requiring CSI-IM. Without it, baseline UEs may always need both CSI-RS resource and CSI-IM resource for CSI acquisition. Since the source gNB is aware of the UE capability, it can also indicate to the candidate gNB whether CSI-IM resource is required or not.

Following the same logic as the ltm-NZP-CSI-RS-ResourceSet, the candidate gNB should also generate the CSI-IM resource set in the LTM-Candidate IE and send it to the source gNB together with the CSI-IM resource.

#### – CSI-IM-ResourceSet The IE CSI-IM-ResourceSet is used to configure a set of one or more CSI Interference Management (IM) resources (their IDs) and set-specific parameters. CSI-IM-ResourceSet information element -- ASN1START -- TAG-CSI-IM-RESOURCESET-START CSI-IM-ResourceSet ::= SEQUENCE { csi-IM-ResourceSetId CSI-IM-ResourceSetId, csi-IM-Resources SEQUENCE (SIZE(1..maxNrofCSI-IM-ResourcesPerSet)) OF CSI-IM-ResourceId, ... } -- TAG-CSI-IM-RESOURCESET-STOP -- ASN1STOP

Therefore, it is proposed:

- To introduce the three IEs, ltm-NZP-CSI-RS-ResourceSet ltm-CSI-IM-Resource, and ltm-CSI-IM-ResourceSet into LTM preparation related procedures in F1AP and XnAP.

- RAN3 to discuss the following two options to provide the CSI-IM resource

      - Option 1: the source gNB requires the candidate gNB to provide CSI-IM resource by an explicit indicator

      - Option 2:  the candidate gNB decides whether to provide the CSI-IM resource to the source gNB by implementation

- Removal of the “to Release List” IEs

In the latest XnAP and F1AP specifications, the “to Release List” IEs for CSI-RS resource and CSI-IM resource are introduced.

However, such kind of IEs are derived from RRC protocol. Technically, any release of CSI-RS resource and CSI-IM resource over Uu interface is regarded as a kind of LTM modification in RAN3. RAN3 has agreed in the WI phase not to support LTM modification for LTM DC cases.

“Do not support LTM modification on the already prepared LTM candidate PScells in DC scenario in Rel-19.”

And unfortunately, LTM modification (neither source gNB initiated, nor candidate gNB initiated) was never discussed for inter-CU LTM without SCG case in the WI phase.

RAN3 may:

- either confirm the not support of LTM modification for inter-CU LTM, which leads to the removal of the “to Release List IEs” , or

- agree to support LTM modification  for inter-CU LTM and clarify the scenario and stage 2 and stage 3 functional impact. For example, source gNB initiated LTM modification can be supported by triggering multiple times of the LTM Configuration Update procedure before LTM cell switch command.

- RAN3 to discuss the following two options on the LTM modification for inter-CU LTM

      - Option 1: LTM modification is not supported for inter-CU LTM in Rel-19

      - Option 2: LTM modification is supported for inter-CU LTM in Rel-19 and continue discussion on the detailed stage 2 and stage 3 functional impact

- If option 1 is agreed, to remove the “to Release List” IEs for CSI-RS resource and CSI-IM resource in Xn and F1 specifications.

- To update F1AP and XnAP accordingly.

3	Conclusion

This document proposes:

Proposal 1:	To introduce the three IEs, ltm-NZP-CSI-RS-ResourceSet ltm-CSI-IM-Resource, and ltm-CSI-IM-ResourceSet into LTM preparation related procedures in F1AP and XnAP.

Proposal 2:	RAN3 to discuss the following two options to provide the CSI-IM resource

-	Option 1: the source gNB requires the candidate gNB to provide CSI-IM resource by an explicit indicator

-	Option 2:  the candidate gNB decides whether to provide the CSI-IM resource to the source gNB by implementation

Proposal 3:	RAN3 to discuss the following two options on the LTM modification for inter-CU LTM

-	Option 1: LTM modification is not supported for inter-CU LTM in Rel-19

-	Option 2: LTM modification is supported for inter-CU LTM in Rel-19 and continue discussion on the detailed stage 2 and stage 3 functional impact

Proposal 4:	If option 1 is agreed, to remove the “to Release List” IEs for CSI-RS resource and CSI-IM resource in Xn and F1 specifications.

Proposal 5:	To update F1AP and XnAP accordingly.

The XnAP CR and F1AP CR are provided in [2] and [3].

Reference

[1] R1-2503242 	DRAFT LS on Rel-19 higher layers parameters list Post RAN1#121

[2] R3-257128	Correction on CSI-RS Resource Set and CSI IM Resource Transfer for inter-CU LTM

[3] R3-257129	Correction on CSI-RS Resource Set and CSI IM Resource Transfer for inter-CU LTM