3GPP TSG RAN WG3 Meeting #130	R3-258008

Dallas, USA, 17 - 21 November, 2025

# 3GPP TSG-SA WG2 Meeting #171	S2-2509834

Wuhan, China, 13 - 17 October 2025	(Revision of S2-2509810)

Title:	Reply LS on allocation of CN assigned subgroup ID for LP-WUS

Response to:	S2-2508191/R3-255941

Release:	Rel-19

# Work Item:	     NR_LPWUS-Core

Source:	SA2

To:	RAN3, RAN2

Cc:	RAN1

Contact Person:	Qian Chen

E-mail Address:	qian (dot) xb (dot) chen (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA2 thanks RAN3 for the LS (S2-2508191/R3-255941) on allocation of CN assigned subgroup ID for LP-WUS.

The LS highlights the following:

# Overall description

Regarding the range of CN assigned subgroup ID for LP-WUS, RAN3 made the following agreements:

- The range of CN assigned subgroup ID in NG/Xn/F1 message is 0.. 30.

This agreement means that when the AMF allocates CN assigned subgroup ID for the UE, it should consider the least range of subgroup ID available over NG-RAN cells corresponding to the UE registration area.

RAN3 would like to provide following background information: according to the TS 38.213, for CN assigned subgroup ID range, dependent on the per-cell PO (Paging Occasion) to LO (LP-WUS Occasion) association:

- in case PO-to-LO association is 4, subgroup ID #0 to #6 can be used;.

- in case PO-to-LO association is 2, subgroup ID #0 to #14 can be used;

- in case PO-to-LO association is 1, subgroup ID #0 to #30 can be used;

RAN3 would like to ask SA2 to take the above into consideration, and discuss the potential specification impact, if any.

# Actions

To SA2:

ACTION: RAN3 would like to ask SA2 to take the above into consideration, and discuss potential specification impact, if any.

SA2 discussed the issue and would like to receive some further information from RAN3 and RAN2:

a) 	What is the typical area that a PO-to-LO association configuration will apply to? (e.g., is the configuration uniform in a PLMN, different in every cell, etc, and is it mandatory for an LP-WUS capable gNB to support all 3 PO-to-LO association configurations?) This information will help SA2 to understand what kind of solution to propose.

b) 	Whether the paging PO-to-LO association configuration will be changed often, e.g., to save base station energy when the paging load is less?

c) 	Will RAN3 specify the relation between subgroup ID value range and the PO-to-LO association (e.g. based on RAN1 specification)? This could avoid SA2 having to refer to a complex part of TS 38.213.

SA2 would appreciate replies to the above questions.

2. Actions:

To RAN3, RAN2

ACTION: 	SA2 kindly asks RAN3 to provide answers for a, c) and RAN2 to provide answers for b).

3. Dates of Next SA2 Meetings:

SA2#172   				17 – 21 November, 2025			         Dallas, US

SA2#173   				09 – 13 February, 2026			              tbd, IN