3GPP TSG-RAN WG3 Meeting #130	R3-258051

Dallas, US, 17th ~21th November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 1321 | Rev | 2 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on A-IoT leftovers on IE details |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, CMCC, China Unicom, China Telecom, Nokia, LG Electronics, CATT, NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ambient_IoT_Solutions-Core |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As discussed in RAN3#129, the details of several IEs may need further discussion, and there are comment temporarily added in the asn.1 part. - based on CT4 agreed CR C4-253537, RAN3 needs to do the following updates: - update the encoding of A-IoT Correlation Identifier IE from OCTET STRING to INTEGER (0..65535, ...), and remove the related comment; - update the encoding of AIOTF Identifier IE from OCTET STRING to OCTET STRING (SIZE(16)), and remove the related comment; - about the encoding of Time Interval IE” - it is defined as INTEGER (1..256, ...) Unit: 0.1 second, it seems to be a reasonable design and no strong need to update; and it is needed to add reference to TS 23.369 for this IE. - about the encoding of A-IoT Device Identification Requested IE: - current encoding of both the single device inventory branch and the group device inventory branch, is BIT STRING (SIZE(1..1024, ...)), we see no strong motivation to update them. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Do the updates as below: - update the encoding of A-IoT Correlation Identifier IE from OCTET STRING to INTEGER (0..65535, ...), and remove the related comment; - update the encoding of AIOTF Identifier IE from OCTET STRING to OCTET STRING (SIZE(16)), and remove the related comment; - keep current encoding of Time Interval IE, remove the comment related, add reference to TS 23.369 for this IE; - keep the current encoding of the A-IoT Device Identification Requested IE, and remove the related comment; Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because it only updates some details for A-IoT newly introduced IEs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Leftovers remain for the A-IoT related IEs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.3.69, 9.3.3.70, 9.3.3.72, 9.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: undo the changes for the Inventory Report Transfer IE. Further update the encoding of AIOTF Identifier IE to OCTET STRING (SIZE(32)) instead of size 16. Rev2: resubmission. |  |  |  |  |  |  |  |  |

-----------------Start of the First Change-----------------

#### 9.3.3.69	A-IoT Correlation Identifier

This IE uniquely identifies an A-IoT session within an AIOTF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| A-IoT Correlation Identifier | M |  | INTEGER (0..65535, ...) |  |

#### 9.3.3.70	AIOTF Identifier

This IE uniquely identifies the AIOTF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| AIOTF Identifier | M |  | OCTET STRING (SIZE(32)) | Coded as the NfInstanceId IE defined in TS 29.571 [35]. |

-----------------Start of the Next Change-----------------

#### 9.3.3.72	Inventory Assistance Information

This IE includes the assistance information for the A-IoT Inventory Service.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Expected D2R Message Size | M |  | INTEGER (1..256,…) | Unit: Byte. This IE indicates the expected size of the A-IoT NAS PDU carried in Inventory Report. |
| Approximate Number of Target A-IoT Devices | O |  | INTEGER (1..65535, …) |  |
| Time Interval | O |  | INTEGER (1..256, ...) | Unit: 0.1 second. This IE indicates the time interval as defined in TS 23.369 [60]. |

-----------------Start of the Next Change-----------------

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

//skip unchanged part

AIoTAreaID ::= SEQUENCE {

pLMNidentity		PLMNIdentity,

nID					NID							OPTIONAL,

aIoTAreaCode		OCTET STRING (SIZE (3)),

iE-Extensions		ProtocolExtensionContainer { { AIoTAreaID-ExtIEs} } OPTIONAL,

...

}

AIoTAreaID-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AIoT-CorrelationIdentifier ::= INTEGER (0..65535, ...)

AIoT-FollowonCommandIndication ::= ENUMERATED {true, ...}

RAN-AIOT-Device-NGAP-ID ::= INTEGER (0..4294967295)

AIoT-CommandAssistanceInformation ::= SEQUENCE {

estimateofExpectedD2RMsgSize		INTEGER (1..256, ...)					OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { AIoT-CommandAssistanceInformation-ExtIEs} } OPTIONAL,

...

}

AIoT-CommandAssistanceInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AIoT-DeviceIdentificationRequested ::= CHOICE {

singleDevice					BIT STRING (SIZE(1..1024, ...)),

groupDevices					BIT STRING (SIZE(1..1024, ...)),

allDevices						NULL,

choice-Extensions		ProtocolIE-SingleContainer { { AIoT-DeviceIdentificationRequested-ExtIEs} }

}

AIoT-DeviceIdentificationRequested-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

AIoT-DeviceReportList ::= SEQUENCE (SIZE(1..maxnoofDevices)) OF AIoT-DeviceReportItem

AIoT-DeviceReportItem ::= SEQUENCE {

aIoT-NASPDU					AIoT-NASPDU,

rAN-AIOT-Device-NGAP-ID		RAN-AIOT-Device-NGAP-ID									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { AIoT-DeviceReportItem-ExtIEs} }	OPTIONAL,

...

}

AIoT-DeviceReportItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AIOTFIdentifier ::= OCTET STRING(SIZE (32))

AIOTFName ::= SEQUENCE {

aIOTFNameVisibleString		AIOTFNameVisibleString				OPTIONAL,

aIOTFNameUTF8String			AIOTFNameUTF8String					OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { AIOTFName-ExtIEs } } OPTIONAL,

...

}

AIOTFName-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AIOTFNameVisibleString ::= VisibleString (SIZE(1..150, ...))

AIOTFNameUTF8String ::= UTF8String (SIZE(1..150, ...))

AIoT-InventoryAssistanceInformation ::= SEQUENCE {

expectedD2RMsgSize							INTEGER (1..256, ...),

approximateNoofTargetDevices				INTEGER (1..65535, ...)				OPTIONAL,

timeInterval								INTEGER (1..256, ...) 				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { AIoT-InventoryAssistanceInformation-ExtIEs} } OPTIONAL,

...

}

AIoT-InventoryAssistanceInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

-----------------End of the Changes-----------------