3GPP TSG-RAN WG3 Meeting #130	R3-258088

Dallas, US, 17 – 21 November 2025

Title:	(pCR to TR 38.765) Study on Integrated Sensing And Communication (ISAC) for NR

Agenda Item:	13.1

Source:	China Telecom, Xiaomi

Document for:	pCR

1	Introduction

This contribution provides the baseline pCR to the TR 38.765[3] from RAN3 perspective, captures the agreed TPs listed in [1] ~ [2].

2	Reference

- R3-257325, (TP to BL pCR to 38.765) Network Architecture, ZTE Corporation, Huawei, Xiaomi, Nokia, Nokia Shanghai Bell, NEC, Ericsson, China Telecom, Samsung, LG Electronics, CATT, InterDigital.

- R3-257326, (TP to BL pCR to 38.765) Basic sensing call flow, Xiaomi, Huawei, Samsung, Nokia, Nokia Shanghai Bell, ZTE, Ericsson, China Telecom, LG Electronics, NEC, OPPO, InterDigital, Telecom Italia, CATT, CMCC.

- R1-2507422, Updated TR skeleton for study on Integrated Sensing And Communication (ISAC) for NR			Xiaomi, China Telecom.

3		pCR to TR 38.765

<<<<<<<<<<<<<<<<<<<< Change Begins >>>>>>>>>>>>>>>>>>>>

7	Network architecture

Editor’s note: This section is to capture the study outcome of network architecture. Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. No inter-gNB coordination will be studied.

This clause identifies and describes the logical architecture to support the sensing in the overall 5G system architecture.

Figure 7.1 depicts a logical architecture for ISAC, where the Nx interface is between the gNB and the SF.

Figure 7.1	Logical architecture for ISAC

This logical architecture is independent of the transport, e.g. direct or via the AMF, between the gNB and the SF.

Editor’s Note:  The details of "Nx" interface need further discussion and decision.

8	RAN-CN procedures and signalling

Editor’s note: This section is to capture the study outcome of procedures and signaling aspects between RAN and CN for gNB-based monostatic sensing.

8.x	Sensing Reporting

Editor’s Note: the following may need further refinement.

Figure 8.x-1: Message flow for sensing reporting

1.	The SF sends sensing request to the gNB.

2.	The gNB sends sensing response to the SF.

3.	If requested in Step 1, the gNB sends sensing report to the SF.

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>