3GPP TSG-RAN WG3 #130		 R3-258113

Dallas, USA, November 17 - 21, 2025

Agenda Item: 	10.4

Source:	Boost Mobile Network

Title:	Control-plane latency and message overhead in split RAN architectures

Document for:	Discussion & Decision

# 1. Introduction

In RAN3#129bis meeting, 6G RAN internal functional split was discussed and following is noted by Chair to trigger further discussion.

Capture pain points and benefits (and potentially derived requirements) of 5G HLS To be continued...

This contribution addresses two technical issues, control plane latency and message overhead, which some believe are pain points with the 5G RAN split architecture.

# 2. Discussion

5G RAN split architecture provides flexibility and scalability in RAN system implementation and deployment. However, there are concerns that it increases control plane latency and message overhead. It is necessary to study the extent to which the split architecture impacts the control plane latency and message overhead.

## 2.1 Control Plane Latency

The split architecture enables diverse deployment scenarios that can meet the varying requirements of operators with different network evolution strategies and business models. In this contribution, representative deployment scenarios for macro mobile networks, covering both distributed and centralized RANs, are listed and analyzed. These example scenarios provide insight into the latency impacts and potential approaches for managing it, without addressing other deployment scenarios such as NTN, NPN, or Small Cells.

Most mobile networks so far adopted a distributed RAN architecture as shown in Figure 1, where all the RAN components including RU, DU, CU-CP, and CU-UP are deployed at each remote site. In practice, the DU, CU-CP, and CU-UP can be co-located and integrated within a single high-performance computing server. Consequently, the interaction between CU and DU occurs internally within the same hardware server, achieving latency on the order of sub-microseconds, which is negligible when compared with the long one-way latency of approximately 10ms typically observed between CU and Core. Therefore, in a distributed RAN configuration, control-plane latency is insignificant and does not pose any performance concern. It should be noted that backhaul latency between the CU (or gNB) and the Core varies depending on the operator’s network. In this paper, only the propagation delay of 5us per kilometer is considered.

Figure 1. Distributed RAN

Figure 2 shows a representative C-RAN configuration, in which RU is deployed at each remote site, while other RAN components including the DU, CU-CP, and CU-UP are centralized in a data center for pooling of computing resources, minimizing the footprint of remote sites, enhancing system resiliency, maintainability, and cloud-native system operation. Typically, the data center is located within 20 to 30 kilometers of the remote sites to support real-time scheduling. A data center houses computing servers stacked in racks, interconnected through ultra-high-speed optical cables ranging from 100 Gbps to 800 Gbps depending on the design of the data center. This enables horizontal message exchanges to occur within a few microseconds. Consequently, the CU-CP and DU can communicate with latency on the order of a few microseconds, which is negligible when compared with the one-way backhaul latency of approximately 10ms typically observed between CU and Core.

Figure 2. Centralized RAN

Observation 1: The additional control-plane latency introduced by the split architecture is not significant in typical D-RAN and C-RAN deployment scenarios.

Figure 3 illustrates a deployment scenario in which non-real-time components such as the CU-CP and CU-UP are centralized in a cloudified data center – assuming a coverage radius of approximately 50km for a metropolitan area – to maximize computing resource pooling and enhance manageability. In practice, the distance does not need to be limited to 50km; the CU can be located in a more centralized site to serve a larger number of DUs. The data center is well suited for computation-intensive applications, including AI/ML data processing and model training. In this deployment scenario, message delivery between the CU and DU takes approximately 0.25ms, assuming a propagation delay of 5 microseconds per kilometer. The processing and transmission delays of intermediate nodes are considered negligible when compared with the propagation delay between the network elements. If the CU and DU are required to exchange messages a couple of times for a single task, the total additional delay may reach 0.5~1ms. Although this accounts for less than 10% of the one-way backhaul delay, it may still have a measurable impact on certain control-plane KPIs, such as RRC Connection Setup Time. This worst case occurs only for UEs connected to base stations located at the edge of the data center’s coverage area or at the boundary of the metropolitan area, depending on the data center’s location. In addition, it should be noted that there is no impact on user-plane performance because such message exchanges between the CU and DU do not occur during user data transmission. Furthermore, control-plane latency has limited impact on UE QoE because such signalling occurs infrequently from the UE’s perspective or operates in the background while the UE remains connected.

Figure 3. Centralized CU

The control-plane latency can be reduced by moving the CU-CP function to the DU server for co-location as illustrated in Figure 4. However, this configuration increases the control-plane latency between the CU-CP and CU-UP. Nevertheless, message exchanges between the CU-CP and CU-UP occur far less frequently from an individual user’s perspective compared with those between the DU and CU-CP.

Figure 4. Centralized CU-UP

Meanwhile, as illustrated in Figure 5, operators with fewer concerns about control-plane latency but aiming to reduce use-plane latency for edge applications may choose to co-locate the CU-UP with the DU, while centralizing the CU-CP in a data center. A centralized CU-CP offers advantages in coordinating DUs across a wide area for purposes such as load balancing and energy saving. It also helps enhance the QoE of UEs by reducing the frequency with which they transition to the IDLE state. Maintaining UEs in INACTIVE states keeps the AMF less busy and effectively offloads its processing load.

Figure 5. Centralized CU-CP

Observation 2: Control-plane latency becomes measurable when the CU-CP and DU are separated by long-haul distances.

Observation 3: The RAN split architecture enables operators to flexibly design their networks depending on their operational requirements.

## 2.2 Message Overhead

There is a concern that the split architecture increases message overhead due to the introduction of the F1 and E1 interfaces through which the DU, CU-CP, and CU-UP exchange messages. It is true and inevitable that the number of messages observed over the network increases when a monolithic network function is split into multiple functional entities. However, in exchange, the split architecture provides system scalability and flexibility across implementation, deployment, maintenance, and evolution. This should be regarded as a trade-off between the increased number of messages and the various benefits described above. In practice, the message overhead is not significant compared with the volume of user traffic flowing over the same physical network. Furthermore, data centers are specifically designed with high-bandwidth transport capabilities to support horizontal communication between software modules that are containerized or virtualized. If some operators remain concerned about the number of messages exchanged over the network, they can choose to co-locate the functions on a single hardware server, which is an option that the split architecture provides as one of its key benefits.

Observation 4: Introducing internal interfaces such as F1 and E1 increases the number of messages exchanged over the network. However, this is a trade-off in exchange for improved system scalability and flexibility.

## 2.3 Single RAN Architecture for 6G

As illustrated by the examples above, the split architecture enables flexible deployment by allowing functional entities to be positioned within the network according to specific performance requirements or operator preferences. Furthermore, once the interfaces and procedures are standardized based on the split architecture, there will be no constraint on implementing the system either as an integrated single entity or as a set of independent functional entities. Therefore, it is proposed that 6G RAN adopt the split architecture as a unified RAN framework.

Proposal 1. 6G RAN shall evolve the 5G HLS and CP-UP split architecture into a unified RAN framework.

# 3. Conclusion

Based on the above discussion, the following observations and proposals are derived:

Observation 1: The additional control-plane latency introduced by the split architecture is not significant in typical D-RAN and C-RAN deployment scenarios.

Observation 2: Control-plane latency becomes measurable when the CU-CP and DU are separated by long-haul distances.

Observation 3: The RAN split architecture enables operators to flexibly design their networks depending on their operational requirements.

Observation 4: Introducing internal interfaces such as F1 and E1 increases the number of messages exchanged over the network. However, this is a trade-off in exchange for improved system scalability and flexibility.

Proposal 1: 6G RAN shall evolve the 5G HLS and CP-UP split architecture into a unified RAN framework.

# Reference

[1] RP-252912: New SID: Study on 6G Radio (RAN#109)