3GPP TSG-RAN WG3 Meeting #130	R3-258117

Dallas, US, 17-21 Nov, 2025

Title: 	(TP for pCR) RAN-CN procedures and signaling for ISAC

Source: 	Huawei

Agenda item:	13.3

Document Type:	other

# 1. Introduction

The new study item on Study on Integrated Sensing and Communication (ISAC) for NR was agreed in [1]. For RAN-CN procedures and signalling, the previous RAN3-129bis meeting agreed the general sensing report procedures in [2], and had the following point to be continued.

To continue the discussion on the necessary interface functions and the detailed signaling procedures for ISAC.

This contribution provides our further views on the procedures and signalling for gNB-based mono-static for UAV use case, by taking latest SA2 agreements into account.

# 2. Discussion

## 2.1 SA2 progress on the signalling and procedures

In the past SA2 meetings, SA2 identified overall six key issues to be solved in TR 23.700-14 in [3]. There are two key issues related to the RAN3 procedures and signalling between RAN and CN:

- Key Issue #4: Sensing Data and the Associated Information Collection and Transport

- Key Issue #6: Configuration of Parameters for Sensing Entities

The previous SA2-171 meeting agreed the following principles on the KI#4.

### 7.1.4 Agreed Principles for KI#4: Sensing Data and the Associated Information Collection and Transport The following principles are agreed for KI#4: - Based on the received sensing service requirements from the Sensing Function, the Sensing Entity (gNB) can determine how to perform the relevant sensing measurement to collect the Sensing data. - Sensing Entity can support one time, periodical Sensing Data to the Sensing Function. Editor's note: The type (one time, periodical, etc.) and content of Sensing Data that will be provided by gNB needs to be coordinated with RAN WGs in the normative phase. - Sensing data can be collected from one or multiple gNBs by Sensing Function for the same sensing service request. - The Sensing Function can generate sensing results based on the information provided by the Sensing Entity (gNB) node(s). NOTE: The architecture aspect of this KI's interim conclusions will be aligned with KI#1 conclusions.

As observed above, SA2 agreed that the gNB can send the one time, periodical sensing data to the SF, based on the sensing service requirements from the SF though details need to be coordinated with RAN. And the SF can generate the sensing results based on the report from the gNB.

Observation 1:  In the SA2 key issue#4, SA2 agreed that the gNB can send the one time, periodical sensing data to the SF, based on the sensing service requirements from the SF though details need to be coordinated with RAN. And the SF can generate the sensing results based on the report from the gNB.

## 2.2 The RAN-CN signalling procedure

In the RAN3 agreed pCR [2], the sensing report procedure is agreed as follows.

## 8.x Sensing Reporting Editor’s Note: the following may need further refinement. Figure 8.x-1: Message flow for sensing reporting 1. The SF sends sensing request to the gNB. 2. The gNB sends sensing response to the SF. 3. If requested in Step 1, the gNB sends sensing report to the SF.

This section is called as Sensing Reporting. From the above figure, this message flow not only includes the sensing reporting, but also includes the sensing initiation related messages. Hence, we propose to rename the section to Sensing Service Initiation, so that to include all related steps for sensing service.

- Rename section 8.x from Sensing Report to Sensing Service Initiation.

### 2.2.1 The interface setup procedure

In TR 23.700-14 [3], e.g., some solutions (#4, #6, #15, #16, #36) discuss the interaction between gNB and SF for sensing capability, which is assumed that each gNB will register its sensing capability at the (authorizing) SF. For example, the gNB may register its location or supported sensing service area and supported sensing mode and its role (as sensing transmitter, sensing receiver, or both).

From our understanding, there is no need to signal the static gNB sensing related capability parameters (e.g. supported sensing mode and its role) to the SF. First there is no need to indicate the detailed sensing related capability, e.g., the supporting sensing mode (only gNB mono-static sensing supported) or its role. Second, these static parameters should be managed and configured by the operator (e.g., via OAM) and not changed dynamically.

- No need to indicate the static sensing related capability from the gNB to the SF.

On the other hand, the gNB should setup the interfaces with the SF for transfer application layer information for an NG-C interface instance. For direct connection, the gNB needs to establish the interface with SF directly, while for indirect connection, the interface is already defined between the gNB and AMF, then between the AMF and the SF. Note that the direct/indirect connection would be under SA2 scope.

Basically, it is natural to reuse the NG-interface between the gNB and the SF. And the NG interface management message can be reused to indicate e.g., the TAC list, and PLMN list etc, while other information can be further discussed.

- RAN3 to update the message flow, adding the interface setup procedure, where the application layer information (e.g., the TAC list, PLMN list etc) can be indicated from the gNB to the SF.

### 2.2.2 The Sensing Request procedure and Sensing Report procedure

After the interface setup procedure, the sensing procedure for the basic gNB-based mono-static sensing service can be summarized as two main phases, which was captured in the pCR[2].

- Phase 1: the SF triggers the sensing procedure towards the gNB

In [2], this message is named as Sensing Request message. So far in [3] there are different names of the procedure or message, e.g., Sensing Request/Response, Sensing Service Request/Response, Sensing Subscribe Request/Response, Sensing Configuration Request, Sensing Active Request, Sensing Session Establishment, Sensing Task Assignment etc.

From RAN3 point of view, we think the class 1 procedure can be named as Sensing Service Request procedure with SENSING SERVICE REQUEST/RESPONSE/FAILURE messages.

- Rename the Sensing Request procedure as Sensing Service Request procedure in the message flow, which includes SENSING SERVICE REQUEST/RESPONSE/FAILURE messages.

The request message can include at least the sensing area information for which we considering the following options to be decided together with SA2:

-	Sensing cell. For each gNB, it may serve several Sensing cells;

-	Sensing Tracking Area. It can be across gNB(s) and one or more gNB can belong to the same Tracking Area.

The example of the Sensing Tracking Area and Sensing cell is given in the following Figure. And also, other alternatives for sensing area can be further discussed, such as geographical area information, or civic address information.

And the report characteristics (e.g., one-time, periodicity) could be also included in the request message as well.

- In the SENSING SERVICE REQUEST message, include at least the sensing area info (e.g., the TAC list, Cell list etc), report characteristics (e.g., one-time, periodicity).

- Phase 2: the gNB transmits the sensing data

For the phase 2 sensing data transmission, it is noticed that there are two different options on how to transmit the sensing data in TR 23.700-14 [3]:

  - Option 1: CP solution - the gNB transmits the sensing data via the control plane.

  - Option 2: UP solution - the gNB transmits the sensing data over GTP-U tunnel.

This issue should be cooperated with SA2 and RAN1 to decide how to transmit the sensing data. From RAN3 point of view, it requires to discuss and compare these solutions in terms of different comparison metrics, e.g., data size limitation, latency and complexity etc when the sensing measurement results format is determined.

If the CP signaling is used to transmit the sensing data, it needs to consider the maximum data size (16K Bytes) under the limitation of the DTLS over SCTP in [4] as follows.

‘DTLS limits the DTLS user message size to the current Path MTU minus the header sizes. For the purposes of running over SCTP, the DTLS path MTU MUST be considered to be 2^14.’

Also as described in 3GPP TS 33.501, the DTLS over SCTP shall be supported for N2, Xn, F1 and E1interfaces. For example, the following is depicted for N2 interface in TS 33.501 [2].

## 9.2 Security mechanisms for the N2 interface <Skip the irrelevance> In addition to IPsec, DTLS shall be supported as specified in RFC 6083 [58] to provide mutual authentication, integrity protection, replay protection and confidentiality protection. Security profiles for DTLS implementation and usage shall follow the TLS profile given in clause 6.2 of TS 33.210 [3] and the certificate profile given in clause 6.1.3a of TS 33.310 [5]. The identities in the end entity certificates shall be used for authentication and policy checks.

While for option2, there is no data size limitation over GTP-U.

In general, we consider that Option2 UP solution is advantageous to transmit the sensing data, considering that the user plane tunnel is more suitable for transmitting large volumes of sensing data and future-proof.

- For the sensing data transfer, RAN3 to discuss and compare the CP and UP solutions in terms of data size, latency and gNB complexity etc. Cooperation with RAN1/SA2 is needed.

About the exact sensing measurement results, the previous RAN1-121 meeting studied the following options in [5]:

### [FL5][H] Proposal 7.1-1-rev3 - The following measurements are identified in the study of NR ISAC. - Level 1: raw data, which is the time-domain samples or subcarrier-domain samples of the channel response from each antenna port, or anything equivalent - Level 2: (partial raw data) - Amplitude and phase profile of delay, and/or Doppler, and/or angle - E.g., Amplitude and phase profile of delay-angle - Level 3 per path (subset of Level 2): - Option 1: Delay, Doppler, angle, and power per detected path for a given time stamp - Definition: one path is associated with one delay, and one or multiple Doppler, and one or multiple angles - A path can correspond to one or multiple detected points - A path can correspond to one or multiple detected objects - Level 4 per point: - Option 1: Range, velocity, angle, and power per detected point for a given time stamp - Definition: A point is defined by one range/delay, one Doppler and one angle - Option 2: Position, velocity, and power per detected point for a given time stamp - Definition: A point is defined by one location and one velocity - Level 5: e.g., {position, velocity, no target ID} - Definition: Position and velocity for a detected object/target for a given time stamp - One value pair {Position, velocity} for a given time stamp is reported for a detected object/target - The association of multiple measurements of {Position, velocity} across different time stamps for the same detected object/target is not reported - Level 6: e.g., {position, velocity, target ID} - Definition: Position and velocity for a detected object/target for a given time stamp - One value pair {Position, velocity} for a given time stamp is reported for a detected object/target - The association of multiple measurements of {Position, velocity} across different time stamps for the same detected object/target is reported - SINR, RSRP from perspective of sensing - Time stamp - Study the following levels of measurements for potential down-selection within Rel-20 SI - Level 2 - Level 3 - Level 4 - Level 5 - Level 6 - SINR, RSRP from perspective of sensing

From RAN3’s perspective, the exact sensing measurements are pending to RAN1 evaluation and quantization. Generally, we consider that the intermediate sensing result per detected point (i.e., cloud point) is very advantageous especially compared with the final sensing results due to the following reasons:

- relatively lower transmission overhead while still effectively capturing the original characteristics of the measurements;

- allows the SF to have precise sensing calculation when considering more information sources (e.g., non-3GPP sensing data, the future UE-provided data), which is not possible when the gNB provides final sensing result directly;

- No significant gNB upgradation to support future sensing types in a large quantitively way while SF upgradation would be much simpler.

And the point definition in TS 23.032 can be taken as starting point, which has already captured the format of various measurements including range, direction, velocity and uncertainty. Typically, the following format in TS 23.032 can be treated as starting point:

- 5.10	Range and Direction

- 8.4a	Relative Velocity with Uncertainty

- For the sensing data transfer, the cloud points can be reported referring to TS 23.032, while the final evaluation on the sensing measurement, and quantization is up to RAN1.

### 2.2.3 The Sensing Service Stop procedure

In addition to the sensing initiation procedure, RAN3 should also discuss the sensing service stop. It seems natural for the SF to trigger the sensing service stop procedure, e.g., when the SF can decide to terminate the periodical sensing report after other CN NFs stop the sensing service registration. This could be defined as class 1 procedure.

In addition, the gNB could also decide to stop the procedure and notify the SF that the previously requested sensing service request can no longer be reported. Upon reception, the SF can consider that the sensing request has been terminated by the gNB. This could be defined as class 2 procedure.

- RAN3 to agree the class 1 sensing service stop procedure initiated by the SF, and the class2 sensing failure indication procedure initiated by the gNB.

# 3. Conclusion

Based on the discussion in this paper, we make the following observations and proposals.

SA2 progress on the signalling and procedures

Observation 1:  In the SA2 key issue#4, SA2 agreed that the gNB can send the one time, periodical sensing data to the SF, based on the sensing service requirements from the SF though details need to be coordinated with RAN. And the SF can generate the sensing results based on the report from the gNB.

The RAN-CN signalling procedure

- Rename section 8.x from Sensing Report to Sensing Service Initiation.

- No need to indicate the static sensing related capability from the gNB to the SF.

- RAN3 to update the message flow, adding the interface setup procedure, where the application layer information (e.g., the TAC list, PLMN list etc) can be indicated from the gNB to the SF.

- Rename the Sensing Request procedure as Sensing Service Request procedure in the message flow, which includes SENSING SERVICE REQUEST/RESPONSE/FAILURE messages.

- In the SENSING SERVICE REQUEST message, include at least the sensing area info (e.g., the TAC list, Cell list etc), report characteristics (e.g., one-time, periodicity).

- For the sensing data transfer, RAN3 to discuss and compare the CP and UP solutions in terms of data size, latency and gNB complexity etc. Cooperation with RAN1/SA2 is needed.

- For the sensing data transfer, the cloud points can be reported referring to TS 23.032, while the final evaluation on the sensing measurement, and quantization is up to RAN1.

- RAN3 to agree the class 1 sensing service stop procedure initiated by the SF, and the class2 sensing failure indication procedure initiated by the gNB.

The Annex 5 provides the TP on the RAN-CN procedures and signalling for the sensing.

# 4. Reference

- RP-252819 New SID: Study on Integrated Sensing And Communication (ISAC) for NR

- R3-258088	(pCR to TR 38.765) Study on Integrated Sensing and Communication (ISAC) for NR

- TR 23.700-14 Study on Integrated Sensing and Communication

- IETF RFC 6083: "Datagram Transport Layer Security (DTLS) for Stream Control Transmission Protocol (SCTP)".

- R1-2507427 Summary #5 on NR ISAC_v041_Moderator

- R3-258116	(TP for ISAC TR) Network architecture for ISAC	Huawei

# Annex – TP to ISAC pCR against R3-257332

CHANGES START

# 8	RAN-CN procedures and signalling

Editor’s note: This section is to capture the study outcome of procedures and signaling aspects between RAN and CN for gNB-based monostatic sensing.

## 8.x	Sensing Service Initiation

Editor’s Note: the following may need further refinement.

Figure 8.x-1: Message flow for sensing service initiation and reporting

0.	The gNB and SF setup the NG interface, where the gNB indicates its application-level information, e.g., TAC list, PLMN ID list etc to the SF.

1.	The SF sends sensing service request to the gNB, including the sensing area, report characteristics (e.g., one-time, periodicity) etc.

2.	The gNB sends sensing service response to the SF.

3.	The gNB performs the sensing measurement, and generates sensing data.

4.	As requested in Step 1, the gNB sends sensing data to the SF.

## 8.y	Sensing Service Stop

Editor’s Note: the following may need further refinement.

The SF initiated sensing service stop procedure is illustrated in Figure 8.y.1.

Figure 8.y-1: Message flow for sensing stop reporting

1.	The SF sends sensing service stop command to the gNB, which shall stop the sensing measurement and reporting.

2.	The gNB sends sensing service stop response to the SF.

The gNB initiated sensing service failure indication procedure is illustrated in Figure 8.y.2.

Figure 8.y-2: Message flow for sensing failure indication

1.	The gNB sends sensing service failure indication to the SF.