3GPP TSG-RAN WG3 Meeting #130	R3-258146

Dallas, US, November 17 - 21, 2025

Agenda Item:	10.4

Source:	ZTE Corporation

Title:	(pCR to TR38.760-3) Further discussion on internal functional split and interfaces

Document for:	Discussion

- Introduction

In last meeting, the internal functional split in 6G, i.e., whether to support the HLS, has been discussed and the following issue has been identified as below.

Capture pain points and benefits (and potentially derived requirements) of 5G HLS To be continued...

In this contribution, we provide the further consideration on internal functional split and interfaces in 6G.

- Discussion

  - Split Architecture

2.1.1 RAN3 Standardization Effort for 5G Split architecture

In 5G standardization phase, RAN3 has taken lots of effort on introducing the HLS architecture and published three sets of technical specifications related to F1 interface (38.47x series), E1 interface (38.46x series for R15 and R16  and switched to 37.48x series since R17) and W1 interface (37.48x series).

For F1 interface, the SI phase starts from April 2016 to March 2017 with the TR 38.801 as the output. And the first stable set of technical specifications was achieved in Release 15.

For E1 interface, a dedicated SI phase starts from June 2017 to December 2017 with the TR 38.806 as the output. And the first stable set of technical specifications was achieved in Release 15.

For W1 interface, a dedicated phase starts from October 2017 to March 2018 with the TR 37.876 as the output. And the first stable set of technical specifications was achieved in Release 16.

| Interface | Final Solution | Output of SI Phase | Output of WI Phase |
| --- | --- | --- | --- |
| F1 | Option 2 | TR 38.801 From 2016.04 | TS 38.470/471/472/473/474/425 From 2017.04 |
| E1 | Solution 2 | TR 38.806 From 2017.06 | TS 38.460/461/462/463 From 2018.02 TS 37.480/481/482/483 From 2022.02 |
| W1 | Option 2 | TR 37.876 From 2017.10 | TS 37.480/481/482/483/425 From 2018.05 |

Table 1. Summary of RAN3 work on the different interfaces for split architecture

Based on the above summary, it takes 3 SIs and 18 Technical Specifications for RAN3 to finalize the stable progress of the split architecture. In addition, after Release 15, almost all newly introduced features of each release have standard impact on F1 or E1 interface, and it also takes lots of time for RAN3 to work on the solution and impacts towards corresponding specifications. Therefore, the support of split architecture in 5G introduced quite high complexity on RAN3’s workload.

Observation 1: Since R15, RAN3 has taken tremendous effort on the standardization of split architecture in 5G.

2.1.2 Commercial Deployment for 5G Split architecture

However, there seems to be no successful large-scale commercial 5G deployment with this split architecture so far. Only the North America area held a fair market share on the split architecture (e.g., Open RAN or V-RAN) deployment, and the market share of split architecture seems not sizable in the rest area of the world. As a reminder, Open RAN investments accelerated rapidly in the initial phase between 2019 and 2022. Open RAN-based investments then declined since 2023,  and short-term visibility is more uncertain.

Observation 2: The market share of commercial deployment for split architecture is not as sizeable as expected after 5G standardization.

During the 6G standardization phase, the following pain points from 5G split architecture must be taken into account :

- Performance including service latency, and UE experience

The F1 and E1 interfaces introduced in split architecture cause higher transmission, propagation, queuing, and processing latency compared to the legacy architecture. Furthermore, the increased overhead of RRC messages due to TLS/IPSec security protocols (e.g., the RRCSetupComplete message increases from 23 bytes to 70-115 bytes) further exacerbates latency. Moreover, under the same input load, the split architecture has a lower service processing capacity due to architectural overhead (primarily the additional latency introduced by the F1 interface), which directly leads to the strain of system resources, resulting in a higher average latency perceived by users [1]. In addition, the split architecture may introduce the reliability risk. To be more specific, the centralized deployment increases the operational risk: If the failure occurs in the data center with centralized CU deployment, it may result in “chain reaction” which potentially affects the base station with DUs involved in the whole area and the UE experience will also be impacted severely.

- Power Consumption

In addition, the power consumption is one key factor of the network performance evaluation. Base stations and related infrastructure that are part of access networks can use up to 50% of the total energy consumed by the network segment. For the split architecture, the density of RUs and the longest distance each DU and CU can support shall affect the power consumption of network. Longer distances and higher densities are typically associated with higher energy needs. In the case of the most stringent values of maximum supported distance, the split architecture has almost 8 times greater consumption than the RAN one [2]. So, the power consumption could be a barrier for the Operators to deploy the split architecture in practical.

- CAPEX/OPEX

With the introduction of the CU-DU split architecture, the deployment costs will increase, such as the addition of the CU and DUs compared with aggregated gNB, the renovation of the legacy base station for interaction with neighbour split nodes, and the expansion of the power suppliers and the batteries. Meanwhile, the separate deployment of the CU and DU shall inevitably lead to the increase of the maintenance and interoperability costs. Along with the actual commercial deployment of split architecture mentioned above, the CAPEX/OPEX is another factor cannot be neglected.

Proposal 1: The pain points from 5G split architecture shall be considered in 6G study phase:

- Performance including service latency and UE experience

- Power Consumption

- CAPEX/OPEX

Currently, the requirement on the support of split architecture in 6G is still not clear, and RAN3 should focus on the interface with real needs on the inter-vendor interoperability. Considering the tremendous effort on the standardization of split architecture in 5G, RAN3 should also take the complexity and time budget into account. If the split architecture is still needed in 6G, the gap analysis is required to understand the real needs from the market.

Observation 3: The requirement from commercial deployment on split architecture in 6G is still not clear.

Based on all the above discussion, the following options can be considered in 6G:

Option 1: No split architecture in 6G, operator owns the 5G split architecture which allows to be evolved to connect to 6GC(which is also pending to SA2 discussion on 6GC) via the evolved gNB-CU and gNB-DU.

Option 2: Split architecture is supported in 6G as a valid deployment.

Option 3: Keep Split Architecture in 6G taking the HLS defined in 5G as baseline.

Option 4: Only keep stage 2 split function specified in 6G, without stage 3 description.

Proposal 2: RAN3 shall consider and evaluate the above options on whether and how to support the split architecture in 6G from Day1.

- 2.2 Interface between 6G RAN nodes

Regarding the interface between the 6G RAN nodes, RAN3 has introduced the Xn interface in NR, similar as the X2 interface in LTE, which can be considered as the start point. And the possible deployment of 6G RAN nodes can be illustrated as below.

Figure 5. Possible deployment of 6G RAN nodes

- To achieve the connection between the 6G RAN nodes, the Xn+ interface, similar as Xn interface, can be applied, especially for the Standalone scenario in the Day 1 of 6G. In details, the connection between two 6G NodeB (6NB)s or one 6NB and one evolved gNB (gNB+) or two gNB+s can be achieved by means of the Xn+ interface.

Proposal 3: The Xn+ interface, similar as Xn interface, can be introduced to achieve the connection between the RAN nodes in 6G.

In the 5G standardization phase, the general principle of Xn interface is given as below [3].

4.2 Xn interface general principles The general principles for the specification of the Xn interface are as follows: - the Xn interface is open; - the Xn interface supports the exchange of signalling information between two NG-RAN nodes, and the forwarding of PDUs to the respective tunnel endpoints; - from a logical standpoint, the Xn is a point-to-point interface between two NG-RAN nodes. A point-to-point logical interface should be feasible even in the absence of a physical direct connection between the two NG-RAN nodes.

The Xn interface has already been applied in the 5G commercial market, and there seems to be no strong requirement to introduce principle enhancement on top of the existing Xn interface. Therefore, the general principles of Xn interface should be considered as the baseline of the interface between RAN nodes in 6G (e.g., Xn+ interface).

Proposal 4: The general principles of Xn interface defined in 5G should be considered as the baseline of Xn+ interface in 6G.

- Summary

Observation 1: Since R15, RAN3 has taken tremendous effort on the standardization of split architecture in 5G.

Observation 2: The market share of commercial deployment for split architecture is not as sizeable as expected after 5G standardization.

Proposal 1: The pain points from 5G split architecture shall be considered in 6G study phase:

- Performance including service latency and UE experience

- Power Consumption

- CAPEX/OPEX

Observation 3: The requirement from commercial deployment on split architecture in 6G is still not clear.

Based on all the above discussion, the following options can be considered in 6G:

Option 1: No split architecture in 6G, operator owns the 5G split architecture which allows to be evolved to connect to 6GC (which is also pending to SA2 discussion on 6GC) via the evolved gNB-CU and gNB-DU.

Option 2: Split architecture is supported in 6G as a valid deployment.

Option 3: Keep Split Architecture in 6G taking the HLS defined in 5G as baseline.

Option 4: Only keep stage 2 split function specified in 6G, without stage 3 description.

Proposal 2: RAN3 shall consider and evaluate the above options on whether and how to support the split architecture in 6G from Day1.

Proposal 3: The Xn+ interface, similar as Xn interface, can be introduced to achieve the connection between the RAN nodes in 6G.

Proposal 4: The general principles of Xn interface defined in 5G should be considered as the baseline of Xn+ interface in 6G.

The corresponding pCR to TR 38.760-3 is given in the Annex.

4	Reference

- [1] Chatzimiltis S, Shojafar M, Mashhadi M B, et al. Surviving the Storm: The Impacts of Open RAN Disaggregation on Latency and Resilience[J]. arXiv preprint arXiv:2505.00605, 2025.

- [2] BALDINI, G., BOLLA, R., BRUSCHI, R., CARREGA, A., DAVOLI, F. et al., Toward Sustainable O-RAN Deployment: an in-depth Analysis of Power Consumption, IEEE TRANSACTIONS ON GREEN COMMUNICATIONS AND NETWORKING, 9, 2, 2025, p. 429-444, IEEE-INST ELECTRICAL ELECTRONICS ENGINEERS INC, https://data.europa.eu/doi/10.1109/TGCN.2024.3426108, JRC135185.

- [3] TS 38.420, v19.1.0

5	Annex

pCR to TR 38.760-3

6.2	RAN Internal Interfaces

Editor’s Note: This chapter shall comprehensively address study on CU-DU and CP-UP aspects including functional split considering interim milestone TSG#112 (June/2026) to take a decision during TSG#115: March 2027

Editor’s Note: This chapter should also address the interfaces between 6G RAN nodes (equivalent to Xn).

6.2.1	General Principles

Editor’s note: The aim of this section is to describe general design principles and requirements for RAN Internal Interfaces

6.2.1.1	Interface between RAN nodes

Editor’s Note: This chapter should also address the interface between 6G RAN nodes.

Figure 6.2.1.1. Deployment of 6G RAN nodes

The interface allowing to interconnect two 6G NodeB (6NB)s, or one 6G NodeB and one evolved gNB (gNB+), or two evolved gNBs with each other is referred to as the Xn+ interface.

NOTE:	The name of the interface can be further discussed in normative phase.

The general principles for the specification of the Xn+ interface are as follows:

-	the Xn+ interface shall be open;

-	the Xn+ interface shall support the exchange of signalling information between the endpoints, and the forwarding of user data to the respective tunnel endpoints;

-	from a logical standpoint, the Xn+ is a point-to-point interface between the endpoints. A point-to-point logical interface should be feasible even in the absence of a physical direct connection between the endpoints.

-	the Xn+ interface shall support control plane and user plane separation;

-	the Xn+ interface shall separate Radio Network Layer and Transport Network Layer;

-	the Xn+ interface shall be future proof to fulfil different new requirements, support new services and new functions.

6.2.1.2	Evaluation and interface(s) for split architecture

Editor’s Note: This chapter is used to evaluate and address the CU-DU and/or CP-UP aspects considering interim milestone.

The following aspects should be evaluated on the split architecture in 6G from Day1:

-	Commercial deployment requirements

The market share of commercial deployment for split architecture is not as sizeable as expected after 5G standardization.

-	Standardization complexity

In 5G, it takes 3 SIs and 18 Technical Specifications for RAN3 to finalize the stable progress of the split architecture. In addition, after Release 15, almost all newly introduced features of each release have standard impact on F1 or E1 interface, even worse, some features are completed in 5G without taking split architecture into account which brings inconsistency handling. Such standardization complexity should be avoided in 6G.

-	CAPEX and OPEX

With the introduction of the CU-DU split architecture, the deployment costs will increase, such as the addition of the CU and DUs compared with the aggregated gNB, the renovation of the legacy base station for interaction with neighbour split nodes, and the expansion of the power suppliers and the batteries. Meanwhile, the separate deployment of the CU and DU shall inevitably lead to the increase of the maintenance and interoperability costs.

-	Power consumption

The power consumption is one key factor of the network performance evaluation. Base stations and related infrastructure that are part of access networks can use up to 50% of the total energy consumed by the network segment. For the split architecture, the density of RUs and the longest distance each DU and CU can support shall affect the power consumption of network. Longer distances and higher densities are typically associated with higher energy needs.

-	Performance including service latency and UE experience

The F1 and E1 interfaces introduced in the split architecture cause higher transmission, propagation, queuing, and processing latency compared to the legacy architecture. Furthermore, the centralized CU deployment may result in “chain reaction” which potentially affects the base station with DUs involved in the whole area and the UE experience will also be impacted severely.

The possible WFs to be considered on split architecture in 6G:

Option 1: No split architecture in 6G, operator owns the 5G split architecture which allows to be evolved to connect to 6GC (which is also pending to SA2 discussion on 6GC) via the evolved gNB-CU and gNB-DU.

Option 2: Split architecture is supported in 6G as a valid deployment.

Option 3: Keep Split Architecture in 6G taking the HLS defined in 5G as baseline.

Option 4: Only keep stage 2 split function specified in 6G, without stage 3 description.